3GPP TSG-RAN WG4 Meeting # 106bis-e													R4-2306322

Online, April 17 – April 26, 2023

Agenda item:			5.9.4

Source:	OPPO

Title:	WF on RRM requirements for FR1+FR1 NR-DC

Document for:	Approval

- Topic #1: RRM Core requirements for FR1-FR1 NR-DC

- Sub-topic 1-1:  SCG activation/deactivation requirements for FR1-FR1 NR-DC

Issue 1-1-2: Requirements for RACH-less SCG activation (Tsearch)

GTW Agreements:

Requirements for RACH-less SCG activation (Tsearch)

  - Define requirements for cases when

    - 4) Target PSCell is known and TCI is known

  - Do not define requirements for other cases

    - 1) Target PSCell is unknown but TCI is known

    - 2) Target PSCell is unknown and TCI is unknown

    - 3) Target PSCell is known but TCI is unknown

Candidate options:

For the case 4) when Target PSCell is known and TCI is known, requirements for RACH-less SCG activation for FR1+FR1 NR-DC can be:

  - Option 1 (Xiaomi, Huawei, Nokia, Ericsson, OPPO):

    - If RLM and BFD are configured and TCI state is known, Tsearch = 0 ms if the target cell is a known FR1 PSCell.

  - Option 2 (Apple, Ericsson, OPPO):

    - If RLM and BFD are configured and TCI state is known, Tsearch = 0 ms if the target cell is a known FR1 PSCell.

      - Add the side condition of Es/Iot>=-2dB to avoid the case that RLF or BF is detected during the SCG activation procedure.

  - Option 3 (vivo):

    - if RLM and BFD are configured and TCI state is known, Tsearch = 0 ms if the target cell is a known FR1 PSCell.

      - Add the ‘SNR of TCI state ≥ -3dB’ as the sub-condition of known TCI state for SCG Activations.

Tentative agreement for 2nd round:

Option 1

Issue 1-1-3: Requirements for RACH based PSCell activation

Candidate options:

  - Option 1 (Nokia): Update the requirements:

    - For RACH based PSCell activation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR1 PSCell configured with bfd-and-RLM with value true and no RLM has occurred, then Tsearch = [1]* Trs ms, otherwise if Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

  - Option 2: Follow previous agreements:

    - For RACH based PSCell activation, if the target cell is a known NR FR1 PSCell, Tsearch = 0 ms. If the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms.

Tentative agreement for 2nd round:

Option 2

Issue 1-1-4：Whether to jointly define SCG activation delay in R18 WI and R17 maintenance

Tentative agreement for 1st round：

RAN4 update the requirements for SCG activation delay for FR1+FR1 NR-DC in R18 if further progress achieved in R17 SCG activation delay requirement.

Issue 1-1-5: Whether to indicate the beam failure during SCG activation procedure

Candidate options:

  - Option 1 (Apple): UE report beam failure of the PSCell during PSCell activation (after receiving the SCG activation message)

  - Option 2 (vivo): Deprioritize this issue due to a corner case when BFD happens during activation procedure

  - Option 3 (Apple, OPPO, Ericsson): Ask RAN2 for clarifications

Tentative agreement for 2nd round:

Need further check and come back in next meeting.

Issue 1-1-1: General aspects related to BFD and RLM

Candidate options:

- Option 1:

  - A UE which has not detected either BFD or RLF on the deactivated PSCell, need no additional measurements at PSCell activation.

  - A UE which has detected either BFD or RLF on the deactivated PSCell is allowed additional measurements at PSCell activation.

- Option 2: If UE has detected either BFD or RLF on the deactivated PSCell for RACH-less SCG Activation, UE fall back to RACH based activation.