3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2306335

Electronic Meeting, April 17th – April 26th, 2023

Title: 	WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA

Source: 	Huawei, HiSilicon

Agenda item:	5.12.3

Document for:	Approval

Introduction

This contribution is to capture the agreements and the progress on RRM requirements for intra-band non-collocated EN/DC/NR-CA in RAN4 #106bis-e meeting.

Way-forward

Topic#1: FR1 non-collocated EN-DC/NR-CA for Type 2 UE for 2 layer MIMO case

Sub-topic 1-1 MRTD/MTTD requirements

Issue 1-1-1: MRTD/MTTD requirements for FR1 inter-band EN-DC with overlapping DL band in R16

- Agreements

  - Impact on legacy MRTD/MTTD requirements due to UE capability of interBandMRDC-WithOverlapDL-Bands-r16 should be discussed in RRM maintenance part instead of in this WI.

Issue 1-1-2: MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE

- Agreements

  - Intra-band contiguous CA operation is only applied for co-located deployment.

  - The MRTD/MTTD requirements for intra-band non-contiguous CA for Type 2 UE are specified without deployment restrictions

Sub-topic 1-2 Interruption requirements

Issue 1-2-1: Interruption requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE

- Agreements

  - For intra-band CA, the interruption requirements are applied based on the following assumptions

    - Contiguous CCs are within the same sub-block.

    - Non-contiguous CCs are cross different sub-blocks.

Sub-topic 1-3 SCell activation requirements

Issue 1-3-1: Impacts on SCell activation requirements

- Agreements

  - For non-collocated FR1 intra-band non-contiguous CA

    - The existing activation delay requirements (clause 8.3.2) for a known FR1 SCell can be re-used for Type 2 UE.

    - The existing activation delay requirements (clause 8.3.2) for an unknown FR1 SCell for non-contiguous intra-band CA can be re-used for Type 2 UE.

    - FFS whether any CRs are needed to clarify the requirements applicability.

Sub-topic 1-5 Other RRM requirements

Issue 1-5-2: Impacts on SCell BFD requirements

- Agreements

  - For non-collocated FR1 intra-band non-contiguous CA, Type 2 UE can be configured with BFD/CBD measurements on up to two serving cells in the same band.

Topic#2: FR1 non-collocated EN-DC/NR-CA for New Type UE for 4 layer MIMO case

Sub-topic 2-1 General aspects

Issue 2-1-1: RRM discussion for Type 4a/4b UE for 4 layer MIMO

- Agreements

  - Whether to define RRM requirements for Type 4 UE needs to wait for RF’s conclusion on whether to introduce Type 4 UE.

Issue 2-1-2: RRM discussion for Type 3a/3b UE for 4 layer MIMO

- Agreements

  - To postpone the discussion on Type 3a/3b UE till the WI scope is confirmed.