3GPP TSG-RAN WG4 Meeting#106bis                              R4-2306360

E-meeting, April 17 – April 26,2023

Title: 	WF on NCR-MT RRM requirements

Agenda Item:	     5.29.7

Source: 	ZTE

Document for:	Approval

Agreement reached in 1st round

Issue 1-2: Random access

Recommendations WF:

- Only define the 4 step RACH for NCR-MT.

Issue 2-2: the gradual timing adjustment requirement Tp and Tq

Recommendations WF:

- Reuse IAB-MT gradual timing adjustment requirement Tp and Tq;

- The requirement is applicable for LA NCR-MT only.

Issue 2-4:  TA adjustment accuracy requirement

Recommendations WF:

- reuse the same requirement as Rel-16 IAB-MT.

Issue 3-1:  Active BWP switching requirement

Recommendations WF:

- Not to define active BWP switching requirement for NCR-MT.

Issue 4-1: adaptive beamforming for NCR-MT

Recommendations WF:

- Not to define the TCI switching and uplink spatial relationship switching requirement for WA NCR-MT

Issue 6-1:  CA/DC related RRM requirements for NCR-MT

Recommendations WF:

Not to define CA/DC related RRM requirement for NCR-MT

Agreement reached in 2nd round

Issue 1-2: Random access

Agreement:

To define 4 step rach for LA NCR-MT only and without RA requirement for WA NCR-MT.

Issue 2-3:  the timer accuracy requirement

Agreement:

- to define timer accuracy requirement only for LA NCR-MT.

- The requirement defined in Clause 7.2 of 38.133 can be reused.

Issue 2-4:  TA adjustment accuracy requirement

Agreement:

- To define the TA adjustment requirement for both WA and LA NCR-MT

Issue 4-3: adaptive beamforming for NCR backhaul link

Agreement:

- Not to define the RRM requirement for NCR-Fwd backhual link.

Agreement reached at GTW

Issue 1-1: RRC re-establishment

- Agreements

  - Define the RRC re-establishment requirement for LA NCR-MT only in Rel-18

Issue 2-1:  Initial transmit timing requirement Te

- Agreements

  - Define Te requirement for WA and LA NCR-MT

Issue 4-1: adaptive beamforming for NCR-MT

- Agreements

  - Do not define TCI switching and uplink spatial relationship requirements for LA NCR-MT

Issue 4-2: adaptive beamforming for NCR access link

- Agreements

  - FFS whether to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency.

Issue 5-1:  BFD/BFR/RLM for NCR-MT

- Agreements

  - For LA NCR-MT

    - Define RLM/BFD/BFR requirements

  - For WA NCR-MT

    - Do not define BFD/BFR requirements

    - FFS for RLM requirements.

Summarized agreement for the ease of readability

Issue 1-1: RRC re-establishment

- Agreements

  - Define the RRC re-establishment requirement for LA NCR-MT only in Rel-18

Issue 1-2: Random access

Recommendations WF:

- Only define the 4 step RACH for NCR-MT.

- To define 4 step rach for LA NCR-MT only and without RA requirement for WA NCR-MT.

Issue 2-1:  Initial transmit timing requirement Te

- Agreements

  - Define Te requirement for WA and LA NCR-MT

Issue 2-2: the gradual timing adjustment requirement Tp and Tq

Recommendations WF:

- Reuse IAB-MT gradual timing adjustment requirement Tp and Tq;

- The requirement is applicable for LA NCR-MT only.

Issue 2-3:  the timer accuracy requirement

Agreement:

- to define timer accuracy requirement only for LA NCR-MT.

- The requirement defined in Clause 7.2 of 38.133 can be reused.

Issue 2-4:  TA adjustment accuracy requirement

Recommendations WF:

- reuse the same requirement as Rel-16 IAB-MT.

- To define the TA adjustment requirement for both WA and LA NCR-MT

Issue 3-1:  Active BWP switching requirement

Recommendations WF:

- Not to define active BWP switching requirement for NCR-MT.

Issue 4-1: adaptive beamforming for NCR-MT

Recommendations WF:

- Not to define the TCI switching and uplink spatial relationship switching requirement for WA NCR-MT

- Do not define TCI switching and uplink spatial relationship requirements for LA NCR-MT

Issue 4-2: adaptive beamforming for NCR access link

- Agreements

  - FFS whether to define RRM requirement for NCR-Fwd access link for beam configuration and switching latency.

Issue 4-3: adaptive beamforming for NCR backhaul link

Agreement:

- Not to define the RRM requirement for NCR-Fwd backhual link.

Issue 5-1:  BFD/BFR/RLM for NCR-MT

- Agreements

  - For LA NCR-MT

    - Define RLM/BFD/BFR requirements

  - For WA NCR-MT

    - Do not define BFD/BFR requirements

    - FFS for RLM requirements.

Issue 6-1:  CA/DC related RRM requirements for NCR-MT

Recommendations WF:

Not to define CA/DC related RRM requirement for NCR-MT