3GPP TSG-RAN WG4 Meeting # 106bis-e											R4-2306363

Electronic Meeting, 17 April –26 April, 2023

Title: 	WF on RRM requirements for sidelink unlicensed operation

Agenda Item:	5.31.4

Source: 	LG Electronics

Document for:	Approval

- Transmit timing

Issue 1-1: Transmit timing error requirement

- RAN4 needs to investigate if existing timing error requirement can be reused based on RAN1’s conclusion for S-SSB transmission design.

Issue 1-2: (non-)Available condition for SyncRef UE

- New definition for (non-)availability of SyncRef UE for SL-U is required.

- RAN4 needs to wait for RAN1’s conclusion for S-SSB transmission design.

- Initiation / Cease of SLSS transmission

Issue 2-1: Requirements for SyncRef UE as synchronization reference source

- Extending the measurement period to 4+x S-SSB periods, x is the S-SSB periods in which the SLSS is not available due to LBT failures, and is capped by x_max. FFS for detailed description and x_max.

Issue 2-2: Requirement when exceeding the maximum allowed LBT failures during evaluation to initiate/cease SLSS transmission

- RAN4 to continue discussion whether and how to define requirement and procedure when exceeding the maximum allowed LBT failure during evaluation to initiate/cease SLSS transmission

Issue 2-3: NR-U gNB as a synchronization reference source

- NR-U gNB as a synchronization reference source is out of scope in Rel-18.

Issue 2-4: First SLSS transmission

- RAN4 to continue discussion whether to introduce additional time or condition for initial SLSS transmission after evaluation period due to LBT failure.

- Selection / Reselection of V2X synchronization Reference source

Issue 3-1: Requirements for Tdetect, SyncRef UE_V2X and allowed drop rate

- Extend detection period considering S-SSB not available due to LBT failure during the detection period

- FFS: allowed Tx dropping rate and Rx dropping rate

- FFS: Maximum allowed LBT failure

- For SL DRX and SL non-DRX case, it will be discussed based on above bullets.

Issue 3-2: Requirement for Tmeasure, PSBCH-RSRP

- Tmeasure,PSBCH-RSRP during which the UE shall be capable of performing PSBSCH-RSRP measurements of 3 identified intra-frequency SyncRef UEs is defined as blow Table as a starting point. y is the S-SSB periods in which the SLSS is not available due to LBT failures and FFS for detailed description and y_max which is capped.

| SL-DRX cycle [ms] | Tmeasure,PSBCH-RSRP [ms] |
| --- | --- |
| No SL-DRX | (2 + y)*160 |
| SL-DRX cycle ≤ 160ms | (2 + y)*160 |
| SL-DRX cycle > 160ms | (2 + y)*SL-DRX cycle |

Issue 3-3: Requirement when exceeding the maximum allowed LBT failures

- RAN4 to continue discussion whether and how to define requirement and procedure when exceeding the maximum allowed LBT failure for selection/reselection of V2X synchronization source.

- L1 SL-RSRP measurement

Issue 4-1: SL-RSRP measurement requirement

- RAN4 needs to investigate whether to have L1 SL-RSRP requirement impact depending on RAN1’s conclusion for procedure design (e.g., MCSt, two candidate starting symbols, etc.).

- Congestion control measurement

Issue 5-1: RSSI measurement

- RAN4 needs to investigate whether to have SL RSSI measurement impact depending on RAN1’s conclusion for congestion control.

- Interruption

Issue 6-1: Interruption

- RAN4 to continue discussion whether to introduce interruption requirements for

  - Proposal 1-1: RAN4 needs to investigate the effects of LBT procedure to the existing interruption on SL especially for synchronization source change and transitions between active and non-active during DRX.

    - whether the interruption interval [1 ms] or definition of interrupt interval need to consider LBT failure or not.

  - Proposal 1-2: Study and introduce the interruption requirements due to LBT operation during SL-DRX off duration.

    - at the moment of performing LBT operation during DRX off duration, an interruption may occur to WAN.

- Scheduling availability

Issue 7-1: Scheduling availability

- The legacy requirements are applicable to SL-U.

- Others

Issue 8-1: LBT failure level

- No further discussion on this issue unless specific observation/necessity is identified

Issue 8-2: Measurement accuracy requirements

- Following issue will be discussed in performance phase.

  - RAN4 to study if the measurement accuracy requirement defined for NR-U for the CCA band can be reused.