3GPP TSG-RAN4 Meeting #106bis-e		R4-2306365

Electronic Meeting, 17 April –26 April, 2023

Agenda item:			5.32.4

Source:	Moderator (Ericsson)

Title:	WF on R18 RedCap RRM requirements

Document for:	Approval

### Defining of requirements for eDRX cycle > 10.24 sec in FR1 and FR2

Extended eDRX range requirements in RRC INACTIVE

- INACTIVE state requirements for eDRX cycle > 10.24 sec

  - The sample number needed to perform the serving cell and intra/inter-frequency neighbor cell detection/evaluation/measurements for corresponding IDLE mode eDRX cycles can be reused for INACTIVE mode eDRX cycles.

When configured with both IDLE and INACTIVE eDRX configurations for serving cell measurements

- Wait for the RAN2 decisions on the T value and PTW length design before defining serving cell measurements requirements

When configured with both IDLE and INACTIVE eDRX configurations for neighbour cell measurements

The detection/measurement/evaluation delay requirements for intra-frequency/inter-frequency neighbour cells shall be specified in the step of:

- INACTIVE eDRX cycle, if the eDRX_Inactive=2.56/5.12/10.24sec

- FFS if eDRX_Inactive  20.48sec

When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for serving cell measurements

- Measurements/evaluation period shall be constrained to be within a single PTW irrespective of whether IDLE or INACTIVE eDRX PTW is being used.

  - Note: Wait for the RAN2 decisions on the T value and PTW.

When to measure when configured with both IDLE and INACTIVE eDRX configurations larger than 10.24s for neighbour cell measurements

The number of samples needed for Tdetect,NR of intra-freq or inter-freq cell measurement (measured in DRX cycles) may belong to more than one PTW.

When to measure when IDLE and INACTIVE eDRX PTW do not coincide

RAN4 to wait for RAN2 progress to discuss following:

- Whether to clarify the measurement behavior when the PTWs of IDLE and INACTIVE states are not coinciding.

Transition requirements

RAN4 to discuss the need for transition requirements (UE behavior and corresponding requirement for the transition between short INACTIVE eDRX (≤10.24sec) and long INACTIVE eDRX(20.48sec)) after the INATIVE eDRX with PTW requirements are finalized.

CG-SDT requirements with PTW

  - FFS whether to define requirements for CG-SDT for Rel-18 eRedCap UE with PTW

CG-SDT requirements with PTW: RSRP2 time range

Following option is to be discussed after RAN4 has identified whether new requirements are to be specified for Rel-18 eRedCap with PTW:

- RSRP2 time range in FR1 with eDRX cycle > 10.24 sec is defined as (T2 –640ms) ≤ T2’ ≤ T2.

### RRM impact due to baseband reduction

Impact on CGI reading requirements

- FFS whether to define CGI reading requirements for Rel-18 eRedCap UEs

- If CGI reading requirements for Rel-18 eRedCap UEs are introduced

  - If SIB1 is scheduled within 5MHz BW

    - Reuse existing requirements for SIB1 reading for the case

  - FFS for the requirements for the case of SIB1 scheduled with BW > 5MHz

Impact on paging requirements

No impact on paging reception requirements due to baseband BW reduction.

Impact on THARQ

There is not any impact on THARQ due to baseband BW reduction.

### Defining of requirements for eDRX cycle > 10.24 sec in FR1 and FR2

Applicability of Rel-17 RLM/BFD relaxation requirements to Rel-18 RedCap

Since relaxation of relaxed RLM/BFD does not belong to any of the Rel-18 RedCap objectives, and there is no consensus to develop those within Rel-18 WI scope, this issue is closed, and no more discussions needed.