3GPP TSG-RAN4 Meeting #106bis-e		R4-2306384

# Electronic Meeting, April 17-26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | DraftCR | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on conditions for NB-IoT bands in annex B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIoT_eMTC_NTN_req-Perf |  |  |  |  |  | Date: |  |  | 2023-04-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add conditions in annex B for bands for NB-IoT with satellite access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The conditions for bands 255 and 256 for NB-IoT (UE category NB1 and NB2) with satellite access are defined for the following set of requirements: - Intra-frequency measurements for cell re-selection in RRC idle/inactive states for HD-FDD - Intra-frequency measurements in RRC connected state for HD-FDD The minimum NRSRP and minimum NSCH_RP are determined based on REFSENS of the corresponding band in Table 7.3B-1 for HD-FDD in TS 36.102 (i.e. -108.2 dBm for band 255 and band 256). According to TS 36.133, the minimum NRSRP/minimum NSCH_RP for any NB-IoT band is -129 dBm/15 kHz in normal coverage and -138 dBm/15 kHz in enhanced coverage in RRC idle/inactive/connected states based on REFSENS of -108.2. Therefore, for both 255 and 256 bands, the minimum NRSRP/minimum NSCH_RP is -129 dBm/15 kHz in normal coverage and -138 dBm/15 kHz in enhanced coverage in RRC idle/inactive/connected states. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRM requirements will be incomplete. The UE cannot pass RRM requirements and corresponding test cases for bands supported for UE category NB1 and NB2 with satellite access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.1.X3, B.2.X3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

## B.1.X3	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1 and NB2 for satellite access

This clause defines the NB-IoT intra-frequency NRSRP, NRSRP Ês/Iot, NSCH_RP and NSCH Ês/Iot applicable for a corresponding operating band for satellite access. The band groups for UE category NB1 and NB2 for satellite access are defined in Section 3.5.1A. The UE category NB1 and NB2 applicability of the conditions in Appendix B.1.X3 is defined in Section 3.6.

The conditions for measurements of intra-frequency NB-IoT cells in normal coverage for cell re-selection are defined in Table B.1.X3-1.

The conditions for measurements of intra-frequency NB-IoT cells in enhanced coverage for cell re-selection are defined in Table B.1.X3-2.

Table B.1.X3-1: NB-IoT intra-frequency measurements for HD-FDD in normal coverage for satellite access

| Parameter | E-UTRA operating band groupsNote 1 | Minimum NRSRP | Minimum NSCH_RP | NRS Ês/Iot | NSCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | NFDD_SAB_G | -129 | -129 | -6 | -6 |
| NOTE 1: E-UTRA operating band groups for satellite access are defined in Section 3.5.1A. |  |  |  |  |  |

Table B.1.X3-2: NB-IoT intra-frequency measurements for HD-FDD in enhanced coverage for satellite access

| Parameter | E-UTRA operating band groupsNote 1 | Minimum NRSRP | Minimum NSCH_RP | NRS Ês/Iot | NSCH Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm/15kHz | dBm/15kHz | dB | dB |
| Conditions | NFDD_SAB_G | -138 | -138 | -15 | -15 |
| NOTE 1: E-UTRA operating band groups for satellite access are defined in Section 3.5.1A. |  |  |  |  |  |

----------------------START OF NEXT CHANGE----------------------------

## B.2.X3	Conditions for NB-IoT intra-frequency measurements by UE Category NB1 and NB2 for satellite access

This clause defines the NB-IoT intra-frequency NSCH_RP and NSCH Ês/Iot applicable for a corresponding operating band for satellite access. The band groups for UE category NB1 and NB2 for satellite access are defined in Section 3.5.1A. The UE category NB1 and NB2 applicability of the conditions in Appendix B.2.X3 is defined in Section 3.6.

The conditions for intra-frequency measurements in normal coverage are defined in Table B.2.X3-1 and B.2.X3-3.

The conditions for intra-frequency measurements in denhanced coverage are defined in Table B.2.X3-2 and B.2.X3-4.

Table B.2.X3-1: NB-IoT intra-frequency measurements for HD-FDD in normal coverage for satellite access

| Parameter | E-UTRA/NR operating band groupsNote 1 | Minimum NSCH_RP | NSCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | NFDD_G NFDD_SAB_G | -129 | -6 |
| NOTE 1: E-UTRA operating band groups for satellite access are defined in Section 3.5.1A. |  |  |  |

Table B.2.X3-2: NB-IoT intra-frequency measurements for HD-FDD in enhanced coverage for satellite access

| Parameter | E-UTRA/NR operating band groupsNote 1 | Minimum NSCH_RP | SCH Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm/15kHz | dB |
| Conditions | NFDD_SAB_G | -138 | -15 |
| NOTE 1: E-UTRA operating band groups for satellite access are defined in Section 3.5.1A. |  |  |  |

----------------------END OF CHANGE----------------------------