3GPP TSG-RAN WG4 Meeting #xxx	R4-2306467

<Meeting Location>, <Date> ‒ <Date>, 2023

Title: 	WF on CA_n5-n8

Agenda Item:	4.30.3

Source: 	Skyworks Solutions, Inc.

Document for:	Approval

# Scope

LS to RAN2 is not in scope, the way forward addresses which options are specified to support CA_n5-n8 and their related requirements

# CA_n5-n8 band combinations options

## Background

With the direct overlapping of n8UL and n5DL the only possibility to support such combination relies on restricted spectrum in n5 and n8 as shown in Figure 1 that correspond to the spectrum available in China.

Figure 1: requested spectrum support in China

In the rest of the document the restricted spectrum ranges are denoted as follows:

- 824-835MHz: n5R UL

- 869-880MHz: n5R DL

- 904-915MHz: n8R UL

- 949-960MHz: n8R DL

There are 3 options on the table described in the following table

| option | UL configuration | DL/UL Modes of operation | Spectrum restrictions | issues |
| --- | --- | --- | --- | --- |
| 1 | 1UL n5 only | CA_n5-n8/n5 | no | Multiplexing and Loading of n5DL filter by n8 UL filter |
| 2 | 2UL non simultaneous n5Rx/n8Tx | CA_n5-n8/n5 (option 1) n5/CA_n5-n8 (new 1DL/2UL case) => LS to RAN2 | no | Multiplexing and Loading of n5DL filter by n8 UL filter |
| 3 | 2UL CA_n5-n8R | CA_n5-n8/n5 (option 1) CA_n5-n8/n8R CA_n5-n8/CA_n5-n8R | At least n8R UL | Only supports n8R UL on top of full n8 single band support with at least one dedicated filter |

Observations:

- Option 1 CA_n5-n8/n5 is a subset of option 2 and 3 and does no require any new feature/mode of operation

- Option 2 requires input from RAN2 on new 1DL/2UL case but can support the entire n5 and n8 bands

- Option 3 needs to manage the restricted frequency range at least on n8R UL. To enable operation, it only requires a dedicated filter for n8R UL: n5 UL/DL and n8 DL can be supported entirely

As shown with above observation, option 1 is a low hanging fruit that doe does not require any new feature or dedicated filter and is anyhow a subset of option 1 and 2 modes of operation and thus is a fallback of both. Also the related requirement MSD requirement il anyhow needed for the other two options

For frequency range restrictions on option 3 to maximize the usability other than in China and minimize the RFFE impact in terms of cost and size, it is not useful to restrict the n5 range or n8 DL range. Also minimizing the frequency range restriction justifies that no new bands are introduced

In terms of specification, it is possible to manage those 3 options with different BCS as a starting point for a signaling option with BCS. For option two this is pending RAN2 input on the 1DL/2UL mode of operation:

- BCS0: CA_n5-n8 1UL with a note saying that only n5UL is supported

- BCS1: CA_n5-n8 2UL CA_n5-n8 with a note saying that only non simultaneous n5Rx/n8Tx is supported

- BCS2: CA_n5-n8 2UL CA_n5-n8 with a note saying that only 904-915MHz n8 UL range is supported

- In this case BCS0 is the baseline and BCS1 and 2 optional (note that BCS support is anyhow optional)

## Band combinations to be specified

Tentative agreement from GTW: FFS the down selection. The RF requirements for the 3 options can be discussed before the decision is made.

Way forward/Agreement on band combinations to be specified

- In single band operation, the entire band n5 and n8 frequency ranges must be supported

- In CA_n5-n8 operation the following options are discussed for sepcification:

  - pending RAN2 input on option 2:

    - CA_n5-n8 1UL n5, the entire band n5 and n8 frequency ranges must be supported. This configuration is needed as fallback of the next two 2UL cases.

    - CA_n5-n8 2UL non-simultaneous n5Rx/n8Tx as optional, the entire band n5 and n8 frequency ranges must be supported

    - CA_n5-n8 2UL CA_n5-n8R as optional, the entire band n5 and n8 frequency ranges must be supported except for n8ULthat is restricted to 904-915MHz

  - Further down selection is discussed at next meeting

## Signaling of support for different Band combinations options

Way forward/Agreement on capturing band combinations to be specified and potential signaling needs: following options are discussed as potential signaling solutions:

- Option 1: use different BCS with notes clarifying the UL restrictions based on agreed cases to be specified

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Bandwidth combination set |
| --- | --- | --- | --- |
| CA_n5A-n8A | n5X | n5 | 0 |
|  |  | n8 |  |
|  | CA_n5A-n8AY | n5 | 1 |
|  |  | n8 |  |
|  | CA_n5A-n8AZ | n5 | 2 |
|  |  | n8 |  |
| NOTE X: In BCS0, n8 UL is not supported NOTE Y: In BCS1, the concurrent operation of band n5 DL and band n8 UL is not supported NOTE Z: In BCS2 the band n8 support is restricted to the 904-915MHz frequency range in UL |  |  |  |

  - Channel bandwidth support per UL configuration case is FFS (some case can be above 10MHz CBW)

- Option 2: Optional capability is needed for option 2: CA_n5-n8 2UL non-simultaneous n5Rx/n8Tx

- Option 3: Frequency restriction or New frequency band (for n8) is needed for option 3: CA_n5-n8 2UL with n8 restricted to 904-915MHz

- Option 4: Other are not precluded

- Options are not necessarily exclusive

## Non-simultaneous n5Rx/n8Tx band combinations type

In this case there is two possible DL/UL operating modes

- CA_n5-n8 DL/n5 UL

- n8 DL/CA_n5-n8 UL

It is clear that both 2DL and 2UL are supported but just not simultaneously thus it seems natural that this is still a 2DL/2UL case.

Tentative agreement from GTW: The issue can be discussed after the reply LS from other group is received.

Way forward/Agreement: The issue can be discussed after the reply LS from other group is received.

# CA_n5-n8 UE RF requirements

Tentative agreement from GTW: FFS the down selection. The RF requirements for the 3 options can be discussed before the decision is made.

## Delta T and Delta R

Delta T and Delta R should be specified in such a way that it covers all implementation options and cover filter loading effects and account for filter loading effect or switching in/out some filters and additional filters for

Way forward/Agreement

- Delta T and Delta R should be specified in such a way that it covers all implementation options

- Delta T and Delta R values are specified as follows to cover all implementations for Options 1/2/3:

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] | ΔRIB,c [dB] |
| --- | --- | --- | --- |
| CA_n5A-n8A | n5 | 0.5 | 0.4 |
|  | n8 | 0.5 | 0.4 |

## 1UL MSD Requirements for n5 UL case and other option fallback

Regardless of 1L with n5 only being specified as a separate band combination this is needed as fallback of option 2 and 3.

For all options there is no need for frequency restriction on n5 otherwise a new band could be requested to support this. In this case the n5 UL can be at the top of band n5 and n8 DL at the bottom of n8. In all the cases, there is no need to develop new n5 duplexer and legacy n5 duplexers are suitable and should be supported. Furthermore, n5 could be implemented with an n26 filter as it is the case in many UEs.

Way forward/Agreement

-	Following cross-band MSD test point is evaluated for future meetings based on existing band n5/n26 duplexer band n8 rejection performance and n5 transmitter noise floor and specified if justified

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n8 | 844 | 10 | 15 | 25 (RBstart=27) | 951.5 | 5 | TBD | >ACLR2 |

## 1UL MSD Requirements for dedicated n8R UL filter case

<Way forward>

-	The following 1UL cross band MSD is specified

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n8 | n5 | 909 | 10 | 15 | 25 (RBstart=0) | 877.5 | 5 | [12.3] | ACLR2 |

- A note is added to clarify applicability to the case where n8 UL only supports 904-915MHz frequency range

## 2UL MSD Requirements for non-simultaneous n5Rx/n8Tx case

In the case where the is no frequency restriction in n5 and n8 frequency range, a direct hit IMD3 is possible with similar cases at [25]dB.

<Way forward>:

-	The Following IMD3 MSD test point is evaluated for future meetings and specified if justified

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 846.5 | 5 | 25 (RBstart=0) | N/A | N/A | FDD | N/A |
|  | n8 | 892.5 | 5 | 25 (RBstart=0) | 937.5 | [TBD] | FDD | IMD3 |

- A note is added to clarify applicability to the case where simultaneous n5Rx/n8Tx is not supported

## 2UL MSD Requirements for dedicated n8R UL filter case

In the case where the is frequency restriction on n8 UL frequency range, the IMD3 of n5UL at the top of n5 and n8UL at the bottom of n8R UL falls just above n8 DL.

<Way forward>:

-	The Following near miss IMD3 MSD test point is evaluated for future meetings and specified if justified

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n5-n8 | n5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | n8 | 906.5 | 5 | 25 | 951.5 | TBD | FDD | Near miss IMD3 |

- A note is added to clarify applicability to the case where n8 UL only supports 904-915MHz frequency range

# References

R4-2304168 	CA_n5-n8 handling	Nokia, Nokia 	Shanghai Bell

R4-2304353	CA_n5-n8 UE architecture and RF requirements	Apple

R4-2304435	Discussion on the solutions for CA_n5-n8	CATT

R4-2304454	Considerations on CA_n5-n8	Qualcomm Finland RFFE Oy

R4-2304563	Input to CA_n5-n8 for LBLB and LBLBLB band combinations WI	Skyworks Solutions Inc.

R4-2305073	Discussion on UE RF requirements and related transmission schemes for CA_n5-n8	vivo

R4-2305131	Discussion and draft LS on CA band combination of n5-n8	ZTE Corporation

R4-2305152	Discussion on options for overlap in CA_n5-n8	China Telecom

R4-2305253	Discussion on CA_n5-n8	Xiaomi

R4-2305379	Discussion on candidate solutions and RF requirements for CA_n5-n8	Huawei, HiSilicon