3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2306468

Online, April 17th – 26th, 2023

Title: 	WF on CA_n5-n28 and CA_n8-n20-n28

Agenda Item:	4.30.3

Source: 	Huawei, HiSilicon

Document for:	Approval

1 Background

A new follow-up WI [1] for Enhancement for 700/800/900MHz band combinations was approved in RAN#99 meeting after the completion of SI. Based on the outcomes of TR 38.872 [2], there are some remaining issues to be further discussed for CA_n5-n28 and CA_n8-n20-n28 in work item phase. Companies provided a lot of papers [3~7] to discuss the Tib/Rib, MSD due to one UL band cross band isolation and two UL bands cross band isolation for CA_n5-n28. Companies also provided the contributions [8~10] to discuss the Tib/Rib, MSD due to two IMD issue for CA_n8-n20-n28.

Based on contributions and offline GTW discussion, it seems that companies are very close to reach a consensus. In this paper, we’d like to address these remaining issues for CA_n5-n28 and CA_n8-n20-n28.

2 Remaining issues for CA_n5-n28

Issue 3-1: Tib/Rib for CA_n5-n28

<Agreement>: Option 1: Table 5.2.2.4-0 in TR 38.872

Table 5.2.2.4-0: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0.7 |
|  | n28 | 0.7 |

Table 5.2.2.4-0a: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0.2 |
|  | n28 | 0.2 |

Issue 3-2: MSD for 1UL CA_n5-n28

<Agreement>: The following MSD test point for cross band isolation issue with one UL band is agreeable.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |

Issue 3-3: MSD for 2UL CA_n5-n28

<Agreement>: The following MSD test point for cross band isolation issue with one UL band is agreeable by averaging the values in TR 38.872. RAN4 agrees to include a generic sentence into 2UL non-IMD table, which describes the generic principle for MSD in case of no IMD but still nonzero 2UL cross-band CA (i.e. the widest BW’s are used for both bands).

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 3.1 | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

3 Remaining issues for CA_n8-n20-n28

Issue 3-1: Tib/Rib for CA_n8-n20-n28

<Agreement>: Option 2 is agreeable considering the bigger values in TR 38.872

Table 5.3.5.3-3: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.8 |
|  | n20 | 0.7 |
|  | n28 | 0.7 |

Table 5.3.5.3-4: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n8-n20-n28 | n8 | 0.3 |
|  | n20 | 0.2 |
|  | n28 | 0.2 |

Issue 3-2: MSD for CA_n8-n20-n28

<Agreement>: The following REFSENS exceptions due to IMD interference is agreeable.

REFSENS exceptions due to IMD interference for CA_n8-n20-n28

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode | of IMD |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n8-n20-n28 | n8 | N/A | 5 | 25 | 951.5 | 24.3 | FDD | IMD3 |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | 717.5 | 5 | 25 | 772.5 | N/A | FDD | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n20 | 834.5 | 5 | 25 | 793.5 | N/A | FDD | N/A |
|  | n28 | N/A | 5 | 25 | 781.5 | 24 | FDD | IMD3 |

Reference

[1] RP-230764, New WID on enhancement for 700/800/900MHz band combinations, CATT, China Telecom

[2] TR 38.872, Study on enhancement for 700/800/900MHz band combinations

[3] R4-2304453	Considerations on CA_n5-n28, Qualcomm Finland RFFE Oy

[4] R4-2304564	Input to CA_n5-n28 for LBLB and LBLBLB band combinations WI, Skyworks Solutions Inc.

[5] R4-2305132	Views on RF requirements for CA band combination of n5-n28, ZTE Corporation

[6] R4-2305254	Discussion on CA_n5-n28, Xiaomi

[7] R4-2305377	Discussion on RF impacts and requirements for CA_n5-n28, Huawei, HiSilicon

[8] R4-2304565	Input to CA_n8-n20-n28 for LBLB and LBLBLB band combinations WI, Skyworks Solutions Inc.

[9] R4-2305255	Discussion on CA_n8-n20-n28, Xiaomi

[10] R4-2305378	Discussion on RF impacts and requirements for CA_n8-n20-n28, Huawei, HiSilicon