3GPP TSG-RAN WG4 Meeting #106-bis-e		R4-2306476

Electronic Meeting, 17th  –26th  April, 2023

Title:	LS reply on switching time for DL PRS or UL SRS frequency hopping for RedCap UEs

Response to:

Release:	Rel-18

Work Item:	NR_pos_enh2

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

Name:	Chunhui Zhang

E-mail Address:	Chunhui (dot) Zhang (at) ericsson (dot) com

Attachments:

1. Overall Description:

RAN4 thanks RAN1 for LS R1-2302127. RAN4 has discussed the question in the LS and reached the following agreements:

- For RedCap UE UL SRS Tx frequency hopping, RAN4 considers the switching time of {70us, 140us} for FR1 as the starting point

  - SRS Tx frequency hopping range can be up to 100MHz.

  - Which specific value for frequency hopping is applied depends on UE capability, if multiple values are agreed.

- For RedCap UE DL PRS Rx frequency hopping, RAN4 considers the switching time of {70us, 140us} for FR1 as the starting point

  - PRS Rx frequency hopping range can be up to 100MHz

  - Which specific value for frequency hopping is applied depends on UE capability, if multiple values are agreed

RAN4 will notify RAN1 of further relevant agreements if there are any updates in future meetings.

RAN4 also thinks additional switch time may be needed for SRS transmission between the initial/active BWP to the first hop and last hop to initial/active BWP, and therefore, has below question:

- Is the additional switch time for SRS transmission between the initial/active BWP to first hop and switch time between last hop to the initial/active BWP relevant for RedCap frequency hopping and should it be discussed in RAN4?

2. Actions:

To RAN WG1 group.

ACTION: 	RAN4 kindly requests that RAN1 consider the above information in its future work and provide answer for question above.

3. References:

4. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #107		May 22 – 26, 2022				Incheon

TSG RAN WG4 Meeting #108		August 21 – 25, 2023				Toulouse