3GPP TSG-RAN4 Meeting #106-bis-e	R4-2306479

# Online, 17th – 26th April 2023

Title: 	WF on CA_n71-n85 and related band 12 and 71 combinations

Agenda Item:	4.1

Source: 	T-Mobile USA

Document for:	Approval

# MSD for combinations with B71/n71 and B85/n85 or B12/n12

## MSD for New larger CBW for n71 in CA_n71-n85

Agreements:

•	20MHz UL MSD uses averages of P1,2,5 with 8.2dB MSD for n85 and 3.8dB MSD for n71

•	Agreements on MSD test point for 30MHz UL postponed until August in case symmetric 35 MHz is added

•	How to specify both 20 MHz and 30 MHz test points is FFS

•	Companies to confirm that support for n71 UL CHBWs are indicated via the channelBWs-UL IE in UE capabilities, and that support for UL 25 and UL 30 will indicate support for symmetric 25 and 30 MHz with no new signalling needed

•	If a UE supports UL 30 MHz CHBW the 30 MHz CA_n71-n85 MSD test point applies.

•	If 30MHz is kept as maximum UL BW, the following test point to be used:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2 | ACLR2 |
| n71 | n85 | 683 | 30 | 15 | 20 (Rbstart=140) | 730.5 | 5 | TBDX | ACLR2 |
| n85 | n71 | 705.5 | 15 | 15 | 20 (Rbstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |
| NOTE X: n71 30MHz UL channel bandwidth support is optional in this release, applicable to UE signaling support for optional symmetrical 30MHz UL/DL configuration |  |  |  |  |  |  |  |  |  |

## MSD for DC_71_n12 and DC_12_n71

Agreements:

•	20MHz UL MSD uses averages of P1,2,5 (from CA_n71-n85) with 8.2dB MSD for 12/n12 and 3.8dB MSD for 71/n71

•	The following test point is to be used for 20 MHz UL:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | 12 | 683 | 20 | 15 | 20 (RBstart=86) | 731.5 | 5 | 8.2 | ACLR2 |
| 71 | n12 | 683 | 20 | 15 | 20 (RBstart=86) | 731.5 | 5 | 8.2 | ACLR2 |
| 12 | n71 | 706.5 | 15 | 15 | 20 (RBstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |
| n12 | 71 | 706.5 | 15 | 15 | 20 (RBstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |

## MSD for CA_n12-n71

Agreements:

•	20MHz UL MSD uses averages of P1,2,5 (from CA_n71-n85) with 8.2dB MSD for n12 and 3.8dB MSD for n71

•	ΔTIB,c of 0.8dB and ΔRIB,c of 1dB is also captured for n71 and n12

•	The following test point is to be used:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n12 | 683 | 20 | 15 | 20 (RBstart=86) | 731.5 | 5 | 8.2 | ACLR2 |
| n12 | n71 | 706.5 | 15 | 15 | 20 (RBstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |