3GPP TSG-RAN WG4 Meeting #xxx	R4-2306482

<Meeting Location>, <Date> ‒ <Date>, 2023

Title: 	WF on issues with non-contiguous NRU UL CCs

Agenda Item:	4.1

Source: 	Skyworks Solutions, Inc., BT-plc

Document for:	Approval

# Non-contiguous intra-band ULCA in NR-U band

There currently a number of requests for NRU-NRU band combinations of CA_n46-n102 for which the following UL configurations are requested:

- 1UL 1CC case n46 or n102: since BCS only request up 80MHz the A-MPR are in place for those UL configurations

- Inter-band combinations can be completed based on these UL configuration

- 1UL two contiguous CC CA_n46C, CA_n102B and CA_n102C: MPR is available for these and A-MPR work is starting for the European NS

- Inter-band combinations can be completed once A-MPR is specified

- 1UL two non-contiguous CC CA_n46(2A) and CA_n102(2A): there is no specification for MPR nor A-MPR and is a new feature that is not planned in any R18 WI and this work is not suitable within a basket WI

- Inter-band combinations with these non-contiguous UL intra-band NRU ULCA are on hold until there appropriate MPR and A-MPR is agreed in RAN in a non-spectrum WI. The basket status can be updated accordingly

- 2UL 2CC CA_n46A-n102A: there is no agreement on applicable architecture 1PA/2PA, applicable frequency separation, and many IMD issues:

  - In band (n46 and n102 NS emissions restrictions) nor a clear OOB requirement or mask

  - OOB for coexistence with neighbor licensed bands  n79 and n77/78 at -50dBm/MHz

  - Although WiFi 7 has agreed to enable multi-link operation (MLO) this is only deployed on access points that have multiple UL antennas and dedicated filters to filter the different sub-bands

  - This is not compatible with current smartphone implementations with a single PA for n46+n012/n96 and no sub-band filters (and this is both for WiFi and is implemented for NR-U)

  - Note that CA_n46-n96 also only supports 1UL

- 2UL 3CC CA_n46A-n102B/C: there is no agreement on applicable architecture 1PA/2PA, and on top of above cases creates triple beat issues that may be an issue for NR-U OOB emissions in n46 and n102 NS

Given the above considerations the following table summarizes the NRU-NRU band combinations, their UL configurations and possible handling.

| DL configuration | UL configurations |  |  |
| --- | --- | --- | --- |
|  | UL configurations requested in WI | UL configurations to priorities | UL configurations FFS |
| CA_n46A-n102A | CA_n46A-n102A | 46A and n102A | CA_n46A-n102A |
| CA_n46A-n102(2A) | CA_n46A-n102A | 46A and n102A | CA_n46A-n102A |
| CA_n46A-n102B | CA_n102B CA_n46A-n102A/B | CA_n102B after A-MPR is specified for all relevant NS (at least Europe) | CA_n46A-n102A CA_n46A-n102B |
| CA_n46A-n102C | CA_n102C CA_n46A-n102A/C | CA_n102C after A-MPR is specified for all relevant NS (at least Europe) | CA_n46A-n102A CA_n46A-n102C |
| CA_n46A-n102D | CA_n46A-n102A | 46A and n102A | CA_n46A-n102A |
| CA_n46A-n102E | CA_n46A-n102A | 46A and n102A | CA_n46A-n102A |
| CA_n46(2A)-n102A | CA_n46A-n102A | 46A and n102A | CA_n46A-n102A |
| CA_n46(2A)-n102(2A) | CA_n46A-n102A | 46A and n102A | CA_n46A-n102A |
| CA_n46(2A)-n102B | CA_n102B CA_n46A-n102A/B | CA_n102B after A-MPR is specified for all relevant NS (at least Europe) | CA_n46A-n102A CA_n46A-n102B |
| CA_n46(2A)-n102C | CA_n102C CA_n46A-n102A/C | CA_n102C after A-MPR is specified for all relevant NS (at least Europe) | CA_n46A-n102A CA_n46A-n102C |
| CA_n46(2A)-n102D | CA_n46A-n102A | 46A and n102A | CA_n46A-n102A |
| CA_n46(2A)-n102E | CA_n46A-n102A | 46A and n102A | CA_n46A-n102A |
| CA_n46C-n102A | CA_n46C CA_n46A/C-n102A | CA_n46C after A-MPR is specified for all relevant NS (at least Europe) | CA_n46A-n102A CA_n46C-n102A |
| CA_n46C-n102(2A) | CA_n46C CA_n46A/C-n102A | CA_n46C after A-MPR is specified for all relevant NS (at least Europe) | CA_n46A-n102A CA_n46C-n102A |
| CA_n46C-n102B | CA_n46C CA_n102B CA_n46A-n102A/B CA_n46C-n102A | CA_n46C CA_n102B after A-MPR is specified for all relevant NS (at least Europe) | CA_n46A-n102A CA_n46A-n102B CA_n46C-n102A |
| CA_n46C-n102C | CA_n46C CA_n102C CA_n46A-n102A/C CA_n46C-n102A | CA_n46C CA_n102C after A-MPR is specified for all relevant NS (at least Europe) | CA_n46A-n102A CA_n46A-n102C CA_n46C-n102A |
| CA_n46C-n102D | CA_n46C CA_n46A/C-n102A | CA_n46C after A-MPR is specified for all relevant NS (at least Europe) | CA_n46A-n102A CA_n46C-n102A |
| CA_n46C-n102E | CA_n46C CA_n46A-n102A CA_n46C-n102A | CA_n46C after A-MPR is specified for all relevant NS (at least Europe) | CA_n46A-n102A CA_n46C-n102A |
| CA_n46D-n102A | CA_n46A-n102A | 46A and n102A | CA_n46A-n102A |
| CA_n46D-n102(2A) | CA_n46A-n102A | 46A and n102A | CA_n46A-n102A |
| CA_n46D-n102B | CA_n102B CA_n46A-n102A/B | CA_n102B after A-MPR is specified for all relevant NS (at least Europe) | CA_n46A-n102A CA_n46A-n102B |
| CA_n46D-n102C | CA_n102C CA_n46A-n102A/C | CA_n102C after A-MPR is specified for all relevant NS (at least Europe) | CA_n46A-n102A CA_n46A-n102C |
| CA_n46D-n102D | CA_n46A-n102A | 46A and n102A | CA_n46A-n102A |
| CA_n46D-n102E | CA_n46A-n102A | 46A and n102A | CA_n46A-n102A |

Way forward/Agreement

- NRU-NRU Inter-band CA with 1UL 1CC can be completed (n46A, n96A, n102A UL configurations)

- NRU-NRU Inter-band CA with 1UL contiguous 2CC can be completed once their ULCA AMPR are specified (CA_n46B/C, n86/B/C, n102B/C)

- NRU-NRU Inter-band CA with 1UL with non-contiguous 2CC cannot be evaluated until a RAN agreement enables the development of the related transmitter requirements in a non-spectrum WI (not in a basket): for example MPR, A-MPR, bandwidth separation class, SEM. (CA_n46(2A), CA_n96(2A) and CA_n102(2A)

- Related UL configurations are removed from the intra and two band baskets WI

- NRU-NRU Inter-band CA with 2UL NRU-NRU with 2 or 3CC (CA_n46-n102, CA_n46-n96 w/wo 2CC in one band) need further discussion and justification in terms of smartphone UE hardware

- These are put on hold until there is an agreement to support, for which type of UE and form factor constraints, PA and filter architecture

# References

R4-2305781	Issues with 2UL NRU-NRU inter-band combinations and invalid intra-band non-contiguous ULCA requests	Skyworks Solutions Inc.