3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2306493

Electronic Meeting, 17 April – 26 April 2023

Source:	Ericsson, BT plc

Title:	TP for TR 37.718-21-11: Including band combinations DC_20-67_n3

Agenda item:			4.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.718-21-11 to add DC_20-67_n3 configurations.

# 2. Text Proposal

---Start of changes---

## 5.x	DC_20-67_n3

### 5.x.1	Configuration for DC

Table 5.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_20A-67A_n3A | DC_20A_n3A | CA_20A-67A | n3 |

### 5.x.2	Co-existence studies

There are IMD4 impact from UL 20_n3 affecting DL band 67.

### 5.x.3		∆TIB and ∆RIB values

For DC_20-67_n3, the TIB,c and RIB,c values are reused from DC_20_n3-n67 and are given in the tables below.

Table 5.1.3-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_20-67_n3 | 0.5 | - | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.1.3-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_20-67_n3 | 0.1 | 0.1 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.x.4	REFSENS requirements

Based on the co-existence studies there is a need to define MSD value.

The MSD value for band n67 is reused from DC_20_n3-n67.

| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20-67_n3 | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | 67 | N/A | 5 | 25 | 746 | 9.4 | IMD4 |
|  | n3 | 1765 | 5 | 25 | 1860 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-230473, Revised WID: Rel-18 Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei, HiSilicon