3GPP TSG-RAN WG4 Meeting #106bis-e                                                    	R4-2306504

Electronic Meeting, 17– 26 April, 2023

Source:	Huawei, Hisilicon

Title:	TP to TR 38.718-02-01 Addition of CA_n3A-n7A

Agenda item:	 	4.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.718-02-01 to include CA_n3A-n7A with UL configuration CA_n3A-n7A BCS 4 & 5.

During the previous meeting，some companies advised to double-check the cross-band isolation for CA_n3A-n7A to determine whether it should align with CA_n3-n41. In this paper, we’d like to discuss and address this issue.

2 Discussion

We use main/diversity antenna topology as RF front-end architecture in our analysis, as shown in figure 1. It is noted that the use of quad-plexer is also a valid approach in the implementation of CA_n3-n7 in practice.

Figure 1 main/diversity antenna topology

In comparison to the n41 RX filter operating within the 2496-2690 MHz, the n7 RX filter operates within the 2620-2690 MHz which is significantly distant from the n3 TX transmission frequency of 1710-1785 MHz.  As a result, it is easier for the n7 RX filter to achieve high suppression at the n3 transmission frequency, compared to the n41 filter that operates closer to the n3 TX frequency range.

The attenuation of n41 filter can be shown in the Figure 2（a） and （b）. The minimum attenuation for band n3 TX frequency range is 20dB.

Figure 2 (a) Electric Characteristic of n41 post PA filter

Figure 2 (b) Electric Characteristic of n41 post PA filter

The attenuation of n7 filter can be shown in the Figure 3 (a) and (b). The minimum attenuation value at the band n3 TX frequency range is 40dB.

Figure 3  (a) Electric Characteristic of n7 Ant. →RX filter

Figure 3  （b）Electric Characteristic of n7 Ant. →RX filter

It can be seen that n7 duplexers have good attenuation at n3 TX frequency range than that of n41, and cross-band isolation is not an issue for CA_n3-n7. The crossband isolation MSD have been defined in the Table 7.3A.6-1 within the current TS38.101-1，and the same MSD value can be applied to BCS 4 and 5.

Table 7.3A.6-1 in [TS38.101-1]: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n7 | n3 | 2525 | 50 | 15 | 45 (RBstart=0) | 1877.5 | 5 | 0.6 | >ACLR2 |

Now let us consider the spurious emission from the UL of n7 to the DL of n3. It can be found that the minimum attenuation value for band n3 RX filter is 36 dB at the UL frequency range of n7 in Figure 4 (a) and (b).

Figure 4 (a）Electric Characteristic of n3 Ant. →RX filter

Figure 4 （b）Electric Characteristic of n3 Ant. →RX filter

Table 1 summarize the MSD analysis results for victim Rx of band n7 for CA_n3-n7.

Table 1 MSD calculation for victim Rx of band n7

| Reference sensitivity power levelfor n7 （5MHz channal BW）（dBm） | -98 |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Unit | parameter | Main | Diversity |
| n7 TX power at antenna port (Primary path) | dBm | 23 |  |  |
| n7 PA output noise power at n3 freq | dBm/Hz |  | -130 | -130 |
| Antenna isolation Primary <-> Diversity |  | 10 |  |  |
| n3 Rx bandwidth | MHz | 5 |  |  |
| n7 PA output noise power |  |  | -63.01 | -63.01 |
| n3 Ant to RX port on n7 TX freq | dB | 36 |  |  |
| Diplexer loss, dB |  | 1.5 |  |  |
| diplexer out of band isolation | dB | 10 |  |  |
| n7 PA noise power at DPX- COM port |  |  | -109.01 | -120.51 |
| front end loss |  | 4 |  |  |
| n7 PA noise power at ant port |  |  | -105.01 | -116.51 |
| noise figure dB | dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) |  | -165 |  |  |
| Thermal noise, dBm |  |  | -98.01 | -98.01 |
| TX IM2 noise dBm |  |  | -94 | -116 |
| totle noise level at ANT port, dBm |  |  | -92.31 | -97.88 |
| SNR requirement for QPSK |  | -1 |  |  |
| REFSENSE (referred to antenna) (5MHz BW) |  |  | -93.31 | -98.88 |
| combined REFSENS(5MHz BW), dBm |  |  | -99.94 |  |
| Margin | dB | 4 |  |  |
| combined REFSENS(5MHz BW) after considering Margin, dBm |  |  | -95.94 |  |
| MSD | dB |  | 2.06 |  |

Based on the analysis above, the MSD is 2.1dB. But the MSD have been defined in the Table 7.3A.6-1 within the current TS38.101-1, and the same MSD value can be applied to BCS 4 and 5.

Text Proposal

<TP for TR 38.718-02-01>

<Start of Text Proposal>

5.x	CA_n3A-n7A

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band n3+n7

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n3+n7

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n7A | CA_n3A-n7A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |

5.x.1.3	UE co-existence studies

The harmonic / harmonic mixing issue have been analyzed in previous release.

5.x.1.4	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c have been specified in current spec.

5.x.1.5	REFSENS requirements

For CA_n3A-n7A, there are no harmonics or harmonic mixing issues. Therefore, MSD due to harmonic or harmonic mixing is not needed. Since n7 is very close to Wi-Fi spectrum, the filter needs to provide enough rejection to Wi-Fi. This may impact the filter rejection to n3. However, in comparison to the n41 RX filter operating within the 2496-2690 MHz, the n7 RX filter operates within the 2620-2690 MHz which is significantly distant from the n3 TX transmission frequency of 1710-1785 MHz. As a result, it is easier for the n7 RX filter to achieve high suppression at the n3 transmission frequency, compared to the n41 filter that operates closer to the n3 TX frequency range.

Commercially available n7 duplexer has the attenuation for band n3 TX frequency range better than 40dB. The cross band isolation MSD have been defined in the Table 7.3A.6-1 within the current TS38.101-1，and the same MSD value can be applied to BCS 4 and 5.

5.x.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirements

<End of Text Proposal>