3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2306509

Electronic Meeting, 17 – 26 April 2023

Source:	Ericsson, Rogers

Title:	TP for TR 38.718-02-01: to include band combination CA_n7A-n71A

Agenda item:			4.10.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.718-02-01to add CA_n7A-n71A configuration as specified in WID [1].

# 2. Text Proposal

---Start of changes---

## 5.X	CA_n7-n71

### 5.X.1	Common for 1 band UL and 2 bands UL CA

#### 5.X.1.1		Operating bands for CA

Table 5.X.1.1-1: CA band combination of band n7 and n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500MHz | – | 2570 MHz | 2620MHz | – | 2690 MHz | FDD |
| n71 | 663MHz | – | 698MHz | 617MHz | – | 652MHz | FDD |

#### 5.X.1.2		Channel bandwidths per operating band for CA

Table 5.X.1.2-1: Supported bandwidths per CA band combination of band n7+n71

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n71A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |

#### 5.X.1.3		UE co-existence studies

Table 5.X.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n7-n71. It is shown:

- 4th order harmonic of Band n71 Tx may fall into the own Rx Band of Band n7.

- 4th order harmonic mixing for Band n71

Table 5.X.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |

Table 5.X.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 | 30851 | 3260 |

#### 5.X.1.4		∆TIB and ∆RIB values

For CA_n7-n71, the TIB,c and RIB,c values are reused from DC_7_n71 as specified in TS 38.101-1 and are given in the tables below.

Table 5.X.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n7-n71 | 0.6 | 0.3 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.X.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n7-n71 | 0.2 | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.X.1.5 		REFSENS requirements

As the co-existence studies in 5.X.1.3 show, there are 4th order harmonic and 4th order harmonic mixing, so MSD is required. MSD values are reused from DC_7_n71 and CA_n5-n77.

Table 5.X.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n71 | n71 | 5 | 15 | 8 (RBstart=0) | 5 | 14.6 | NOTE 4 | UL4/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 5.X.1.5-3: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n7 | n71 | 5 | 15 | 25 (RBstart=0) | 5 | 5.7 | NOTE 8 | UL1/ DL4 |
| NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

#### 5.X.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

---End of changes---

# Reference

[1]		RP-230133, Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE