3GPP TSG-RAN WG4 Meeting #106bis-e                           R4-2306534

Electronics meeting, 17th-26th, April, 2023

Source:	Samsung, KDDI, Qualcomm

Title:	TP for TR 38.898: DC_1A_n41A

Agenda item:			4.17.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.898 to include DC_1A_n41A according to the request in [1].

2. Reference

[1] Revised WID on High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL)

# 3. Text Proposal

<Start of Text Proposal>

### 5.1	DC_1_n41

#### 5.1.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

#### 5.1.2	Maximum output power for DC

Table 5.1.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

#### 5.1.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC.

- The 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n41.

- The 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of band 1.

- The 2nd, 3rd, 4th, and 5th order IMD do not fall into Rx frequencies of band 1 and n41.

- Cross band isolation existing, the interference from band1 UL falls into n41 DL, and the interference from n41 UL also falls into band 1 DL.

The new MSD values are defined in below table.

Table 5.x.3-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1 | n41 |  | 8.5 | 8.5 | 8.5 |  | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 |
| n41 | 1 | 11.8 | 11.8 | 11.8 | 11.8 |  |  |  |  |  |  |  |  |  |

Table 5.x.3-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 1 | n41 | 15 |  | 100 | 100 | 100 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n41 | 1 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |

#### 5.1.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so there is no additional values.

<End of Text Proposal>