3GPP TSG-RAN4 Meeting #106-bis-e	R4-230xxxx

# Online, 17th – 26th April 2023

Title: 	WF on adding new channel bandwidths support to existing NR bands

Agenda Item:	4.26

Source: 	Ericsson

Document for:	Approval

# Band n71

## How to handle new UL BW for existing DL BW

Agreements:

Existing 25MHz and 30MHz REFSENS with 20MHz UL is maintained as the baseline requirement in the REFSENS Table and applies for legacy UEs

The symmetrical UL/DL 25MHz and 30MHz CBW is optional in R18 (and earlier) and mandatory from Rel-19.

The symmetrical UL/DL 25MHz and 30MHz CBW REFSENS values are captured in a way that makes it clear it is the optional requirement (using notes for example).

20 MHz UL CBE will still be used for DL CBW=35 MHz.

FFS if any signalling is needed to indicate the optional symmetrical UL/DL CBW support. . Companies to confirm that support for n71 UL CHBWs are indicated via the channelBWs-UL IE in UE capabilities, and that support for UL 25 and UL 30 will indicate support for symmetric 25 and 30 MHz with no new signalling needed.

## PC2 consideration

Agreements:

For Release 18 completeness the PC2 1Tx and 2Tx RSDs should be specified for the optional symmetrical UL/DL 25MHz and 30MHz case in the High power UE for FR1 for FDD single band(s) with power class 2 WI.

PC2 FDD should not be considered complete for n71 until PC2 FDD 1Tx and 2Tx RSD is specified for symmetrical UL/DL 25MHz and 30MHz CBW.

## BCS4/5 consideration

Agreements:

Once the n71 symmetrical UL/DL CBW single band work is finalized, potential new MSD test points and values should be assessed for CA_n71B, CA_n71(2A), CA_n71-n85, DC_5A_n71A, DC_12A_n71A and DC_13A_n71A.

The work should be done within the “new CBW for existing band” WI to complete band n71.

FFS on how to deal with the legacy and new cross-band MSD requirement for the impacted band combinations.

## Delta-MPR

FFS if 0.5 dB delta-MPR should be added for 25 and 30 MHz CBW. This should be technically justified, if needed.

## NS_35 A-MPR

FFS A-MPR needed for 25 and 30 MHz CBW for PC3.

FFS A-MPR needed for 25 and 30 MHz CBW for PC2.

## REFSENS (PC3)

Agreement:

UL RB allocation: 20 RBs at the bottom of the channel for both 25 and 30 MHz CBW

REFSENS is FFS.

Following results were proposed, a tentative agreement would be to average the results:

Option 1: -76.6 dBm and -67.3 dBm can be used for 25MHz and 30MHz CBW respectively with 20RB0 UL allocation.

Option 2: -75.9 dBm and -69.9 dBm for 25 MHz and 30 MHz channels, respectively.

## RSD (PC2)

FFS PC2 RSD for 25 and 30 MHz CBW.

# Band n8

Agreement:

PC2 considerations for band n8 and 25 MHz CBW will be handled in the PC2 FDD WI.

# Band n39 and n98

## PC1.5

Agreement:

PC1.5 considerations for bands n39/n98 and 35 MHz CBW will be handled in this “adding new channel BW in NR bands”.

The request can only be considered completed when NS_50 A-MPR for PC3/PC2 and PC1.5 35MHz are specified.