3GPP RAN WG4 Meeting #106bis-e		R4-2306546

Online, April 17th – 26th, 2023

Title:	WF on the notation for single SUL CA combinations

Agenda item:	4.14.2

WI/SI:	NR_2SUL_cell_combos_R18-Core

Release:	Rel-18

Source:	Apple

Document for:	Approval

# 1	Background

- In RAN #98-e meeting, a new basket work item on “NR CA band combinations with two SUL cells in Rel-18” was proposed [1]. Despite there was a concern on the notation for the initially proposed configurations with NR intra-band CA pairing with two SUL bands to form aggregation of two SUL cells, the work item was proceeded with approval, leaving the notation concern as an open issue to be resolved in the working group.

- In last RAN4 meeting, there were discussions and proposals on how the known notation issue may potentially be resolved and the agreement was captured in the approved “WF on the notation for band combinations with two SUL” [2] where the agreed notation changes are summarized in the table below.

| NR CA configuration (Agreed new notation) | Corresponding old notation in [1] | Uplink CA configuration or SUL configuration |
| --- | --- | --- |
| CA_n41A-n95A_n79A-n98A | CA_SUL_n41A-n95A_SUL_n79A-n98A | SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A |
| CA_ n41A-n98A_n79A-n95A | CA_SUL_n41A-n98A_SUL_n79A-n95A | SUL_n41A-n98A SUL_n79A-n95A CA_n41A-n79A |
| CA_n41A-n83A_n79A-n98A | CA_SUL_n41A-n83A_SUL_n79A-n98A | SUL_n41A-n83A SUL_n79A-n98A CA_n41A-n79A |
| CA_n41A-n83A_n79A-n95A | CA_SUL_n41A-n83A_SUL_n79A-n95A | SUL_n41A-n83A SUL_n79A-n95A CA_n41A-n79A |
| CA_n78C_n81A-n84A | CA_SUL_n78A-n81A_SUL_n78A-n84A | SUL_n78A-n81A SUL_n78A-n84A CA_n78C |
| CA_n78C_n80A-n84A | CA_SUL_n78A-n80A_SUL_n78A-n84A | SUL_n78A-n80A SUL_n78A-n84A CA_n78C |

- The motivations behind the above notation changes are:

  - Intra-band CA configuration (contiguous or non-contiguous) needs to be explicitly presented in the full CA notation instead of relying on UL configuration for indirect indication.

  - The SUL band can already be indicated based on the band number, therefore, the pairing of an SUL band and an NR band without the “SUL” prefix in the notation does not really lose the clarity that the combination is an NR SUL cell.

- In this meeting, it was identified that the existing notations for single SUL CA combinations may also need to be revised based on the following two reasons [3]:

  - For the existing intra-band CA configurations with single SUL cell, such as SUL_41C-n80A and SUL_n41(2A)-n99A, the prefix “CA” should be added as by nature they are CA combinations where the notation should follow the CA convention.

  - For CA with single SUL cell, the prefix “SUL” can be removed to align with the agreed notations for dual SUL CA combinations.

- The proposed notation changes for CA with single SUL cell are summarized in the next section.

# Way forward

Proposal: Notations for single SUL CA combinations are revised as below for the three example band combinations. Further clarification and explanation on notation in the specifications can be considered.

| CA configuration | Existing notation | Proposed notation | Note |
| --- | --- | --- | --- |
| Intra-band contiguous | SUL_n41C-n80A | CA_n41C_n80A | For the two NUL carriers within CA_n41C-n80A, the SUL band n80 is corresponding to one of them in one cell. |
| Intra-band non-contiguous | SUL_n41(2A)-n99A | CA_n41(2A)_n99A | For the two NUL carriers within CA_n41(2A)-n99A, the SUL band n99 is corresponding to one of them in one cell. |
| Inter-band | CA_n1A_SUL_n78A-n80A | CA_n1A_n78A-n80A | For the two NUL carriers within CA_n1A_n78A-n80A, the SUL band n80 is corresponding to n78 in one cell. |

# References

RP-223553 “NR CA band combinations with two SUL cells in Rel-18” CMCC, China Telecom, 3GPP TSG RAN Meeting #98-e, December 12th – 16th, 2022

R4-2303459 “WF on the notation for band combinations with two SUL”, CMCC, 3GPP TSG RAN WG4 Meeting #106, Athens, Greece, February 27th – March 3rd, 2023

R4-2304348 “Notation for SUL CA combinations”, Apple, 3GPP TSG-RAN WG4 Meeting #106bis-e, online, April 17th – 26th, 2023