3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2306547

Online, April 17 – April 26, 2023

Title: 	WF on simultaneous Rx-Tx inter-band combinations

Agenda Item:	4.2

Source: 	Huawei, HiSilicon, Skyworks Solutions Inc.

Document for:	Approval

- Topic 1: Alignment on the NR CA MSD test point with the SUL counterparts

- Issue 1-1: Cross band isolation of simultaneous Rx/Tx for CA_n40-n41 and SUL_n41-n97

< Way forward 1-1 >: The test point, as well as the MSD value of cross band isolation in support of simultaneous Rx/Tx for SUL_n41-n97 is aligned with that of CA_n40-n41

- Issue 1-2: Cross band isolation of simultaneous Rx/Tx for CA_n34-n41 and SUL_n41-n95

< Way forward 1-2 >: The test point, as well as the MSD value of cross band isolation in support of simultaneous Rx/Tx for SUL_n41-n95 is aligned with that of CA_n34-n41

- Topic 2: Simultaneous Rx-Tx and MIMO/SRS-AS for CA_n40-n41

- Proposal 1: Discuss the following options and implications for simultaneous Rx-Tx and MIMO with CA_n40-n41:

    - Option 1b: A new optional signalling could be introduced to indicate that a UE supporting MIMO and simultaneous Rx/Tx cannot support both features at the same time for a specific combination.

      - MediaTek

    - Option 1c: Relax the 4Rx requirement of n41 if carrier aggregation is configured with band n40 and simultaneous Rx-Tx. This relaxation would only be applicable to handheld devices and allow improved performance of simultaneous Rx-Tx together with MIMO operation on band n40 and n41.

      - Apple, Huawei

- Proposal 2 (MediaTek): For CA_n40A-n41A, UE is not expected to enable UL MIMO/TxD or SRS antenna switching at the same time when enabling simultaneous Rx/Tx operation

      - MediaTek

- Proposal 3: Extend the scope of LS [1] to ask clarifications on SRS-AS scheduling restrictions or even on disabling SRS-AS [3] to TDD-TDD band combinations (e.g., CA_n40-n41) with simultaneous RxTx operation.

      - Skyworks, Qualcomm, Huawei

<Way forward 2-1>: For UE supporting CA_n40A-n41A with simultaneous Rx/Tx together with MIMO operation on band n40 and n41

- Further discussion in the next meeting

< Way forward 2-2>: Scheduling restriction for SRS antenna switching when UE indicates txSwitchImpactToR as clarified in LS to RAN1 [R4-2303633] is also applicable for TDD-TDD band combinations (e.g. CA_n40-n41) in simultaneous Rx/Tx operation

- FFS on spec impact

- The affected TDD-TDD band combination should be studied case by case