3GPP TSG-RAN WG4 Meeting # 106-bis-e										R4-2306557

Electronic Meeting, 17 April –26 April, 2023

Title: 	WF on NR-U enhancement topics

Agenda Item:	4.29.6

Source: 	Apple

Document for:	Approval

# Harmonisation

<Way Forward>: FCC is close to finalizing VLP requirements. The open FNPRM proposes -8 dBm PSD limit and 14 dBm EIRP max for VLP. Those are the same requirement specified for Canada and Dominican Republic. It is proposed to include the requirements by tentatively specifying NS_y3 for US VLP.

Updated NS flag table:

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | [NS_y1] |
| UK | N/A | NS_01 | [NS_y2] |
| Morocco | N/A | NS_01 | [NS_y2] |
| UAE | N/A | NS_01 | N/A |
| Saudi Arabia | N/A | NS_01 | N/A |
| Kenya | N/A | NS_01 | [NS_y5] |
| Qatar | N/A | NS_01 | [NS_y2] |
| Jordan | N/A | NS_01 | [NS_y2] |
| Russian Federation | N/A | NS_01 | [NS_y5] |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | [NS_y3] |
| Canada | NS_54 | NS_59 | [NS_y3] |
| Brazil | N/A | NS_53 | [NS_y4] |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | [NS_y2] |
| Colombia | N/A | NS_53 | N/A |
| Dominican Republic | N/A | [NS_60] | [NS_y3] |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |
| Hong Kong | N/A | NS_58 | [NS_y1] |
| Australia | N/A | NS_01 | [NS_y5] |
| New Zealand | N/A | NS_01 | [NS_y5] |
| Malaysia | N/A | NS_01 | [NS_y2] |
| Japan | N/A | [NS_x1] | [NS_y6] |

# NR-U A-MPR

Japan LPI and VLP A-MPR

<Agreement>: The following A-MPR and emission requirements are agreeable:

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3, Note 4) |  |
| --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | See Table PC5 MPR | ≤ 2.0 | ≤ 3.0 |
|  | QPSK |  | ≤ 2.5 | ≤ 3.5 |
|  | 16 QAM |  | ≤ 3.0 | ≤ 3.5 |
|  | 64 QAM |  | ≤ 3.5 | ≤ 4.5 |
|  | 256 QAM |  | ≤ 5.0 | ≤ 5.5 |
| CP-OFDM | QPSK |  | ≤ 4.5 | ≤ 5.0 |
|  | 16 QAM |  | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM |  | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3 and NOTE 4. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to [6415 MHz], 40 MHz channels centered at the nearest NR-ARFCN corresponding to [6405 MHz] and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [6385 MHz]. NOTE 5: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 6: Channel bandwidth sizes of 60MHz and 100MHz are not applicable for this network signalling. |  |  |  |  |

EU/CEPT and Hong Kong

<Way forward>: If companies provide re-evaluation next meeting, then discuss the new proposals otherwise increase values of Partial Allocations (highlighted in red) to meet Full allocations. Values are captured in rolling draft with [].

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 12 | ≤ 14 | ≤ 8 | ≤ 10 |
|  | QPSK | ≤ 12 | ≤ 14 | ≤ 8 | ≤ 10 |
|  | 16 QAM | ≤ 12 | ≤ 15 | ≤ 9 | ≤ 10 |
|  | 64 QAM | ≤ 12 | ≤ 15 | ≤ 11 | ≤ 11 |
|  | 256 QAM | ≤ 13 | ≤ 15 | ≤ 13 | ≤ 14 |
| CP-OFDM | QPSK | ≤ 13 | ≤ 15 | ≤ 10 | ≤ 10 |
|  | 16 QAM | ≤ 13 | ≤ 15 | ≤ 11 | ≤ 10 |
|  | 64 QAM | ≤ 13 | ≤ 15 | ≤ 13 | ≤ 12 |
|  | 256 QAM | ≤ 15 | ≤ 15 | ≤ 15 | ≤ 15 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 and 5995 MHz and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |

<Agreement>: The following emission requirements for EU/CEPT and Hong Kong are agreeable:

UK, Morocco, Qatar, Jordan and Malaysia

<Agreement>: The A-MPR for UK, Morocco, Qatar, Jordan and Malaysia is agreeable:

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ 6.0 | ≤ 6.0 |
|  | QPSK | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 6.0 | ≤ 6.0 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 6.5 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

Canada and Dominican Republic

<Agreement>: The A-MPR for Canada and Dominican Republic is agreeable:

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | QPSK | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | 16 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | 64 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | 256 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
| CP-OFDM | QPSK | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | 16 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | 64 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
|  | 256 QAM | ≤ 16.0 | ≤ 18.5 | ≤ 12.5 | ≤ 15.5 | ≤ 11.0 | ≤ 14.0 | ≤ 9.5 | ≤ 12.5 | ≤ 8.5 | ≤ 11.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

Brazil

<Agreement>: The A-MPR for Brazil is agreeable:

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | QPSK | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 16 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 64 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 256 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
| CP-OFDM | QPSK | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 16 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 64 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
|  | 256 QAM | ≤ 13.0 | ≤ 15.5 | ≤ 9.5 | ≤ 12.5 | ≤ 8.0 | ≤ 11.0 | ≤ 6.5 | ≤ 9.5 | ≤ 6.0 | ≤ 8.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

Australia, New Zealand and Kenya

<Agreement>: The A-MPR for Australia, New Zealand and Kenya is agreeable:

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | QPSK | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.0 | ≤ 9.5 | ≤ 6.0 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

# Full/Partial RB allocation

<Way forward>: Further discuss streamlining of definitions for Full and Partial RB allocations and use the following as starting point

  - Full RB allocation:

    - when all RB’s in a 20 MHz channel, or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted, or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configurations in Table 6.2F.2-2

  - Partial RB allocation:

    - when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted or when sub-bands are transmitted according to configurations in Table 6.2F.2-2

# Further discussion on NR-U PC3 MPR

<Way forward>: Agreed MPR is starting point and changes are considered in case issues are identified.

- Companies are requested to bring simulation or measurement data allowing to identify oversights and issues with current MPR agreement.

# Extended range of NS values

One of the open issues from the previous RAN WG4 discussions was a technical aspect on how a UE belonging to the earlier release can implement NS flags from later releases and how the network can know about it so that the right NS flags can be signalled. The modifiedMPR-Behavior IE could be used to indicate support for new NS flags. Concerns have been raised with this approach and request for clarification as the network needs to make use of the indication. The modifiedMPR-Behavior bit is only available in after capability exchange and cannot determine the NS values indicated by the network in system information.

<Way forward>: Clarify the use of the modifiedMPR-Behavior bit to support network signalling flags of later releases:

- Upon initial connection the UE can autonomously connect to any network with known flags.

- The modifiedMPR-Behavior bit is signalled to the network. The UE indicates that it supports flags from later releases of a certain unlicensed band (e.g. Rel-17 UE indicates support of Rel-18 flags)

- The network can use this information to configure and schedule transmission on this unlicensed band where support of Rel-18 flag is required.