3GPP TSG-RAN WG4 Meeting #106bis													R4-2306561

Electronic meeting, 17 – 26th April 2023

Agenda item:			4.32

Source:	Apple Inc.

Title:	WF on NTN L-/S-band

Document for:	Information

# Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments that use mixed L-/S-band pairing, whereupon the UL part is one the L-band, while the DL part is one the S-band. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. So, during the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band where the UL is on the L-band and the DL is on the S-band.

This document presents a way forward and agreements made during the RAN#106bis meeting.

# Topic #1: System parameters

Agreements for the system parameters:

-	5MHz, 10MHz and [15MHz] channel bandwidths are specified for the band

NOTE: The 15MHz channel bandwidth may need further technical discussions.

-	The following channel raster points are specified for the band, UL channel raster points at 322000 – 325300 and the DL channel raster points at 496700 –500000.

-	"Case A" sync pattern is specified for the band with the sync raster points at 6215 – 6244 in steps of <1>.

# Topic #2: UE RF requirements

Agreements for the UE RF requirements:

-	Existing MPR (same as for other NTN bands) is re-used for this band.

-	UE co-existence requirements:

-	Use L-band UE co-existence requirements as a baseline considering additional bands as follows:

-	NR band n54, n77, n78, n105; E-UTRA band 31, 54, 72, 73, 87, 88, 103;

-	Other bands can be considered during next meetings.

-	Reference sensitivity:

-	n53 REFSENS + "insertion loss" dB; where potential options for the insertion loss are as follows

-	Option 1: 0dB (resulting in same REFSENS as band n53);

-	Option 2: 0.5dB (resulting in same REFSENS as band n255);

-	Other options are not excluded.

# Topic #3: RRM requirements

No new agreements.

# Topic #4: SAN

Agreements from the UE RF system parameters are applied to SAN:

-	5MHz, 10MHz and [15MHz] channel bandwidths are specified for the band.

-	The following channel raster points are specified for the band, UL channel raster points at 322000 – 325300 and the DL channel raster points at 496700 –500000.

-	"Case A" sync pattern is specified for the band with the sync raster points at 6215 – 6244 in steps of <1>.