3GPP TSG-RAN WG4 Meeting # 106-bis-e												R4-2306569

Electronic Meeting, 17 April –26 April, 2023

Agenda item:			6.4.5

Source:	Anterix

Title:	WF on UE RF requirements of US 900 MHz band

Document for:	Approval

# 1 			Introduction

This contribution is a summary and WF for UE RF issues for the US 900 MHz band discussed as part of agenda item 6.4.5.   UE coexistence simulation and testing results will be presented at the next meeting . Earlier agreements are used as a baseline.

# 2			UE RF Requirements

2.1		Way Forward: Channel Numbering

All EARFCN channel numbers will be added to the specification for NB-IoT, 1.4 MHz and 3MHz channel bandwidths.  A note is also to be added to indicate a EARFCN restriction, equivalent to 899 MHz UL and 938 MHz DL for 3 MHz channel bandwidth.

2.2		Way Forward: UE Coexistence

Proposal for an emission limit of -30 dBm/MHz for UE coexistence to protect the downlinks of Band 5 and Band 26 when the 3 MHz LTE channel is centered at 899 MHz was offered.  A-MPR is zero in this proposal.

There is agreement to provide additional simulation and test results at the next RAN4 #107 meeting before a decision is made on way forward.