3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2306583

Online, April 17th – 26th, 2023

Title: 	WF on MSD simplification due to harmonic and harmonic mixing for EN-DC

Agenda Item:	5.1.5

Source: 	Huawei, HiSilicon, […]

Document for:	Approval

1 Background

In the clause 7.3.1 of TR 38.846 [1], we have the following agreement on the REFSENS exception due to harmonic/harmonic mixing interference specified for ENDC band combinations.

For reference sensitivity exception due to harmonic/harmonic mixing specified for ENDC/NEDC band combinations, it’s suggested to follow the same principles as for NR CA BCS4 WI in WF R4-2210565 as a starting point.

In this meeting, one TP [2] was provided to align the MSD test configurations due to harmonic/harmonic mixing interference with NR CA specified in TS 38.101-1.

In this paper, we’d like to clarify the criteria on how to reduce MSD test points due to harmonic interference.

2 Way forward

<Way forward 1>: In order to implement MSD test reduction, P4, P5, P6and P7 proposed in WF R4-2210565 [3] are also applicable to ENDC band combinations.

<Way forward 2>: To adopt New MSD table format for MSD due to harmonic/ harmonic mixing interference for ENDC band combinations below as an example.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| 1 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| 1 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |

<Way forward 3>: For the band combinations with same frequency range between NR CA and ENDC (e,g, CA_n1-n77 / DC_1_n77), the same test point can be considered as many as possible.

Reference

[1] TR 38.846, Study on simplification of band combination specification for NR and LTE, v0.3.0

[2] R4-2305381	TP for TR 38.846 to align the MSD test configurations due to harmonic and harmonic mixing for ENDC, Huawei, HiSilicon

[3] R4-2210565	WF on criteria on Rel-17 enhanced MSD table format, Skyworks Solutions, Inc.