3GPP TSG-RAN WG4 Meeting # 106-bis-e	R4-2306606

Electronic Meeting, 17 April –26 April, 2023

Title: 	WF on NR FR2 HST UE RF

Agenda Item:	5.13.6

Source: 	Samsung

Document for:	Approval

Topic #1: RF requirement for simultaneous multi-panel operation

Sub-topic #1 bi-directional deployment scenario

#### Issue 1-1: 2AoA spherical coverage area and DL power requirement

- Proposals

  - Proposal 1: Extend the one-directional spherical coverage to bi-directional spherical coverage. We can consider using the same one-directional spherical coverage in both forward and backward directions for bi-directional scenarios. (Nokia 4480)

  - Proposal 2: Regarding M% coverage, FR2 PC6 devices are specific and the 2AoA coverage area should be exactly the same as that of legacy 1AoA area, i.e., Area-1 and Area-2. (Samsung 4831)

  - Proposal 3: the starting point of YdBm value should also be the legacy 1AoA spherical coverage spec in dBm of PC6. (Samsung 4831)

- Agreement

  - For 2AoA spherical coverage area for bi-directional scenario, take legacy 1AoA spherical coverage area as the starting point, i.e., Area-1 and Area-2.

  - The concerned two AoA directions should be selected from different coverage areas, i.e., Area-1 and Area-2 respectively

  - Further discuss the DL power level requirements

    - “legacy requirement + XdB, X>=0” as the starting point for DL power level

#### Issue 1-2: interference mode assumption

- Proposals

  - Proposal 1: RAN4 to decide the interference mode assumption between mode 1 and mode 2. (Huawei 4631)

    - Interference mode 1: only activate two RRHs transmitting two layers at the same time

    - Interference mode 2: all the RRHs (>2 RRHs) transmit two layers at the same time

- Agreement

  - Interference mode 1, i.e., at most two RRHs are transmitting at the same time on different layers.

#### Issue 1-3: void (merged to Issue 1-1)

#### Issue 1-4: angular separation

- Proposals

  - Proposal 1: it is proposed to adopt single angular separation value, i.e. 150° in theta of UE coordination. (Samsung 4831)

- Agreement

  - Cover all possible angular separations in core requirements as long as the two test points are from Area-1 and Area-2 respectively

  - FFS if 150° angular separation in theta of UE coordination can be considered for test verification.

#### Issue 1-5: UE orientation

- Proposals

  - Proposal 1: spherical coverage of HST devices is specified with UE coordination and no need to consider different UE orientations. (Samsung 4831)

- Agreement

  - There is no need to consider different UE orientations when specifying requirements

#### Issue 1-6: AoA+ offset and AoA- offset

- Proposals

  - Proposal 1: it is proposed to only consider single AoA offset, i.e., not to consider both AoA+ offset and AoA- offset. (Samsung 4831)

- Agreement

  - There is no need to consider AoA+ offset and AoA– offset when specifying requirements

#### Issue 1-7: fall back requirement (1AoA requirement for UE supporting 2AoA)

- Proposals

  - Proposal 1: If single AoA is transmitted to the UE, Rel-17 requirement could be reused for UEs supporting 2-panel reception. (Huawei 4631)

- Agreement

  - Focus on 2AoA requirements firstly

Sub-topic #2 uni-directional deployment scenario

#### Issue 2-1: feasibility of uni-directional scenario

- Observations and Proposals

  - Nokia paper R4-2304480

    - Observation 2: in order to deploy the uni-directional RRH pair, two RRHs need to be installed along the railway. The cost of such deployment may be doubled for the operator.

    - Observation 3: Based on the current studies of uni-directional scenarios in [R4-2300998], the performance is not outstanding compared with the performance of bi-directional scenarios.

    - Proposal 2: UE vendor and operator’s inputs are needed to continue the study of the uni-directional deployment.

  - Samsung paper R4-2304831

    - Observation 2: the uni-directional simultaneous multi-panel operation requires ‘RRH paired’ deployment with 150m distance to the track on both sides.

    - Observation 3: the uni-directional simultaneous multi-panel operation requires upgrading the UE panel assumption from 2 panels to 4 panels.

    - Observation 4: new UE assumption with 4 panels still have worse performance under uni-directional scenario than that of bidirectional scenario.

    - Observation 5: coverage hole is expected within legacy coverage Area-1 and Area-2 for uni-directional scenario.

    - Observation 6: Testability issue also needs to be considered if requirements for uni-directional deployment scenario would be needed.

- Agreement

  - uni-directional simultaneous multi-panel operation is not pursued in Rel-18

  - Study on uni-directional simultaneous multi-panel operation is not precluded in future releases, including both scenario A and scenario B.