3GPP TSG-RAN WG1 Meeting #106bis-e	R4-2306608

Electronic meeting, 17th – 26th April 2023

Agenda Item:	5.14.1.2

Source:	Ericsson

Title:	TP to TR 38.876: Update of assumptions for scenarios 2, 3, 10, 11

Document for:	Approval

# 1	Introduction

The text proposal adds that some additional simulations were performed at 300km distance and some minor corrections.

# 3	Text Proposal

#### 6.2.1.2 TN Network Layout

A rural hexagonal grid layout is assumed for TN network clusters with the parameters of table 6.2.1.1.

Table 6.2.1.2-1: Simulation scenarios for ATG coexistence study

| Parameters |  | Values |
| --- | --- | --- |
| Network layout |  | For scenario 11: hexagonal grid, 37 macro sites, 3 sectors per site with wrap around For other scenarios: hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance |  | 7.5 km (4GHz) 3.5 km (2GHz) |
| BS antenna height |  | 30 m |
| UE location | Outdoor/indoor | Outdoor only |
|  | Indoor UE ratio | 0% |
|  | LOS/NLOS | LOS and NLOS, see subclause 6.2.5 |
|  | UE antenna height | 1.5m |
| UE distribution (horizontal) |  | Uniform |
| Minimum BS - UE distance (2D) |  | 35 m |

#### 6.2.1.3 ATG Network Layout

A single ATG BS with a single cell is assumed for the ATG network. The aircraft are assumed to fly in a straight line from the minimum distance to the maximum horizontal distance from the ATG BS in the horizontal boresight of the ATG sector. The minimum and maximum horizontal distance of the ATG UE from the ATG BS are as depicted in table 6.2.1.3-1 and depend on the assumption of sub-arrays or not for the antenna model

In the vertical domain, ATG UEs are distributed in height between 3000m and 10000m

Table 6.2.1.2-1: Simulation scenarios for ATG coexistence study

| Parameters | No sub-arrays | Sub-arrays |
| --- | --- | --- |
| Minimum ATG BS-UE horizontal distance | 20km | 50km |
| Maximum ATG BS-UE horizontal distance | 100km | 100km |

Figure 6.2.1.3-1 ATG BS layout (In horizontal boresight direction of ATG antenna)

For Scenarios 2, 3, 10, 11, in addition to simulations in the above assumptions some extra simulations were carried out in which the ATG UE is assumed to be around 300km from the ATG BS. The TN network is also located at 300km, and the ATG UE is assumed to be positioned within a straight line from the ATG BS and within the TN ground network area.

### 6.2.2 System parameters

#### 6.2.2.1 ATG parameters

The system parameters for ATG BS for co-existence study are assumed as below.

Table 6.2.2.1-1: system parameters for ATG BS

| ATG BS altitude | 30m |
| --- | --- |
| Carrier frequency | 2GHz, 4GHz |
| Frequency reuse factor | 1 |
| Duplex mode | FDD@2GHz, TDD@4GHz |
| Channel bandwidth | 20MHz@2GHz, 100MHz@4GHz |
| Subcarrier spacing (SCS) | 15k@2GHz, 30k@4GHz |
| Number of cells | one cell |
| UE distribution | Single ATG UE per ATG cell Horizontal: Random between minimum and maximum distance in the straight line within range described in section 6.2.1.3 Vertical: Based on ATG UE uniform distribution among 3 to 10km |
| Indoor UE percentage | 0% |
| Number of DL active UEs per cell (NOTE 2) | one UE |
| Number of UL active UEs per cell (NOTE 2) | one UE |
| DL scheduled bandwidth per UE | Full bandwidth |
| UL scheduled bandwidth per UE | Full bandwidth |
| UL target SNR (NOTE 3) | 15dB |
| Traffic model | Full buffer |
| ATG BS maximum output power | 46dBm sum of two polarization for 2GHz 53dBm sum of two polarization for 4GHz |
| ATG BS noise figure | 5dB |
| Handover margin | Not needed |
| NOTE 1: ATG BS is assumed to serve UEs in the rural environment. NOTE 2: Same as the number of BS beam(s). NOTE 3: Target SNR for simulation is based on CL values and only compensates pathloss in the simulation assumptions. |  |

#### 6.2.2.2 ATG UE parameters

The system parameters for ATG UE are assumed as below.

Table 6.2.2.2-1: system parameters for ATG UE

| ATG UE altitude | Vertical: Distributed between 3km and 10km |
| --- | --- |
| Carrier frequency | 2GHz, 4GHz |
| ATG UE max TX power in dBm | 40dBm TRP for 2GHz 43dBm EIRP for 4GHz |
| ATG UE min TX power in dBm | - [-33dBm] for 100MHz - [-40dBm] for 20MHz |
| ATG UE noise figure | 9dB |

#### 6.2.2.3 TN BS and UE parameters

The system parameters for TN BS and TN UE are assumed as below.

Table 6.2.2.3-1: system parameters for TN BS and UE

| Parameters | Rural | Rural |
| --- | --- | --- |
| Carrier frequency | 2GHz | 4GHz |
| Channel bandwidth | 20MHz | 100MHz |
| Scheduled channel bandwidth per UE (DL) | 1 | 1 |
| Scheduled channel bandwidth per UE (UL) | 1 | 1 |
| The number of active UE (DL) (NOTE 1) | 1 | 1 |
| The number of active UE (UL) (NOTE 1) | 1 | 1 |
| Traffic model | full buffer | full buffer |
| DL power control | No | No |
| UL power control | Yes | Yes |
| UL target SNR (NOTE 3) | 15dB | 15dB |
| TN BS-UE min distance in meters | 35m | 35m |
| TN BS max TX power in dBm (NOTE 2) | 46dBm | 53dBm |
| TN UE max TX power in dBm | 23dBm | 23dBm |
| TN UE min TX power in dBm | -40dBm | -40dBm |
| TN BS Noise figure in dB | 5dB | 5dB |
| TN UE Noise figure in dB | 9dB | 9dB |
| Handover margin | 3dB | 3dB |
| NOTE 1: Same as the number of BS beam(s). NOTE 2: TN BS max TX power is defined as the sum over both polarizations. NOTE 3: Target SNR for simulation is based on CL values and only compensates pathloss in the simulation assumptions. |  |  |