3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2306609

Online, April 17th – 26th, 2023

Title: 	WF on ATG UE RF requirements

Agenda Item:	5.14.6

Source: 	Huawei, HiSilicon

Document for:	Approval

1 Background

Referring to the latest work item [1], the main objectives for ATG UE is listed as below.

    - Identify key characteristics where it is necessary to differentiate ATG ground-based BS and UEs from conventional ground based BS and UEs

      - Aim to reuse existing requirements for BS and UE where possible, e.g.,

        - Reuse TN BS requirements for ATG BS

    - Specify RF requirements for ATG UE/BS

      - Considering the results of co-existence simulations in terms of impact on emissions and RX requirements, cell sizes and link budgets, technology capabilities, likely BS and UE architectures and other relevant aspects.

      - Taking into account identified differences between ATG and fully ground based systems

      - Consider BS type 1-C/1-H/1-O and specify the requirements

      - Consider conductive requirements for UE

In this meeting, companies provided some contributions [2~11] to discuss how to specify the remaining RF requirements for ATG UE. In this meeting, we’d like to capture some progress in this paper.

2 Tx requirements

Sub-topic 1-1 Operating band

<Way forward>: TDD band n39 can be added as the operating band of ATG

Sub-topic 1-2 The maximum output power

Agreement:

- New capability can be introduced for ATG UE to indicate [the minimum of maximum output power] at maximum modulation order and full PRB configurations.

  - The capability report granularity is 1dB.

Agreement:

- Upper limit for new capability for maximum conductive output power or TRP

  - 40dBm

Agreement:

- Introduce two UE types to distinguish the antenna types for ATG UE

- <Way forward>:FFS on accuracy/ tolerance.

- <Way forward>:FFS whether and/or how to improve the term of [the minimum of maximum output power].

Sub-topic 1-3 Minimum output power for ATG

Candidate options:

  - Option 2: Considering some margins, the minimum output power for ATG could be written as below with 25Db relaxation:

| Channel bandwidth | (MHz) | 5,10,15,20 | 25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 |  | 30 |
| Minimum output power | (dBm) | -40+25 | -40+25+10log10 (BWChannel /20) | -40+25+10log10 (BWChannel /20) |
| Measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |  |
| NOTE: The minimum output power value is rounded to the nearest number down to one decimal point. |  |  |  |  |

  - Option 3: Define UE minimum output power to be -25 dBm for 2 GHz, and -20 dBm for 4 GHz.

  - Option 5: the minimum output power for ATG UE could be [-3dBm/100MHz] and scale with other BW.

<Way forward>: FFS the assumption of minimum distance between ATG BS and ATG UE to derive the minimum output power.

Sub-topic 1-4 UL 256QAM for ATG UE

<Way forward>: FFS whether 256QAM should be supported for ATG UL and the existing transmit modulation quality requirements can be reused.

Sub-topic 1-5 Configured transmitted power

<Way forward>: Further discuss the following two options with the consideration that the range of this IE P-Max should be extended.

  - Option 1: it’s proposed to define the following Pcmax,f,c

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c, PMaxOutputPowerClass}

PCMAX_H,f,c = PEMAX,c

where

PEMAX,c is the value given by [either the p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE], whichever is applicable according to TS 38.331[7];

PMaxOutputPowerClass is the maximum UE output power at maximum modulation order and full PRB configurations which is indicated by ATG UE;

  - Option 2: for configured output power of ATG CPE, propose not to configured Tx power defined similar as IAB-MT.

Sub-topic 1-6 Additional SEM

<Way forward>: Not to define Additional SEM for ATG CPE unless there are any regulatory requirement

Sub-topic 1-7 UE coexistence requirement for ATG CPE

<Way forward>: FFS whether general spurious emission requirements can meet the UE-to-UE coexistence requirements considering the worst case.

3 Rx requirements

Sub-topic 2-1 Maximum input level

<Way forward>: Further to discuss whether we need to specify two sets of RF requirements for maximum input level.

<Way forward>: FFS the assumption of minimum distance between ATG BS and ATG UE to derive the maximum input level.

<Agreement>: Pinterference for ACS case 2 should align with the maximum input level

Reference

[1] RP-220962	New WID: Air-to-ground network for NR, CMCC

[2] R4-2304206	Discussion on ATG UE Tx requirements, CMCC

[3] R4-2304674	On ATG UE Tx reuqirements, Qualcomm Incorporated

[4] R4-2305005	Discussion on remaining ATG UE RF Tx requirements, Ericsson

[5] R4-2305373	Discussion on ATG UE Tx requirements, Huawei, HiSilicon

[6] R4-2305394	Further discussion on ATG UE Tx RF requirements, ZTE Corporation

[7] R4-2304207	Discussion on ATG UE Rx requirements, CMCC

[8] R4-2304675	On Rx requirements for ATG UE RF, Qualcomm Incorporated

[9] R4-2305006	Discussion on remaining ATG UE RF Rx requirements, Ericsson

[10] R4-2305375	Discussion on ATG UE Rx requirements, Huawei, HiSilicon

[11] R4-2305395	Further discussion on ATG UE Rx RF requirements, ZTE Corporation