3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2306610

# e-meeting, 17th – 26th April, 2023

Source: 	Apple

Title: 	TP for TR 38.876: General aspects

Agenda Item:	5.14.2.1

Document for:	Approval

# Introduction

This contribution presents a text proposal on general aspects for ATG for TR 38.876.

# Reference

[1] R4-2303640, TR38.876

# Text proposal

----------------------------------------------------------Start of Text Proposal------------------------------------------------------------

# 1	Scope

The present document covers the RF and co-existence aspects of the work item “Air-to-ground network for NR” [2]

The objectives for the study are the following:

-	Study and evaluate adjacent channel co-existence for ATG scenarios.

-	Study and specify RF core requirements for ATG network and the ATG UE such that ATG deployment are well supported.

-	Study and specify RRM requirement supporting ATG network deployment and ATG UE mobility

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-230279, Air-to-ground network for NR, CMCC

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ACRONYM>	<Explanation>

# 4	Background

Air-to-ground (ATG) network refers to in-flight connectivity technique, using ground-based cell towers that send signals up to an aircraft’s antenna(s) of onboard ATG terminal. As a plane travels into different sections of airspace, the onboard ATG terminal automatically connects to the cell with strongest received signal power, just as a mobile phone does on the ground. In this network, a direct radio link will be established between BS on the ground and CPE type of UE mounted in the aircraft.

From the trials and commercial operation [https://inflight.telekom.net/ean/] of adapted LTE ATG solutions, some characteristics are considered for ATG network deployment scenarios.

- Extremely large inter-site distance (ISD) and large coverage range: In order to control the network deployment cost and considering the limited number of flights, large ISD is preferred, e.g., about 100km to 200km. At the same time, when the plane is above the sea, the distance between the plane and the nearest base station could be more than 200km and even up to 300km. Therefore, ATG network should be able to provide up to 300km cell coverage range.

- Utilizing non-disjoint frequency for deploying both ATG and terrestrial networks, i.e. same operating band but ATG network and TN use adjacent carriers: Operators are interested to adopt the same frequency for deploying both ATG and terrestrial networks to save frequency resource cost, while interference between ATG and terrestrial networks becomes non-negligible and should be addressed.

- Much powerful on-board ATG terminal capacity: On-board ATG terminal can be much powerful than normal terrestrial UE, e.g., with higher EIRP via much larger transmission power and/or much larger on-board antenna gain.

# 5	ATG bands

ATG will operate within existing NR operating bands and does not need new bands and band properties to be identified. Depending on the operator’s request, the following NR bands are intended for ATG deployment:

Table 5-1: ATG operating bands

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |

---------------------------------------------------------End of Text Proposal--------------------------------------------------------------