3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2306611

# e-meeting, 17th – 26th April, 2023

Source: 	Apple

Title: 	TP for TR 38.876: Transmit signal quality

Agenda Item:	5.14.2.2

Document for:	Approval

# Introduction

This contribution presents a text proposal on transmit signal quality for ATG for TR 38.876.

# Reference

[1] R4-2303640, TR38.876

# Text proposal

----------------------------------------------------------Start of Text Proposal------------------------------------------------------------

#### 7.1.2.9 Transmit signal quality

Transmit signal quality include frequency error and transmit modulation quality.

For frequency error, frequency pre-compensation is assumed for ATG UE. The existing requirement defined for NTN UE in TS 38.101-5 will be reused as baseline.

For transmit modulation quality, the following requirement for FR1 UE in 38.101-1 will be reused for ATG UE.

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| [256 QAM] | % | 3.5 |

#### 7.1.2.10 Occupied bandwidth

The Occupied bandwidth requirement in subclause 6.5.1 of 38.101-1 [xx] is applicable for ATG UE operating in FR1.

#### 7.1.2.11 SEM requirements

#### 7.1.2.12 ACLR requirements

#### 7.1.2.13 Spurious emission

The general spurious emission requirement in subclause 6.5.3.1 of 38.101-1 [xx] is applicable for ATG UE operating in FR1.

#### 7.1.2.14 Spurious emissions for UE co-existence

#### 7.1.2.15 Transmit intermodulation

---------------------------------------------------------End of Text Proposal--------------------------------------------------------------