3GPP TSG-RAN WG4 Meeting # 106bis-e                           R4-2306614

Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.876 to introduce ATG UE Tx requirements

Agenda Item:	5.14.2.2

Document for:	Approval

Introduction

In this paper, we provide the following text proposals on MOP/MPR/AMPR requirements referring to the WF [1].

- References

[1] R4-2303645, WF on ATG UE RF requirements, Huawei, HiSilicon

- Text proposal

- <<Start of Change for TR 38.876>>

- 7.1.2.2 MOP requirements

For ATG UE, the maximum output power is declared at maximum modulation order and full PRB configurations within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

ATG UE implementations/antenna patterns might be varying for different operating bands. The output power from the UE may depend on the aircraft type and exact deployment scenario. Currently, there is a lack of knowledge in 3GPP of the appropriate output power for an ATG UE considering all design,regulatory constraints and potential avionics device protection. For this reason, it is agreed that ATG UE is allowed to declare the output power and signal it to the network. The lower limit of conductive MOP or TRP declared by ATG UE is 23dBm.

- 7.1.2.3 MPR/AMPR requirements

ATG UE is allowed to declare the output power at supported maximum modulation order and full PRB configurations. Thus, MPR/AMPR requirements are not needed as ATG UE can declare a lower power whenever power back-off is needed. Additionally, when ATG UE indicate the maximum output power under the specific conditions (the supported maximum modulation order, full PRB configurations, SEM/spurious emission requirements, regulation requirements and so on), the necessary power back-off should be considered. In total, the power back-off has been included in the declared maximum output power, so there is no need to specify MPR/AMPR requirements for ATG UE.

- <<End of Change>>