3GPP TSG-RAN WG4 Meeting #106-bis-e										R4-2306617

Electronic Meeting, 17 April –26 April, 2023

Title: 	WF on LP-WUS

Agenda Item:	5.21.4

Source: 	vivo

Document for:	Approval

Topic #2: LP-WUR architectures

- Sub-topic 2-1 General for WUR architecture

Issue 2-1-1: Frequency range

Agreements:

  - RAN4 focus on FR1 frequency ranges first priority, 2.6GHz can be selected as an example band.

Issue 2-1-2: UE type

Agreements:

  - Based on RAN1 agreements, RAN4 should consider all the UE types mentioned in the SID, e.g. IoT devices, Wearable devices, and e-MBB devices. The cost aspect can also be considered for the design of LP-WUS/WUR scheme.

Issue 2-1-4: general views for WUR architectures

Agreements:

  - Further discuss WUR need to be capable of configuring the same raster point with main receiver.

  - WUS repetition is signal design which is RAN1 task, no RAN4 discussion is needed.

  - Architecture in P3 could belong to the variant of general architectures mentioned in RAN1 LS.

- Sub-topic 2-2 Gap RB definition for LP-WUS

Issue 2-2-1: Guard RBs definition for LP-WUS

Agreements:

- RAN4 use guard RBs (if needed) for LP-WUS, which is Granularity of RB. The traditional guardband for NR channel bandwidth defined in TS 38.101-1 should not be changed.

  - For case when WUS is smaller than NR channel bandwidth

    - For case 2-1, the LP-WUS guard RB is number RBs between LP-WUS and NR signals (edge of WUR RB location to nearest edge of eMBB RB)

    - For case 2-2, the WUS is placed at the edge of the NR channel bandwidth, i.e. the lowest/highest RB of WUS with guard RBs is aligned with the lowest/highest NR transmission bandwidth configuration in spec TS 38.101-1.

  - [For case when the WUS/WUR is same as NR channel bandwidth]

    - For case 1, the LP-WUS guard RBs is number RBs between LP-WUS and traditional guardband (edge of WUR RB location to Outermost of NRB)

    - RAN4 should further check with RAN1 for this case

- FFS whether the guard RBs should be symmetric within the WUS channel bandwidth.

Issue 2-2-2: Whether guard RBs is needed for LP-WUR

Agreement:

- How many RBs (if needed) for guard is FFS. RAN4 should further evaluate this number based on the cases identified in issue 2-2-1.

- The size of guard RBs from implementation perspective for LP-WUS should be determined in RAN4.

- Sub-topic 2-3 UE Adjacent Carrier/Sub-Carrier Selectivity (ACS/ASCS) evaluation

Issue 2-3-1: General evaluation framework for both ACS and ASCS

Agreement:

- The following aspects can be starting point for further discussions

- Framework in RAN4 that the ACS and ASCS value can be evaluated based on the following aspects:

  - Typical filter characteristic, e.g. filter order, pass BW, cut-off frequency

  - Guard RB size within LP-WUS channel bandwidth

  - RF impairment can also be considered

- Averaged power attenuation at ACS or ASCS frequency range

- FFS whether SINR of the wanted signal at detector input is needed

- FFS whether use ICS to instead ASCS

- FFS Coexistence-simulation-based framework can also be considered

  - FFS on details of coexistence study (if needed) of LP-WUS

  - Coverage should be considered

Issue 2-3-2: LP-WUS evaluation scenarios for study purpose

Agreements:

- Consider a limited set of WUS scenarios in table below for study purpose in RAN4

Table 1: LP-WUS evaluation scenarios

| NR RF channel BW | 5MHz for 1.4MHz WUS; 20MHz/100MHz for 5MHz WUS |
| --- | --- |
| Guardband of NR channel | Unchanged, defined in Clause 5.3.3 in TS 38.101-1 |
| WUS BW within NR channel | 1.44MHz, 5.04 MHz |
| WUS RB allocation (Note 1) | [6] RB in 1.44 MHz, total 8 RBs, or other number of RBs [24] RB in 5.04 MHz, total 28 RBs, or other number of RBs |
| WUS placement within NR channel | 3 cases: - case 1: Center; - case 2: edge; - case 3: between center and edge of NR channel |
| Guard RB size of LP-WUS | - 0 RB, 1RB at each side, 2RBs at each side, or other number of RBs. - Asymmetric guard RB can also be considered |
| ACS interferer | According to RF CBW |
| Filter characteristic | 2nd to 5th order Butterworth Both analog and digital filter can be considered |
| Filter passband BW | At least WUS bandwidth (number of RBs), depends on guard RB size |
| LO frequency | Case 1: In the middle of WUS (modeling fixed WUS position) Case 2: In the middle of RF channel (modeling flexible WUS location) |
| Target ACS | TBD |
| Target ASCS | TBD |
| Target WUS SNR | TBD |
| RF impairment | FFS |
| Note 1: the maximum number of allocated WUS RBs, depends on how many Guard RBs are needed. 5MHz WUS within 5MHz NR CBW is not considered currently. |  |

Issue 2-3-3: How to determine guard RBs for LP-WUS

Agreements:

- RAN4 can perform more analysis based on the framework in issue 2-3-1 and selected scenarios in 2-3-2, and further discuss how to determine guard RBs next meeting.

Issue 2-3-4: Whether WUS can be flexibly located within the NR carrier

Agreements:

- FFS whether LP-WUS can be flexible or partially flexible located within NR carrier.

  - pros and cons of flexible WUS location can be studied

Issue 2-3-6: RF impairment impacts

Agreements:

- ACS, ASCS and guard RBs study can consider the receiver RF impairments and the required wake-up signal SNR.

- FFS whether RAN4 should agree on a phase noise profile for wake-up receiver study

- FFS on the CFO assumed in simulation

- FFS on other receiver RF impairment modeling.

- Sub-topic 2-5 BS RF impacts

Issue 2-5-1: Whether and which power boosting level RAN4 should study LP-WUS power boosting

Agreement:

- RAN4 should study the power boosting if triggered by RAN1, to check whether the values are feasible from RAN4 perspective.

Issue 2-5-3: other gNB impacts

Agreements:

- RAN4 recommends RAN1 to prioritize signal design which allow re-use of current gNB HW.

- Sub-topic 2-6 Separate band for LP-WUS

Issue 2-6-1: Whether a dedicated band for WUS is needed

Agreement:

- Wait for RAN1 response for further discussion and decision.

- Sub-topic 2-7 SNR evaluation

Issue 2-7-1: SNR evaluation activity in RAN4

Agreements:

- RAN1 is performing SNR evaluation, the WUS SNR analysis can be done in RAN1. RF impairment aspects, e.g. frequency error and ADC sampling accuracy, if identified and confirmed, can be sent to RAN1 for consideration

- Sub-topic 2-8 Architecture down-selection

Issue 2-8-1: Whether make down-selection is needed in RAN4

Agreement:

- RAN4 further evaluate the pros and cons of each architecture based on agreed framework and selected scenario. Make decision on architecture down-selection next meeting and send decisions to RAN1.

Topic #3: LP-WUS designs

Issue 3-4: other receiver architectures for WUR

Agreement:

- RAN4 focus on the discussions on LS related issues.

- Reference

[1] R4-2306298, Topic summary for [106-bis-e][136] FS_NR_LPWUS, Moderator (vivo), RAN4#106bis-e