3GPP TSG-RAN WG4 Meeting #106Bis-e	R4-2306620

Electronic Meeting, April 17 ‒ 26, 2023

Title: 	WF on UE RF aspects for Rel-18 NR positioning

Agenda Item:	5.23.4

Source: 	Intel Corporation

Document for:	Approval

# 1	General aspects and work plan

## Sub-topic 1-1: PRS/SRS bandwidth aggregation scope

Agreement:

-	No dedicated or separate RF impairment modelling discussion is needed in WI phase for the solutions based on PRS/SRS bandwidth aggregation unless it’s triggered by RAN1 LS.

## Sub-topic 1-2: Carrier phase positioning scope

Agreement:

-	No dedicated or separate RF impairment modelling discussion is needed in WI phase for the solutions based on carrier phase positioning unless it’s triggered by RAN1 LS.

## Sub-topic 1-3: General RF scope - BS

Agreement:

-	No impact to BS RF requirements is foreseen due to the work on expanded and improved NR positioning. The scope of requirements will be limited to RRM.

# 2	RF requirements

## Sub-topic 2-1: PRS/SRS bandwidth aggregation

Agreement:

-	No new UE RF requirement is needed for the solutions based on PRS / SRS bandwidth aggregation

-	No new BS RF requirement is needed for the solutions based on PRS / SRS bandwidth aggregation

## Sub-topic 2-2: Carrier phase positioning

Agreement:

-	No new UE RF requirement is needed for the solutions based on carrier phase positioning

-	No new BS RF requirement is needed for the solutions based on carrier phase positioning

## Sub-topic 2-3: RedCap UE positioning

### Issue 2-3a: UL SRS Tx frequency hopping

Agreement:

- For UL SRS Tx frequency hopping, use the switching time of {70us, 140us} for FR1 as the starting point

- SRS Tx frequency hopping range can be up to 100MHz

- Which specific value for frequency hopping is applied depends on UE capability, if multiple values are agreed

Agreement:

- For UL SRS Tx frequency hopping, use the switching time of {35us, 70us, 140us} for FR2 as the starting point

- SRS Tx frequency hopping range can be up to 400MHz

- Which specific value for frequency hopping is applied depends on UE capability, if multiple values are agreed

### Issue 2-3b: DL PRS Rx frequency hopping

Agreement:

- For DL PRS Rx frequency hopping, use the switching time of {70us, 140us} for FR1 as the starting point

- PRS Rx frequency hopping range can be up to 100MHz

- Which specific value for frequency hopping is applied depends on UE capability, if multiple values are agreed

Agreement:

- For DL PRS Rx frequency hopping, use the switching time of {35us, 70us, 140us} for FR2 as the starting point

- PRS Rx frequency hopping range can be up to 400MHz

- Which specific value for frequency hopping is applied depends on UE capability, if multiple values are agreed

### Issue 2-3c: LS reply

Agreement:

-	Strive to send LS in this meeting

# 3	References

- R4-2306300, “Topic summary for [106-bis-e][138] NR_pos_enh2_UERF,” Moderator (Intel Corporation), RAN4 #106Bis-e, April 2023