3GPP TSG-RAN WG4 Meeting#106bis                               R4-2306624

E-meeting, April 17 – April 26,2023

Title: 	WF on NTN UE RF requirements in Ka-band

Agenda Item:	     5.27.6

Source: 	ZTE

Document for:	Approval

Agreement

Issue 1-1: UE types

- Encourage companies to provide the regulation differences leading to different RF/RRM requirements between the mobile and fixed NTN UEs, and further discuss whether RAN4 needs to define the different UE types.

- Discuss possible differentiation of NTN UE types in above 10 GHz, based on RF parameters (noise figure, antenna types, antenna gain and transmission power, or alternatively minimum EIRP in the peak direction, Effective Isotropic Sensitivity EIS, etc.).

  - The impact on RF and RRM requirements will be investigated, and analysis on RF requirement impact is prioritized.

Issue 2-1:  Beam tracking capability

Tentative agreement:

the (Rx and Tx) beam pointing error are compliant with at least the relevant ETSI harmonized standard. (e.g.

      - EN 303 978    “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands”;

      - EN 303 979    “Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz frequency bands”).