3GPP TSG-RAN WG4 Meeting # 106-bis-e										R4-2306627

Electronic Meeting, 17 April –26 April, 2023

Title: 	WF on remaining open issues for MPR/PAR reduction for coverage enhancement

Agenda Item:	5.28.3

Source: 	Nokia

Document for:	Approval

# Topic #1: General

## 1-0: Solutions to consider for MPR/PAR reduction

< Way forward >:

- RAN4 agrees to analyse provided simulation results as attached to this WF via Excel and provide conclusions based on these at RAN4#107.

  - It is expected a decision on selecting the Rel-18 MPR/PAR reduction solution(s) according to the WID can be taken at RAN4#107

## 1-3: Capturing of Simulation Results

<Agreement >:

- RAN4 agrees that companies shall fill both the ‘OBO’ and ‘10% BLER SINR’ columns in the RAN4 simulation templates for MPR/PAR result collection.

## 1-4: Net gain calculations

<Agreement >:

- Use the following equation as the baseline

Net Gain dB=PoutFilter i-PoutRef+SNR10%Ref-SNR10%Filter i

- Where PoutFilter i is the achieved (TX) output power of the filtered  waveform being compared against the reference, PoutRef is the output power of the reference, SNR10%Ref is the required SNR to achieve 10% BLER with the reference, and SNR10%Filter i is the required SNR to achieve 10% BLER using the filtered waveform being compared against the reference.

- Use the following equation when providing the results for net gain

Net Gain dB=-DOPR_M0 +SNR10%Ref-SNR10%Filter i

# Topic #2: RF simulation parameters

## 2-1: Spectrum extension factor (α)

Agreement:

- For evaluations, RAN4 should consider the spectrum extension factor (α) as: 0 (reference), 0.25, 0.111 (1/9), 0.125 and 0.375.

## 2-2: BS Rx assumption

<Agreement >:

- When expressing the net gain companies shall indicate the BS receiver type used.

# Topic #5: RF specification impact

## 5-1: ACLR requirement

<Agreement >:

- RAN4 shall further study whether the ACLR of a PC2 UE can be applied to a PC3 UE when the output power is boosted to the same level of a PC2 UE.

Comments: