3GPP TSG-RAN WG4 Meeting # 106-e-bis	R4-2306649

# Electronic Meeting, April 17th – April 26th, 2023

Title:	Reply LS on Rel-16 UL Tx switching period

Response to:	(R1-2302198) LS on Rel-16 UL Tx switching period

Release:	Rel-16

Work Item:	NR_RF_FR1-Core

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name: Sanjun Feng

E-mail Address: fengsanjun@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on Rel-16 UL Tx switching period.

RAN4 would like to clarify that current RAN4’s requirements did not cover the case when the network provided a gap between the end of the switch-from carrier’s UL transmission and the start of the switch-to carrier’s transmission that is sufficiently long to absorb the switching period.

RAN4 has decided to update Rel-16/17 38.101-1 and 38.101-3 requirements to incorporate RAN1’s following listed agreements which are copied from the incoming LS.

- If the gNB provides sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier to absorb the switching period,

  - The time of no UL transmission allocated absorbs the switching period

  - Neither of the uplink transmissions (the one ending on the switch-from carrier nor the one starting on the switch-to carrier) are interrupted by the switching period.

  - The setting of uplinkTxSwitchingPeriodLocation has no impact.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take the above feedback into account.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #107                  22th May – 26th May 2023         Incheon, KR

TSG-RAN WG4 Meeting #108                  21st August – 25th August 2023    Toulouse, FR