3GPP TSG-RAN WG4 Meeting #106-bis-e 								R4-2306650

Electronic Meeting, 17 April - 26 April, 2023

Source:	China Telecom

Title:	WF on RedCap HPUE

Agenda Item:	8.1

Document for:	Approval

- Background

In RAN #99, RAN task RAN4 the following work and report back to RAN#100:

- To study how to enable HPUE including PC2 for RedCap in the band agnostic way

- To study the applicable form factor, e.g., sensor, camera

- Way forward

Issue 1: Form factor for PC2 RedCap HPUE

Candidate options for further discussion:

- Option 1: Not restrict the applicable form factor for PC2 RedCap HPUE in the specification

- Option 2: Clearly add the restriction on the applicable form factor for PC2 Redcap in the specification

Issue 2: How to specify HPUE including PC2 for RedCap

Way forward:

- PC2 RedCap HPUE requirements can be specified in the band agnostic way at least for TDD and HD-FDD.

  - The band agnostic way means the PC2 requirements for non-RedCap UE can be re-used for PC2 RedCap UE in a band agnostic way. Only the operating bands for which PC2 on non-RedCap UE has been introduced can be considered for RedCap UE.

- Further discuss whether the band agnostic way can be applied for PC2 FD-FDD bands with 1Rx and 2Rx, based on more analysis on the RF requirements including A-MPR and RSD.

- Note 1: 1Tx architecture is assumed for RedCap PC2.

- Note 2: PC2 for Rel-17 Redcap is considered under this RAN task.