3GPP TSG RAN WG4 Meeting #106-bis-e		 R4-2306660

# 17 April - 26 April 2023, e-Meeting

Agenda Item:	5.24.2.1

Source: 	MediaTek Inc.

Title: 	WF on Tx switching ambiguity issue

Document for:	Discussion

1. Background

RAN4 Agreements captured in the LS R4-2220548 (RAN4 #105):

For Rel-18 UL Tx switching among 4 bands, when switching from 1T+1T on band A and B to 1T+1T on band C and D is performed, and it is not clear whether UE performs Tx switching {from band A to C + B to D} or {from band A to D + B to C}, RAN4 agreed that:

- As baseline UE assumption, no need to resolve the ambiguity issue of the switching pattern for each Tx chain and determine the switching gap based on the worst case by default, i.e., neither of the two Tx chains is expected to be used for transmission during the maximum of the four switching periods, i.e., max {Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C}.

Note: Tswitch_A-C, Tswitch_B-D, Tswitch_A-D, Tswitch_B-C are the switching periods reported by the UE for band pair A&C, B&D,A&D and B&C, respectively.

Pros: Ease of implementation

Cons: Worst transmission throughput performance

2. Agreement

    - RAN4 maintains the baseline assumption in Issue 1-2-3 agreed in R4-2220546 during RAN4#105.

    - Neither of the two Tx chains is expected to be used for transmission during the switching periods in Rel-18

    - Encourage companies to study the benefit to resolve the switching pattern ambiguity issue when 2 Tx chains are switched between two different band pairs for one Tx switching instance

      - At least for single TAG

      - FFS whether it applies to dual TAG

Reference:

- R4-2303693 WF on Multi-carrier enhancements for NR, RAN4#106

- R4-2218928 Continue discussion on R18 Tx switching – Single TAG, RAN4#105

- R4-2301713 Discussion on multi-carrier enhancement for single-TAG Tx switching, RAN4#106

- R4-2303508 WF on ambiguity issue when two Tx chains are switched between different band pairs, RAN4#106