3GPP TSG-RAN WG4 Meeting #106bis-e 	R4-2304058

Electronic Meeting, April 17 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Rel-18 extension of Unified TCI framework for mTRP operation

Agenda item:	5.30.3.3

Document for:	Discussion

Introduction

In RAN4#106, the following agreements were made regarding the scope of the MIMO_evo_DL_UL work in RAN4 [1]:

RRM impact except timing requirements related to two Tas Agreements - RRM requirements impacts - Objective 1 (CSI reporting enhancements) - No RRM impact from Rel-16/17 Type-II codebook refinement objective - FFS whether “UE reporting of time-domain channel properties” has impact on RRM requirements - Objective 2 (Unified TCI framework) - Further study and if needed specify extension of unified TCI framework RRM requirements to M-TRP. - Objective 3 (DMRS Enhancement) - No RRM requirements impact - Objective 4 (enhancements of CSI acquisition for C-JT) - No RRM requirements impact - Objective 5 - SRI/TPMI enhancements: No RRM requirements impact - SRS enhancements: RRM impacts are FFS - Objective 6 - UL precoding indication: RRM impacts are FFS

Considering the impact identified on the extension to Unified TCI framework, this paper discusses aspects regarding that objective including extension to mTRP,

Discussion

Unified TCI Framework extended to M-TRP

Requirements for sDCI and mDCI

In RAN4#106 there was an issue regarding support of sDCI and mDCI [1]:

Issue 3-1-2: For extension of Rel-17 unified TCI framework, whether to support sDCI and mDCI? Way forward: - FFS: Both sDCI and mDCI based MTRP are considered for extension of Rel-17 unified TCI framework for multi-TRP

In Rel-17 the unified TCI framework does not provide support for mTRP operation for single DCI operation, as there is one indicated codepoint, which can be associated to one TCI state for joint configuration (i.e. one entry per codepoint) or to 2 TCI states for separate configuration (i.e. 2 entries per codepoint). In Rel-18, two codepoints can be indicated in same DCI thus enabling s-DCI m-TRP operation.

Rel-18 specifies the indication of two codepoints in the same DCI, thus enabling s-DCI m-TRP operation.

Both s-DCI and m-DCI operations are addressed with either joint or separate TCI configurations. The use cases for s-DCI are illustrated in Figure 1, where joint UL/ DL is shown in Figure 1 a) and separate UL and DL states are configured in Figure 1 b).

| a) | b) |
| --- | --- |

Figure 1: s-DCI use cases for Rel-18 TCI framework.

Observations for s-DCI operations are:

- s-DCI is defined for same active TCI states or different active TCI states across TRPs in joint and separate UL/DL TCI configuration

  - Only one of the TRPs schedules the data for both TRPs, which requires tight coordination among the TRPs

  - PDCCH/PUCCH are only sent by/to one of the TRPs

  - m-TRP with s-DCI requires extension of the numbers of codepoints for indication of the TCI state, i.e. enabling 2 indicated TCI states from the same list of active TCI states (open in RAN1)

  - m-TRP with s-DCI for 4 layers requires extension of 1 codeword to support 4 layers (open in RAN1 if to support 2 codewords)

The use cases for m-DCI are illustrated in Figure 2, where joint UL/ DL is shown in Figure 2 a) and separate UL and DL states are configured in Figure 2 b).

| a) | b) |
| --- | --- |

Figure 2: m-DCI uses cases for Rel-18 TCI framework

Observations for m-DCI operation are:

- m-DCI is only applicable to different TCI states across TRPs for joint and separate UL/DL TCI configuration

  - Each TRP sends its own DCI

  - m-DCI operation supports non-ideal backhaul deployment

  - m-DCI enables independent scheduling per TRP

  - PHY or MAC is not shared between the TRPs

  - Each TRP has its own list of activated TCI states

  - m-TRP with m-DCI is equivalent to two parallel Rel-17 mechanisms, one per TRP

  - m-TRP with m-DCI can already be supported with 2 codewords, where each codeword supports 2 layers

Both sDCI and mDCI have important applications in practical deployments. Single DCI uses a single codeword and therefore a single MCS is to be used for both TCI states. Therefore, sDCI is usually more prone for application when there is similar SINR achieved from both TRPs. On the other hand, mDCI uses independent codewords for indication of TCI states from 2 TRPs, where each DCI may be used to schedule data using different MCS. In that sense mDCI is more robust to adapt to different deployments, since it allows for larger difference in SINR achieved from both TRPs. Therefore, it is proposed that we confirm the agreement in FFS during the last RAN4 meeting.

m-TRP with s-DCI requires extension of the numbers of codepoints for indication of the TCI state i.e. enabling 2 indicated TCI states from the same list of active TCI states (open in RAN1).

m-TRP with m-DCI is equivalent to parallel Rel-17 mechanism per TRP.

s-DCI may be scheduled for either joint or separate TCI configurations.

m-DCI may be scheduled for either joint or separate TCI configurations.

Confirm agreement: Both sDCI and mDCI based MTRP are considered for extension of Rel-17 unified TCI framework for multi-TRP.

Additionally, the following issues are open from last meeting regarding single and multi DCI [1]:

Issue 3-1-6-a: If multi-Rx is not supported, whether to use common requirements or separate requirements to support sDCI or mDCI? - Proposals - P1: Common requirements for both sDCI and mDCI scenarios - Rel-17 Unified TCI state list update delay can apply for MAC CE based TCI states activation for PDSCH in both sDCI and mDCI scenario if single panel scheme is used. (Intel) - P2: Discuss whether different requirements are needed for s-DCI operation and m-DCI operation in each joint or separate TCI frameworks, as shown in the 4 cases (Nokia) Issue 3-1-6-b: If multi-Rx is supported, whether to use common requirements or separate requirements to support sDCI or mDCI? - Proposals - Option 1: Common requirements for both sDCI and mDCI scenarios - Option 2: Different requirements for sDCI and mDCI scenarios Issue 3-1-7: Whether to use common requirements or separate requirements for joint or separate TCI framework? - Proposals - Option 1: Discuss whether different requirements are needed for s-DCI operation and m-DCI operation in each joint or separate TCI frameworks, as shown in the 4 cases (Nokia)

In the last RAN4 meeting, issue 3.1.6-b included proposals regarding whether sDCI and mDCI should be covered with common or separate requirements. In our view, this discussion has to be analysed case by case, since some of the requirements only make sense either for sDCI or for mDCI. The mechanisms for indicating 2 TCI states are not common for the sDCI and mDCI. As an example, indication of 2 TCI states using DCI is not supported by mDCI operation currently, since in multi DCI scenario the indication of 2 TCI states is done independently for each coresetPoolIndex. Note that the configuration of s-DCI or m-DCI is performed via RRC Configuration.

TCI switching commands for mTRP are different for sDCI and mDCI.

RAN4 to define different requirements for sDCI and mDCI scenarios.

RAN4 to define separate requirements for both single-DCI and multi-DCI scenarios in MIMO_evo Rel-18 including support of both configurations of joint or separate frameworks.

Requirements for intra-cell mTRP and inter-cell mTRP scenarios

In intra and inter-cell mTRP scenarios were also discussed for mTRP [1]:

Issue 3-1-3: For extension of Rel-17 unified TCI framework, whether to support intra-cell mTRP and inter-cell mTRP scenarios? - Proposals - Option 1: intra-cell only - Option 2: both intra-cell and inter-cell

RAN1 is addressing both intra-cell and inter-cell scenarios. As such, RAN4 should have requirements for both scenarios as well. Note that inter-cell scenario is only applicable to m-DCI configuration. The following is agreed in RAN1:

Agreement (RAN1#109e) : On unified TCI framework extension, consider all the intra and inter-cell MTRP schemes specified in Rel-16 and Rel-17

Define requirements for both intra-cell and inter-cell MTRP scenarios for the extension of the Rel-17 unified framework.

Simultaneous reception

Another topic was whether to consider simultaneous reception in mTRP for the extension of the Rel-17 unified TCI framework [1]:

Issue 3-1-4: For extension of Rel-17 unified TCI framework, whether to support simultaneous reception in mTRP? - Proposals - Option 1: not consider simultaneous reception in mTRP in Rel-18 - Option 2: Consider simultaneous reception in mTRP in Rel-18, FFS on how to do the extension

NR_FR2_multiRX_DL WI is already discussing requirements for simultantous reception in mTRP scenario. However,

the WI only considers the Rel16 TCI framework, since it is basing the requirements for simultaneous reception on existing mTRP framework without relying on the extension of the unified TCI framework for mTRP operation. Considering that simultaneous reception is also being included as part of the scope for RAN1 work in MIMO_evo Rel-18, it is important that those enhancements are also reflected as part of the RAN4 RRM requirements for TCI switching.

The requirements defined for NR_FR2_multiRX_DL do not include unified TCI framework.

RAN1 is defining extension of unified TCI framework for mTRP operation in Rel-18.

MIMO_evo_D_UL Rel-18 to include simultaneous reception in mTRP for extension of Rel-17 unified TCI framework.

Independent beam management

Regarding the extension of unified TCI framework to mTRP, RAN4 has defined IBM and CBM operation for CA cases where the IBM and CBM definitions are illustrated in Figure 3. The use cases for IBM and for CBM are shown in Figure 4, where CBM is only defined for intra-band CA operation and IBM is addressing inter-band CA operation.

Figure 3 Illustrations of IBM and CBM definitions.

Figure 4 Illustration of intra-band CA versus inter-band CA configuration with IBM and CBM operation, including timing alignment error (TAE) requirements.

There is no inter-band CA CBM from outcome of Rel-17 study in RAN4.

Regarding how to do the extension of unified TCI framework to mTRP, RAN4 could consider that IBM framework should be the baseline.

TCI state switch/activation requirements

The following issues were discussed for TCI state switch/activation requirements for the extension of the Rel-17 unified TCI framework [1]:

Issue 3-1-8: How to specify DCI based TCI state switch requirements? - Proposals - Option 1: FFS. (Intel) - Option 1a: DCI based TCI state switch requirement may be updated depends on RAN1 progress. (Intel)

The configuration of the pool of TCI states is done in RRC and it can be done either for joint or for separate TCI operation. In joint operation the pool is up to 128 DL/UL TCI states. In separate configuration, there are two pools, one for DL and one for UL. They each contain up to 64 (for PDCCH) or 128 (for PDSCH) for DL and 32 or 64 TCI states for UL. This is illustrated in Figure 3.

Then, MAC can activate a pool of up to 8 TCI codepoints, each of them either pointing to a DL, an UL or a DL/UL pair of TCI states. In separate TCI configuration, the maximum number of activated codepoint is 8 therefore the maximum number of activated TCI states can be {8 DL + 8 UL}, in the case that each codepoint points to a pair of DL/UL TCI states.

Finally, the indication is done via DCI whenever the pool of activated TCI states contains more than 1 TCI state.

Figure 3: configuration, activation, and indication of TCI states for Rel-18.

Furthermore, RAN1 has already agreements on the use of DCI for the extension of the unified framework for mTRP operation with sDCI and mDCI:

Agreement (RAN1#109e) On unified TCI framework extension at least for single-DCI based MTRP, the existing TCI field in DCI format 1_1/1_2 (with or without DL assignment) can indicate multiple joint/DL/UL TCI states in a CC/BWP or a set of CCs/BWPs in a CC list - FFS: Detail of mapping joint/DL/UL TCI state ID(s) to a TCI codepoint, e.g., possible combinations of joint, DL, and/or UL TCI state IDs that can be mapped to a TCI codepoint - FFS: Whether to increase the max number of MAC CE activated TCI codepoints, i.e., more than 8 codepoints - FFS: Whether to increase the max number of TCI field bits, i.e., more than 3 bits - Note: This doesn't imply that support of one additional TCI field or a field associating the TCI field to the TRP(s) is precluded Note: The term TRP is used only for the purposes of discussions in RAN1 and whether/how to capture this is FFS Agreement (RAN1#110bis-e) On unified TCI framework extension for M-DCI based MTRP: - For a serving cell configured with joint DL/UL TCI mode, one joint TCI state can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment) For a serving cell configured with separate DL/UL TCI mode, a DL TCI state, an UL TCI state, or a pair of DL and UL TCI states can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 (with or without DL assignment)

Considering those agreements on DCI extension, it is possible to agree that DCI based TCI switching requirements should be defined for the extension o mTRP in Rel 18.

Define DCI-based TCI switching requirements for the extension of mTRP in Rel-18.

Conclusion

In this paper we have analysed the extension of unified TCI framework for mTRP operation. As part of this discussion, the following Observations and Proposals were derived:

Observation 1: Rel-18 specifies the indication of two codepoints in the same DCI, thus enabling s-DCI m-TRP operation.

Observation 2: m-TRP with s-DCI requires extension of the numbers of codepoints for indication of the TCI state i.e. enabling 2 indicated TCI states from the same list of active TCI states (open in RAN1).

Observation 3: m-TRP with m-DCI is equivalent to parallel Rel-17 mechanism per TRP.

Observation 4: s-DCI may be scheduled for either joint or separate TCI configurations.

Observation 5: m-DCI may be scheduled for either joint or separate TCI configurations.

Proposal 1: Confirm agreement: Both sDCI and mDCI based MTRP are considered for extension of Rel-17 unified TCI framework for multi-TRP.

Observation 6: TCI switching commands for mTRP are different for sDCI and mDCI.

Proposal 2: RAN4 to define different requirements for sDCI and mDCI scenarios.

Proposal 3: RAN4 to define separate requirements for both single-DCI and multi-DCI scenarios in MIMO_evo Rel-18 including support of both configurations of joint or separate frameworks.

Proposal 4: Define requirements for both intra-cell and inter-cell MTRP scenarios for the extension of the Rel-17 unified framework.

Observation 7: The requirements defined for NR_FR2_multiRX_DL do not include unified TCI framework.

Observation 8: RAN1 is defining extension of unified TCI framework for mTRP operation in Rel-18.

Proposal 5: MIMO_evo_D_UL Rel-18 to include simultaneous reception in mTRP for extension of Rel-17 unified TCI framework.

Observation 9: There is no inter-band CA CBM from outcome of Rel-17 study in RAN4.

Proposal 6: Regarding how to do the extension of unified TCI framework to mTRP, RAN4 could consider that IBM framework should be the baseline.

Proposal 7: Define DCI-based TCI switching requirements for the extension of mTRP in Rel-18.

- References

- R4-2303258, WF on R18 NR MIMO RRM requirements, Samsung.