3GPP TSG RAN WG4 Meeting #106bis		R4-2304335

Online meeting, April 17th – 26th 2023

Agenda item:	5.6.2

Source:	Apple, Globalstar, Ligado Networks

Title:	Flexible channel raster for the NTN bands

WI/SI:	NR_channel_raster_enh

Release:	Rel-18

Document for:	Decision

# 1	Introduction

During Rel-16 and Rel-17 discussions, several operators expressed an interest in enabling more efficient utilization of "non-standard" channel bandwidths, i.e., the ones which are not present now in TS 38.101 specifications. Referring to the corresponding operator requests, the following channel bandwidths were suggested by operators: 6, 7, 11, 12, 13, 33MHz. As an outcome a new SI was agreed at the RAN#89 meeting aiming to study further which existing solutions can be used and whether new mechanism should be devised [1]. The SI was concluded at RAN#99 meeting with the general conclusions that two methods – overlapping channels from network perspective and the next larger channel – can be used to support irregular channels without requiring changes at the UE side. In addition to that, companies concluded that the existing channel raster design is not flexible and does not allow configuring certain NR channel combinations. In response to that RAN#99 agree a new WI with the only main objective to enable a more flexible raster design [2].

While most of the previous discussions for the irregular channels and flexible channel raster were conducted in the context of the TN bands, the same problem exists also for the NTN bands. In fact, this issue was already raised earlier during the SI on efficient usage of irregular spectrum blocks. Thus, in this discussion paper we propose to consider and apply the channel raster enhancements also to the NTN bands. In fact, channel raster enhancements, which RAN WG4 agrees for the FR1 TN bands, should be also applicable for the NTN bands.

# 2	Channel raster enhancements for the NTN bands

Most of the FR1 NTN bands are "irregular" in a sense that their size is not aligned on the 5MHz boundary. As can be seen from the Table 2-1 below, only the satellite S-band, n256, is aligned on the 5MHz boundary. On the contrary to it, neither L-band (n255) nor L-/S-band (n254) are aligned on 5MHz. Furthermore, the so-called "extended" L-band size is also irregular occupying the 7MHz spectrum block. In other words, even if the L-band and the extended L-band are treated as one band, its total size is still irregular taking the 41MHz spectrum block.

Table 2-1: Summary of the NTN bands

| Satellite band | 3GPP band | Band size (MHz) |  |
| --- | --- | --- | --- |
| L-band | n255 | 34 | 41 |
| Extended L-band | [TBD] | 7 |  |
| L-/S-band | n254 | 16.5 |  |
| S-band | n256 | 30 |  |

As concluded in TR 38.849, there are several methods that can be used to support irregular channels, one of which is overlapping channels from the network perspective. This approach is based on configuring two overlapping cells such that they form a contiguous set of RBs allowing the network to schedule data over all RBs. However, as extensively discussed by RAN WG4, if the network has to configure both overlapping channels on the 100kHz raster time ensuring that the cells are RB aligned, then the least common multiplier between these requirements is 900kHz, i.e. 5RBs. For certain irregular channels it prevents the network from more efficient utilisation of available spectrum resources.

Figure 2-1 below shows the 7MHz spectrum block that can be supported with two 5MHz cells. If both cells have to be configured on the 100kHz raster, then the second cell can be shifted only by 1800kHz, i.e.10RBs, thus allowing the network to use 35RBs in the 7MHz spectrum block. However, if the second cell can be configured on the non-100kHz raster, then a better spectrum utilisation can be achieved with 36RBs still meeting the minimum guard band requirements of each overlapping cell.

Figure 2-1: Exemplary usage of overlapping carriers with 100kHz and non-100kHz alignment for the 7MHz channel.

In the Table 2-1 below we present the summary of potential gains that can be achieved for the irregular NTN spectrum blocks if a more flexible channel raster is enabled also for the NTN bands. For the sake of reference, we present how many RBs can be scheduled when both overlapping cells are on the 100kHz raster and when at least one of the cells can be on non-100kHz raster. As can be seen from the table, for almost all the irregular channels we can gain at least one extra RB; and for 16.5MHz and 34MHz channel we can gain even more than one RB.

Table 2-1: Summary of available RBs for overlapping channels in the irregular spectrum blocks.

| Irregular channel (MHz) | Next smaller channel (MHz) | Minimum guard band (kHz) | 100kHz raster |  |  | non-100kHz raster |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | N_RB | Guard band (kHz) | Utilisation (%) | N_RB | Guard band (kHz) | Utilisation (%) |
| 7 | 5 | 242.5 | 35 | 342.5 | 90 | 36 | 252.5 | 92.6 |
| 16.5 | 15 | 382.5 | 84 | 682.5 | 91.6 | 87 | 412.5 | 94.9 |
| 34 | 30 | 592.5 | 180 | 792.5 | 95.3 | 182 | 612.5 | 96.4 |
| 41 | 40 | 552.5 | 221 | 602.5 | 97 | 221 | 602.5 | 97 |

Proposal:	Channel raster enhancements will be also applied to the NTN bands.

# 3	Conclusions

In this discussion paper we have presented our view on the benefits of applying channel raster enhancements also fort the NTN FR1 bands. As explained in the document, most FR1 NTN bands are irregular occupying 7, 16.5 and 34MHz spectrum blocks. Like the conclusions made by RAN WG4 for the irregular TN spectrum blocks, a more flexible channel raster can facilitate better usage of NTN spectrum resources. Thus, we suggest applying same channel raster enhancements also for the NTN bands. Practically speaking whatever channel raster enhancements RAN WG4 will end up with for the TN FR1 bands should automatically become available also for the NTN bands.

Proposal:	Channel raster enhancements for the TN bands will be also applied to the NTN bands.

# 4	References

RP-202103, "New SID: Study on Efficient utilization of licensed 	spectrum that is not aligned with existing NR channel bandwidths", T-Mobile USA, Ericsson

RP-230813, "New WID on Channel raster enhancement", Ericsson