3GPP TSG-RAN WG4 Meeting # 106-bis-e 	R4-2304593

E-meeting, April 17th – 26th, 2023

Agenda Item: 5.10.3.2

Source: MediaTek Inc.

Title: Discussion on inter-RAT measurements

Document for: Discussion

# Introduction

In Rel-18, a work objective to complete the requirements for measurement without gaps is given in the work item description (WID) [1] as below:

- Define RRM requirements for measurement without gaps for the following cases - Inter-RAT measurements without gaps [RAN4] - Inter-RAT NR measurements - Inter-RAT LTE measurement

In meeting RAN4#106, the above was discussed and the open issues were captured in the WF [2]. The analysis and discussion on the open issues are provided in the next section.

# Discussion on inter-RAT measurements without gaps

The analysis and discussion are provided in the next section.

## Discussion on UE capabilities

## Sub-topic 2-2: UE capabilities ### Issue 2-2-1: On top of which UE capability to support the inter-RAT NR measurement without gap when UE has vacant RF chain available (Case a-1) < Agreement >: - Add the request on the additional signaling from UE to indicate the inter-RAT NR measurements without gap but interruption needed in the inter-RAT NR measurement without gap (case a-1) to the LS to RAN2 ### Issue 2-2-2: On top of which UE capability to support the inter-RAT LTE measurement requirements when UE has vacant RF chain available(Case b-1) < Way forward >: - Option 1: Apple, Qualcomm, Intel, CATT, vivo, OPPO, Huawei, Ericsson, MTK, xiaomi - ONLY on top of ‘nogap-noncsg’ in NeedForNCSG-InfoEUTRAN to define the UE capability to support Case b-1 - Option 2: CMCC - Both NeedForNCSG-InfoEUTRAN and NeedForGap shall be considered also. Issue 2-2-4: On top of which UE capability to support the inter-RAT LTE measurement requirements when LTE CRS to be measured is contained in UE’s active BWP(Case b-2) < Way forward/ >: - Option 1: OPPO, Xiaomi - extend the capability of NeedForGaps - Option 2: Apple, Qualcomm, Intel, CATT, CMCC, vivo,Huawei, Ericsson, MTK - A new per-UE capability should be defined - FFS on the detailed design on this new IE ### Issue 2-2-5: Additional capability to support inter-RAT measurement without gap with mixed numerology < Way forward >: - Option 1: CATT, Huawei, MTK: No additional UE capability is defined for inter-RAT measurement with mixed numerology; instead it can be considered for scheduling restriction - Option 2: MTK: a UE capability can be defined for mixed numerology incapable UEs (FFS) - Option 2a: Intel, vivo: a UE capability for inter-RAT NR can be defined (FFS)

Issue 2-2-2: On top of which capability for case b-1: The existing UE capability reporting for NCSG supports inter-RAT LTE measurements, which can be reused for inter-RAT gapless measurements for case b-1. However, the existing capability of NeedForGap doesn’t support inter-RAT LTE measurements, which means a new UE capability reported is needed to be defined if NeedForGap is considered.

- The existing UE reporting capability of NeedForGap does not support inter-RAT LTE measurements.

In addition, interruption is needed to allow the available RF chain to switch ON and OFF (switched OFF outside the measurement occasion), hence, better power saving. Thus, the UE requires interruption for successful operation of case b-1. Therefore, the UE should be able to indicate ‘ncsg’ and ‘nogap-noncsg’ via NeedForNCSG-InfoEUTRAN for the inter-RAT LTE measurement. However, the requirements for the UE indicating ‘ncsg’ is already specified, while the requirements for ‘nogap-noncsg’ is not specified. Therefore, the requirements for NCSG inter-RAT capability shall be completed (i.e. to define requirements for ‘nogap-noncsg’). Besides, we don’t see the need to define inter-RAT gap-less requirements on top of NeedForGap capability because that means RAN4 needs to define a new ‘duplicated’ capability, similar to that of NCSG capability.

- RAN4 shall reuse existing capability where the UE indicates ‘nogap-noncsg’ and ‘ncsg’ via NeedForNCSG-InfoEUTRAN for the inter-RAT LTE measurement.

- RAN4 shall complete the requirements for NCSG inter-RAT to cover the scenario of ‘nogap-noncsg’.

- RAN4 shall not define additional capability for case b-1.

Issue 2-2-4: On top of which capability for case b-2

On one hand, case b-2 is defined where the reference signals for inter-RAT are contained within the UE’s active BWP and the UE should be able to measure these reference signals regardless of whether additional RF chain is available or not. In addition, further study is needed to decide whether interruption is needed or not. This is because although the reference signals are contained in the active BWP, yet the interruption might be needed because of the impact from frequency offset, timing synchronisation, AGC. On the other hand, NeedForNCSG-InfoEUTRAN via NCSG capability is defined on the basis that an RF chain is available at the UE. Thus, it is not feasible to reuse the NeedForNCSG-InfoEUTRAN for case b-2. Now, given that NeedForGap doesn’t have UE signalling capability to perform inter-RAT LTE measurements, hence a new signalling UE capability needs to be defined. The new capability for case b-2 should be based on static reporting rather than dynamic reporting. Therefore, RAN4 shall send an LS to RAN2 to request them to define a new UE capability after RAN4 reaches conclusion on whether interruption is needed or not.

- RAN4 shall further study whether interruption is needed for case b-2 and to address the issues of AGC, and time and frequency offsets.

- RAN4 shall request RAN2 to define a new UE signalling capability for case b-2.

Issue 2-2-5: Mixed numerology: Mixed numerology is needed for inter-RAT because the typical scenario for 5G NR operation is with SCS = 30KHz, which is different than that of LTE, which is SCS = 15KHz. Thus, RAN4 discussed whether there is a need to define a UE capability for the inter-RAT mixed numerology or to define scheduling restrictions. In general, scheduling restrictions are specified in 5G NR based on the SMTC configuration, however, in LTE the scheduling restrictions doesn’t exist also the SMTC doesn’t exist either. Therefore, RAN4 can agree on specifying scheduling restrictions for inter-RAT NR measurements, yet RAN4 needs to study and specify the requirements of scheduling restrictions for inter-RAT LTE measurements. RAN4 should introduce a new UE capability to handle the mixed numerology, also, define scheduling restriction for incapable UEs.

- RAN4 shall introduce a new UE capability to handle the mixed numerology and define scheduling restrictions for mixed numerology incapable UEs.

## Discussion on UE measurement requirements

## Sub-topic 2-3: Measurement requirements ### Issue 2-3-1: Frameworks used to define inter-RAT NR measurement without gap (case a-1) < Way forward >: - Option 1a: Qualcomm - Using 8.17.4 of TS36.133 inter-RAT NR measurement delay requirements with No DRX as baseline and - define effective MGRP for measurements without gap for FR1 and FR2 - Option 1b: Intel, CMCC, xiaomi, OPPO,vivo, CATT - For the inter-RAT NR measurements without gap (case a-1), the requirements can be based on NR inter-frequency measurement without gap in 9.3.9 TS38.133 - Option 2: Apple, Huawei, MTK - to follow the requirements from NR intra- and inter-frequency requirements based on NeedForGaps( depending on issue 1-2-1) - Option 2a: Huawei - RAN4 to discuss whether to follow the requirements from NR intra- and inter-frequency requirements based on NeedForGaps, or to define requirements based on the assumption that no interruption is expected. - Companies are encouraged to identify any updates needs based on the suggested frameworks also. ### Issue 2-3-2: Framework used to define inter-RAT LTE measurement without gap (case b-1) < Way forward >: - Option 1: Qualcomm, CMCC, OPPO, MTK - For the inter-RAT LTE gap-less measurement, the requirements can be based on the existing inter-RAT LTE measurement requirements in TS38.133[3] 9.4. - Option 1a: Qualcomm, OPPO - update definition of Tinter1 and CSSFinterRAT - Multiple Tinter1 (30ms, 60ms) can be considered, and new indicator needs to be introduced for UE to indicate NW what Tinter1 is applied. - Option 1b: CMCC, OPPO - update definition of Tinter1 - Option 1c: MTK - RAN4 to introduce the requirements to cover the scenario of ‘nogap-noncsg' - Option 2a: Huawei,vivo - For the inter-RAT LTE gap-less measurement, the requirements can be based on the measurement requirements for LTE inter-frequency measurement in TS36.133. - Option 2b: CATT - For the inter-RAT LTE gap-less measurement, the requirements can be based on the measurement requirements for LTE intra-frequency measurement in TS36.133. - Option 3: Apple - RAN4 needs to develop requirement for case b-1 based on NCSG requirements in 9.3.10 TS38.133 with UE reporting ‘nogap-noncsg’, e.g., take NCSG period = 80ms as baseline and remove VIL. - Option 4: Ericsson - directly discuss the UE’s behaviour for inter-RAT LTE measurement without gap (for both case b-1 and b-2) instead of checking the possible reused framework - Companies are encouraged to identify any updates needs based on the suggested frameworks also. Issue 2-3-3: Framework used to define inter-RAT LTE measurement without gap (case b-2) < Way forward >: - Option 1: Qualcomm, Intel, , OPPO, MTK - For the inter-RAT LTE gap-less measurement, the requirements can be based on the existing inter-RAT LTE measurement requirements in TS38.133[3] 9.4. (same as Option 1 for case b-1) - Option 1a: Qualcomm - update definition of Tinter1 and CSSFinterRAT - Multiple Tinter1 (30ms, 60ms) can be considered, and new indicator needs to be introduced for UE to indicate NW what Tinter1 is applied. - Define applicability rules for case b-2 requirements - The requirements for inter-RAT LTE measurement without gap when LTE CRS is in UE’s active BWP are only applicable when UE is in DSS but LTE cell is not serving. - Option 1b: MTK - RAN4 to introduce the requirements to cover the scenario of 'nogap-noncsg' - Option 2a: Huawei, vivo - For the inter-RAT LTE gap-less measurement, the requirements can be based on the measurement requirements for LTE inter-frequency measurement in TS36.133. (same as Option 2 for case b-1) - Option 2b: Apple, CATT - For inter-RAT LTE measurement without gap requirements (case b-2), LTE intra frequency measurement requirements in TS36.133 is considered as baseline. Details needs to be updated. - Companies are encouraged to identify any updates needs based on the suggested frameworks also.

Issue 2-3-1: framework for inter-RAT NR measurement without gap: The requirements are related to the NeedForGap signalling agreed in previous meeting. RAN4 is currently defining the requirements for intra-frequency and inter-frequency measurements in rel-18 NeedForGap, which allow interruption for NeedForGap. Thus, RAN4 can use the inter-frequency requirements as baseline to define the requirements for inter-RAT NR.

- RAN4 shall use the requirements from inter-frequency Rel-18 NeedForGap as baseline to define inter-RAT NR measurement without gap.

Issue 2-3-2: framework for inter-RAT LTE measurement without gap (case b-1): The requirements are related to the NCSG signalling. For the inter-RAT LTE gap-less measurement, the requirements can be based on the existing inter-RAT LTE measurement requirements in TS38.133 clause 9.4, however, the requirements for ‘nogap-noncsg’ needs to be defined.

- RAN4 can use the existing inter-RAT LTE measurement requirements in TS38.133 clause 9.4, however, the requirements for ‘nogap-noncsg’ needs to be defined.

Issue 2-3-3: framework for inter-RAT LTE measurement without gap (case b-2): for Case b-2 RAN4 needs to reach conclusion on whether interruption is needed or not before defining the requirements. Given that the signals in the active BWP are coming from two different sources, i.e. LTE and NR, hence there is a need for an AGC delay to adapt the AGC. Furthermore, the LTE signals comes from different gNB compared to NR signals, which may result in frequency offset and time offset between the LTE and NR signals. Therefore, RAN4 shall discuss the impact of AGC adaptation, time and frequency offset before agreeing on the interruption issue.

- For case b-2, RAN4 shall discuss the impact coming from AGC, time offset and frequency offset.

## Discussion on scheduling restriction, searcher limitation and CSSF

### Issue 2-3-4: Scheduling restriction for inter-RAT NR measurements < Way forward >: - Option 1a: Intel, vivo - The existing scheduling availability specified for inter-frequency measurements in TS 38.133 section 9.2.9.3 can also be applied to the inter-RAT NR measurement without measurement gaps as a start point. - Option 1b: OPPO - The existing scheduling availability specified for inter-frequency measurements without gap in TS 38.133 section 9.3.9.3 can also be applied to the inter-RAT measurement without measurement gaps or interruption. - The existing scheduling availability specified for inter-frequency measurements without gap in TS 38.133 section 9.3.10.3 can also be applied to the inter-RAT measurement without measurement gaps but interruption. - Option 2: Huawei, MTK - The existing scheduling restriction requirements in clause 9.3.10.3 can be taken as baseline - Option 2a: Huawei - except that scheduling restriction is not limited to NCSG occasions ### Issue 2-3-5: Scheduling restriction for inter-RAT LTE measurements < Way forward >: - FFS on : - Proposal 1: Huawei - Scheduling restriction due to inter-RAT LTE measurement are applicable when - UE does not support simultaneous Tx and Rx on the serving cell and target band - Serving cell and target MO have mixed SCS and they are in the same band - Proposal 2: Ericsson - RAN4 to study the following scheduling restriction principles based on LTE measurement RSs, - How to apply the restriction symbols before and after the CRS symbols for inter-RAT LTE measurement without gap. - Whether to introduce new UE capability to support inter-RAT LTE measurement and NR data reception - Proposal 3: Ericsson - When the target inter-RAT LTE frequency layers belong to an inter-band with the serving cells, no scheduling restriction is expected for inter-RAT LTE measurement without gap. - When the target inter-RAT LTE frequency layers belong to an intra-band with the serving cells, scheduling restriction is expected, such as mix-numerology and simultaneousRxTxInterBandCA ### Issue 2-3-8: General principle to define the requirements < Way forward >: - FFS on: - Proposal 1: Ericsson - The inter-RAT LTE measurement without gaps should be performed outside the SMTC/SSB to avoid the performance degradation to legacy NR intra-frequency measurement without gap and L1-RSRP measurement. - Proposal 2: Ericsson - Both NW and UE shall have the same understanding on the measurement occasions for Inter-RAT E-UTRAN measurement without gap. ### Issue 2-3-9: Effective measurement window < Way forward >: - Option 1: Huawei, Ericsson, vivo - RAN4 to introduce an effective measurement window for inter-RAT E-UTRAN measurement without gap. - Option 1a: Ericsson - The effective measurement window can be defined based on measurement duration, measurement periodicity and offset. - Option 2: MTK - FFS ### Issue 2-3-6: Searcher limitation < Way forward >: - Option 1: Ericsson - Inter-RAT LTE measurement without gap(case b-2) can be performed in parallel with NR measurement without searcher limitation - Option 2: Apple, vivo, MTK, Huawei - Performing inter-RAT measurement and NR measurements in parallel without searcher limitation is NOT supported. ### Issue 2-3-7: CCSF < Way forward >: - Option 1: Apple, Intel, Huawei - The updates of CSSF requirements when these inter-RAT measurements without gap introduced (e.g. CSSF_outside_gap) is needed - How to update CCSF can be FFS. - Option 1a: Ericsson - The scaling factor for inter-RAT E-UTRAN measurement without gap equals to the total number of frequency layers for E-UTRAN measurement without gap.

Issue 2-3-4/2-3-5/2-3-8/2-3-9: Scheduling restriction: For scheduling restriction for inter-RAT NR measurement with interruption, the closest requirement is the inter-frequency scheduling restriction with NCSG. However, for case a-1, the candidate-baseline requirements are rel-18 inter-frequency NFG requirements, as discussed in issue 2-3-1. The current agreement in RAN4 is to define interruption requirements with interruption ratio. Now back to the scheduling restriction issue, with NCSG the SMTC occasion for measurement is known but for NFG it is not clear which SMTC occasion is used for measurement, thus the default SMTC pattern can be defined to restrict the scheduling restriction occasions. Therefore, RAN4 can use the existing scheduling availability specified for inter-frequency measurements without a gap in TS 38.133 section 9.3.10.3 as a baseline for the inter-RAT measurement without measurement gaps with interruption. Yet, default SMTC pattern can be defined to restrict the scheduling restriction occasions.

On the other hand, regarding scheduling restrictions for inter-RAT EUTRAN, it is not clear how to define the scheduling restriction without the use of SMTC, therefore, RAN4 should define alike SMTC window for LTE or effective measurement window. Yet, RAN4 should take into consideration and study the collision/overlap between the SMTC from NR with SMTC from LTE.

- For scheduling restriction for inter-RAT NR measurements, RAN4 should use the existing scheduling availability specified for inter-frequency measurements without a gap in TS 38.133 section 9.3.10.3 as a baseline for the inter-RAT measurement without measurement gaps with interruption. Yet, default SMTC pattern can be defined to restrict the scheduling restriction occasions.

- For scheduling restriction for inter-RAT LTE measurements, RAN4 should define SMTC pattern for LTE or effective measurement window, yet FFS for the overlap between the SMTC in LTE and NR.

Issue 2-3-6: Searcher limitation (parallel measurements): The closest requirements that can be taken as baseline is NCSG requirements (i.e. the CSSFwithin_ncsg), which is requirements for interruption scenario without gap. From the existing specification, the legacy requirement requirements handle one frequency at a time because UE’s RF is not guaranteed to measure two inter-frequency/inter-RAT at the same time. Now given that this issue is related to parallel measurements scenarios, we suggest delaying the discussion on searcher limitation after we reach conclusion on parallel measurements.

- RAN4 shall delay the discussion on searcher limitation requirement until RAN4 reaches conclusion on parallel measurements.

Issue 2-3-7: CSSF: In general, the CSSF requirements should follow the agreement of searcher limitation. Therefore, RAN4 shall decide on whether parallel measurements are expected first and then define the CSSF requirements. In our opinion, RAN4 should keep the same principle for NR measurement as Rel-15.

- RAN4 shall keep the same searcher limitation, from Rel-15, for NR in the CSSF requirement or wait until RAN4 reaches a conclusion on parallel measurements.

## Discussion on other aspects

### Issue 2-3-10: Interruption requirements for inter-RAT measurement without gap < Way forward/Agreement >: - Interruption requirements can be defined - for case a-1, - FFS for case b-1 and b-2 ### Issue 2-3-11: Release independent requirements < Way forward >: - FFS on whether to introduce the inter-RAT LTE measurements without gap case b-2 as release independent from Rel-17 - Option 1: Ericsson - RAN4 to discuss whether to introduce the inter-RAT LTE measurements without gap for DSS (case b-2) as release independent from Rel-17 - Option 2: Huawei, MTK - RAN4 shall not discuss the release independent issue until sufficient progress has been achieved.

Issue 2-3-10: Interruption requirements: As mentioned earlier in this document for the issues 2-2-1, 2-2-3, 2-2-4, interruption is needed for case a-1, case b-1 and possibly for case b-2. Therefore, interruption requirements should be defined for case a-1 and b-1, while case b-2 should be for further study to address the AGC issue and timing and frequency offsets.

- RAN4 shall define interruption for case a-1 and case b-1, while FFS for case b-2 to address the AGC and timing and frequency offsets.

Issue 2-3-11: Release independent requirements: whether to support such release independent from Rel-17 depends on the defined requirement for the inter-RAT. For example, how similar or different these requirements from existing NCSG requirements. Therefore, we suggest delaying the discussion on this issue for later. Thus, we support option 1a in issue 2-3-9.

- RAN4 shall not discuss the release independent issue until sufficient progress has been achieved.

# Summary

In this contribution, discussion on inter-RAT measurement without gaps is provided and we have the following proposals:

Proposal 1: RAN4 shall reuse existing capability where the UE indicates ‘nogap-noncsg’ and ‘ncsg’ via NeedForNCSG-InfoEUTRAN for the inter-RAT LTE measurement.

Proposal 2: RAN4 shall complete the requirements for NCSG inter-RAT to cover the scenario of ‘nogap-noncsg’.

Proposal 3: RAN4 shall not define additional capability for case b-1.

Proposal 4: RAN4 shall further study whether interruption is needed for case b-2 and to address the issues of AGC, and time and frequency offsets.

Proposal 5: RAN4 shall request RAN2 to define a new UE signalling capability for case b-2.

Proposal 6: RAN4 shall introduce a new UE capability to handle the mixed numerology and define scheduling restrictions for mixed numerology incapable UEs.

Proposal 7: RAN4 shall use the requirements from inter-frequency Rel-18 NeedForGap as baseline to define inter-RAT NR measurement without gap.

Proposal 8: RAN4 can use the existing inter-RAT LTE measurement requirements in TS38.133 clause 9.4, however, the requirements for ‘nogap-noncsg’ needs to be defined.

Proposal 9: For case b-2, RAN4 shall discuss the impact coming from AGC, time offset and frequency offset.

Proposal 10: For scheduling restriction for inter-RAT NR measurements, RAN4 should use the existing scheduling availability specified for inter-frequency measurements without a gap in TS 38.133 section 9.3.10.3 as a baseline for the inter-RAT measurement without measurement gaps with interruption.

Proposal 11: For scheduling restriction for inter-RAT LTE measurements, RAN4 should define SMTC pattern for LTE or effective measurement window, yet FFS for the overlap between the SMTC in LTE and NR.

Proposal 12: RAN4 shall delay the discussion on searcher limitation requirement until RAN4 reaches conclusion on parallel measurements.

Proposal 13: RAN4 shall keep the same searcher limitation, from Rel-15, for NR in the CSSF requirement or wait until RAN4 reaches a conclusion on parallel measurements.

Proposal 14: RAN4 shall define interruption for case a-1 and case b-1, while FFS for case b-2 to address the AGC and timing and frequency offsets.

Proposal 15: RAN4 shall not discuss the release independent issue until sufficient progress has been achieved.

# References

- RP-230755, “New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, MediaTek Inc, Intel Corporation, March 2023, Rotterdam Netherlands.

- R4-2303305, “WF on NR_MG_enh2 Part 2”, Intel Cooperation, Athens, Greece, March 2023.