# 3GPP TSG-RAN WG4 Meeting # 106bis-e 	                   	      R4-2304672

# Electronic Meeting, April 17 – April 26, 2023

# Title: 	Discussion on L1-RSRP measurement requirements

# Source: 	ZTE Corporation

# Agenda item:	5.25.2.2

# Document for:	Approval

# 1 Introduction

In RAN#96e meeting, the work item [1] on study on on further NR mobility enhancements was approved as one of Rel-18 RAN4 package. During last RAN4 #106 meeting, we have lot reached lots of consensus on L1/L2 inter-cell mobility delay requirements in [2] for inter-cell mobility. In this contribution, we want to share further considerations on these issues.

- 2 Discussion

2.1 General principles

Issue 3-1-3: Whether to use intermediate L3 measurement results in L1 measurement reporting <Agreement>: - In FR2: - Fine beam can be assumed for L1 measurement on intra-frequency neighbor cell. FFS on inter-frequency neighbor cell. - FFS whether to consider rough beam also for L1 measurement on neighbor cell (including intra and inter-frequency).

In legacy L3 measurement, rough beam is assumed for L3 measurement and fine beam is assumed for L1 measurement. From what we understand, FR2 rough beam can not be assumed for L1 measurement.

Proposal 1. FR2 rough beam can not be assumed for L1 measurement.

Issue 3-1-4: Whether L1 measurement configured after receiving L3 measurement report on that cell < Wayforward >: FFS the following Options - RAN4 assumes UE performs L1 measurement on a neighbour cell after UE has performed L3 measurement on that cell - L3 measurement report is not the prerequisite of L1 measurement configuration

If UE performs L1-RSRP measurement after L3 measurement report, then UE can know the timing and coarse synchronization information based on L3 measurement. However, we notice that the current TS doesn’t define L1 measurement for unknown cell. If UE doesn’t perform L3 measurement at first, how could UE acquires the timing information and beam information for serving cell and how to perform L1 measurement. Also according to the definition of know cell that UE needs to send a valid L3 measurement report during the last 5 seconds. L3 measurement report could be the prerequisite of L1 measurement.

Proposal 2. L3 measurement report could be the prerequisite of L1 measurement.

Issue 3-1-6: known cell condition for L1-RSRP measurement < Wayforward>: FFS on following option - In L1-RSRP measurement for neighbour cell, target cell is considered as known if the following conditions are met in this requirement: - The UE has sent a valid L3 measurement report during the last [5] seconds, and - The SSB from the target cell remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3. - Otherwise, it is unknown

As noted above, inter-cell L1-RSRP measurement requirement is defined under known cell condition when the UE has sent a valid L3 measurement report during the last 5 seconds. Also, we don’t believe there has a difference between R-17 L1-RSRP measurement and R-18 LTM in know cell condition.

Proposal 3. In L1-RSRP measurement for neighbour cell, target cell is considered as known if the following conditions are met in this requirement:

- The UE has sent a valid L3 measurement report during the last [5] seconds, and

- The SSB from the target cell remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3.

2.2 Intra-frequency L1-RSRP measurement delay requirement

Issue 3-2-1: Intra-frequency L1-RSRP measurement requirements < Wayforward>: FFS on the following - Option 1: For intra-frequency L1-RSRP measurement on neighbor cell, use the requirements for L1 measurement on NSC in R17 as a baseline: - FFS: whether to consider multiple neighbor cells in a frequency layer, - FFS how to define requirements when RTD between neighbor cell and serving cell larger than a CP. - Option 2: For intra-frequency L1-RSRP measurement on neighbor cell, the requirements are designed based on measurement framework agreement.

In general, for L1 measurement on neighbour cell is similar with L1 measurement on a cell with different PCI in addition to serving cell. In R17, intra-frequency requirements in FR1 and FR2 for L1-RSRP measurement has been defined in section 9.13 of 38.133, which covers L1-RSRP measurements for a cell with different PCI from serving cell. And for R18 LTM, L1 measurement on a neighbour cell can be support multiple target cells. From what we understand, maybe we can use L1 measurement on NSC as a baseline. And for RTD, maybe we can define requirements when RTD between neighbour cell and serving cell smaller than a CP at first. Then we consider RTD between neighbour cell and serving cell larger than a CP.

Proposal 4. For intra-frequency L1-RSRP measurement on neighbor cell, use the requirements for L1 measurement on NSC in R17 as a baseline:

- FFS: whether to consider multiple neighbor cells in a frequency layer

- FFS how to define requirements when RTD between neighbor cell and serving cell larger than a CP.

2.3 L1-RSRP measurement accuracy

Issue 3-4-1: side condition of intra-frequency L1-RSRP measurement accuracy requirements < Wayforward >: FFS the following Options - Option 1 (Apple, MTK, OPPO, Huawei): Reuse legacy value SNR= -3dB - Option 2 (ZTE, Ericsson): SNR =-6dB (same as L3 measurement)

Another issue is whether L1-RSRP measurement accuracy requirements, which only used for beam managements can be applicable to L1/L2 mobility. In TS 38.133, side condition Ês/Iot in intra-frequency L1-RSRP measurement accuracy requirement is specified -3dB. And in legacy mobility, side condition Ês/Iot in intra-frequency and inter-frequency L3-RSRP accuracy requirement is -6dB. Based on our understanding, we could reuse legacy value of L3 measurement for L1/L2 mobility, at least -6 dB can be identical to legacy RRM.

Proposal 5. Propose to use legacy value of L3 measurement for L1/L2 mobility.

- 3 Conclusion

In this contribution, we give some discussions on L1/L2 inter-cell mobility delay requirements mobility, The conclusions are:

Proposal 1. FR2 rough beam can not be assumed for L1 measurement.

Proposal 2. L3 measurement report is not the prerequisite of L1 measurement configuration.

Proposal 3. In L1-RSRP measurement for neighbour cell, target cell is considered as known if the following conditions are met in this requirement:

- The UE has sent a valid L3 measurement report during the last [5] seconds, and

- The SSB from the target cell remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3.

Proposal 4. For intra-frequency L1-RSRP measurement on neighbor cell, use the requirements for L1 measurement on NSC in R17 as a baseline:

- FFS: whether to consider multiple neighbor cells in a frequency layer

- FFS how to define requirements when RTD between neighbor cell and serving cell larger than a CP.

Proposal 5. Propose to use legacy value of L3 measurement for L1/L2 mobility.

# 4	Reference

[1]  RP-221799, New WID on Further NR mobility enhancements, MediaTek.

[2]  R4-2303175, WF on L1/L2 inter-cell mobility, MediaTek.