3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2305781

Online, April 17 – April 26, 2023

Source: 	Skyworks Solutions, Inc.

Title: 	Issues with 2UL NRU-NRU inter-band combinations and invalid intra-band non-contiguous ULCA requests

Agenda Item:	4.1.1.2 Others	[WI code]

Document for:	Approval

# Introduction

In RAN#99, new inter-band combinations were non-contiguous intra-band ULCA NRU bands or NRU-NRU 2UL as UL configuration were requested in [1] and [2] respectively. In this contribution we detail implementation issues and missing generic and specific requirements to enable these types of combination.

# Discussion

## NRU non-contiguous ULCA

In RAN#99, the CA_n46(2A) and CA_n102(2A) UL configurations were requested in [1]:

- These combinations are invalid as there is no generic requirement for non-contiguous ULCA: neither MPR nor AMPR.

- Moreover, all wideband operation configuration strictly avoids non-contiguous sub-band.

- Finally, it is unclear how the different OOB emissions for n46 and n102 can be met with IMDS of two NRU CCs as it uses a single transmit path.

- IMDs of n46(2A) and n102(2A) will generate MSD and coexistence issues with existing bands as shown in Table 1.

Table 1: Range of IMDs of n46(2A) and n102(2A), all values in MHz

| n46(2A) range | 5150 | 5925 | victims | n102(2A) range | 5929 | 6425 | victims |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IMD2 range | 0 | 775 | some LB | IMD2 range | 0 | 496 | na |
| IMD3 range | 4375 | 6700 | n79+n46+n102 | IMD3 range | 5433 | 6921 | n46+n102 |
| IMD4 range | 0 | 1550 | all LB+LMB | IMD4 range | 0 | 992 | all LB |
| IMD5 range | 3600 | 7475 | n77/78+n79 +NRU bands | IMD5 range | 4937 | 7417 | n79+ NRU bands |
| IMD7 range | 2825 | 8250 |  | IMD7 range | 4441 | 7913 |  |

Proposal:

- UL configurations CA_n46(2A) and CA_n102(2A) are not valid in Release 18

- Band combinations using these UL configurations should be changed to n46A and n102A UL configurations which are valid

- The WI and band combination spreadsheet are revised accordingly

## NRU-NRU 2UL configuration

In RAN#99, the CA_n46-n102 UL configurations was requested in [2]:

- These 2UL cases are difficult to support with a single PA (or even 2PAs).

- Moreover, all wideband operation configuration strictly avoids non-contiguous sub-bands.

- Finally, it is unclear how the different OOB emissions for n46 and n102 can be met with IMDs of two NRU CCs as it uses a single transmit path

- For the same reasons the CA_n46-n96 has been restricted to 1UL and non-simultaneous Tx/Rx and is only used with another cellular band

- The IMDs of the CA_n46-n102 would cover a large spectrum and cause issues in NRU bands and to many cellular bands as shown in Table 2.

Table 2: Range of CA_n48-n102 IMDs, all values in MHz

| n46+n102 range | 5150 | 6425 | victims |
| --- | --- | --- | --- |
| IMD2 range | 0 | 1275 | all low bands |
| IMD3 range | 3875 | 7700 | n77, n79, n46, n102 |
| IMD4 range | 0 | 2550 | all LB/MB/HB bands |
| IMD5 range | 2600 | 8975 | all Cband + NRU bands |
| IMD7 range | 1325 | 10250 | all MB/HB/VHB/NRU bands |

Proposal:

- 2UL configurations CA_n46A-n102A are not supported in Release 18

- Band combinations using these UL configurations should be changed to n46A and n102A UL configurations with non-simultaneous TxRx

- The WI and band combinations spreadsheet are revised accordingly.

# Conclusions

In this contribution, we have looked at some NRU band combinations UL configurations and make the following proposals.

Proposal:

- UL configurations CA_n46(2A) and CA_n102(2A) are not valid in Release 18

- Band combinations using these UL configurations should be changed to n46A and n102A UL configurations which are valid

- The WI and band combination spreadsheet is revised accordingly

Proposal:

- 2UL configurations CA_n46A-n102A are not supported in Release 18

- Band combinations using these UL configurations should be changed to n46A and n102A UL configurations with non-simultaneous TxRx

- The WI and band combinations spreadsheet are revised accordingly.

# References

- RP-230067 Revised WID: Rel-18 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y), Ericsson, RAN#99

- RP-230133 Revised WID:Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE Corporation, RAN#99