3GPP TSG-RAN WG4 Meeting #107	R4-2307006

Incheon, South Korea, 22 – 26 May 2023

3GPP TSG RAN WG1 #112bis-e	R1-2304094

e-Meeting, April 17th – 26th, 2023

Title:	LS on PUSCH DMRS bundling for NR NTN coverage enhancement

Response to:	-

Release:	Rel-18

Work Items:	NR_NTN_enh-Core

Source:	RAN WG1

To:	RAN WG4

CC:	-

Contact Person:

#### Name:	Shohei Yoshioka

E-mail Address:	syouhei.yoshioka.py@nttdocomo.com

Send any reply LS to:	3GPP Liaisons Coordinator:  mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 has discussed timing error limit (Table 7.1C.2-1 in 38.133), frequency error limit (Section 6.4.1 in 38.101-5), and phase difference limit (Table 6.4.2.5-1 in 38.101-1) for DMRS bundling in NTN case and agreed the following observations and working assumption.

Observation For NTN-specific PUSCH DMRS bundling, in LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Timing error limit (Table 7.1C.2-1 in 38.133) can be satisfied within at most 13 slots if TA pre-compensation update is not assumed. - FFS: whether/how to consider the initial timing error at the beginning - FFS: TA pre-compensation update is assumed - Frequency error limit (Section 6.4.1 in 38.101-5) can be satisfied over 32 slots if frequency pre-compensation update is not assumed. - FFS: impact of phase difference limit Observation For NTN-specific PUSCH DMRS bundling, - In LEO 1200 with elevation angle 30 deg. and SCS = 15 kHz, RAN1’s understanding is the following: - Phase difference limit (Table 6.4.2.5-1 in 38.101-1) cannot be satisfied over multiple slots (for carrier bandwidth 5 MHz or larger), if the PRB allocation is not within 6 PRBs from the DC carrier, pre-compensation by UE and post-compensation by gNB are not assumed, and 70.5 (us/s) timing drift rate is assumed. - Note: this does not imply that UE shall be scheduled within 6 PRBs from the DC carrier. Working assumption For NTN-specific PUSCH DMRS bundling, to satisfy the phase difference limit without causing phase discontinuity, it is assumed that pre-compensation to keep phase rotation due to timing drift within the phase difference limit can be performed at UE side. - UE shall not perform TA pre-compensation update within an actual TDW if it causes phase discontinuity that may violate the phase difference limit. - FFS: how to determine the actual TDW - FFS: specification impact - Send an LS to RAN4

2. Actions

To TSG RAN WG4

ACTION: 	RAN1 respectfully asks RAN4 to take the above RAN1 observations and working assumption into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN1 Meeting #113				May 22nd-26th, 2023		Incheon

TSG-RAN1 Meeting #114				Aug. 21st-25th, 2023		Toulouse