3GPP TSG-RAN WG4 Meeting #107	R4-2307018

Incheon, South Korea, 22 – 26 May 2023

3GPP TSG RAN WG2 #121bis-e		R2-2304562

Electronic meeting, 17th – 26th April 2023

Title:	LS on Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs

Response to:

Release:	Release 17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	RAN WG2

To:	RAN WG1, RAN WG4

Cc:

Contact person:	Henrik Enbuske

Henrik.enbuske@ericsson.com

Attachments:	none

# 1	Overall description

RAN2 has discussed possible clarifications on monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs based on specification text in RAN2 and relevant sections in RAN1 and RAN4.

Current RAN2 specifications do not explicitly specify what happens for UEs in half duplex mode if a paging occasion conflicts with a CG-SDT occasion.

It is RAN2’s understanding that although information pertaining to this can be found in e.g., 38.213, clause 17.2 or in 38.133, clause 5.1B.2.6, the UE is only required to monitor paging for SI change indication in any paging occasion at least once per modification period during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB.

Similar to connected mode behaviour, since the UE is only required to monitor the paging in any paging occasion at least once per modification period, there should be other paging occasions available (within the modification period) to monitor the paging for SI change even if some of them overlap with the CG-SDT occasion(s).

Hence, RAN2 would like to ask RAN1 and RAN4 to take the above understanding into account and discuss possible amendment on misalignment between RAN2 specifications and RAN1 and/or RAN4 specifications.

# 2	Actions

To RAN WG1 and RAN WG4

ACTION: 	RAN2 would like to ask RAN1 and RAN4 to take the above understanding into account and discuss possible amendment on misalignment between RAN2 specifications and RAN1 and/or RAN4 specifications for CG-SDT with HD-FDD Redcap.

# 3	Dates of next TSG RAN WG2 meetings.

TSG RAN WG2 Meeting #122	22nd - 26th May 2023 	Incheon, KR

TSG RAN WG2 Meeting #123	21st – 25th August 2023	Toulouse, FR