3GPP TSG-RAN WG4 Meeting # 107		R4-2307272

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	5.2.8.5

Source: 		Qualcomm Incorporated

Title: 			Configuration of NTN specific parameters and open issues

Document for:	Approval

# 1.	Introduction

We present our views on the following aspects.

- Additional delay and Requirement Applicability in HO

- RLM and BFD/CBD Requirements

- Time and Frequency Impairments due to Satellite Mobility

- Open issues and incomplete test parameters

# 2. 	Discussion

Additional delay and Requirement Applicability in HO

Handover latency requirements are defined from the slot where UE received RRC message implying handover to the slot where the UE shall be ready to start the transmission of the new uplink PRACH channel. In order to transmit the PRACH, the UE has to obtain SFN level sync from the target cell for which the UE has to read partial-or-full MIB of the target cell. Having said that, in TN, the MIB reading during HO can be avoided by the following constraints on the time difference between serving and target cells – defined in TS38.211.

In contrast to TN, the above constraint may not hold true for NTN, in particular for inter-satellite HO/CHO. Therefore, UE requirements on inter-satellite handover latency/interruption will have to be extended to account for MIB reading time.

Proposal 1: RAN4 to add an additional delay to inter-satellite HO and CHO delay requirements to account for MIB reading time from the target cell to obtain SFN sync and select a proper RO (RACH Occasion) for PRACH transmission.

RLM and BFD/CBD Requirements

For NTN demodulation performance tests, two new channel models are newly introduced as below.

Observation 1: The following new fading models are introduced for NTN UE demodulation requirements

- New channel models: NTN-TDLA (NLOS) and NTN-TDLC (LOS)

  - NTN-TDLA: 100ns of delay spread, 200Hz of Doppler shift

  - NTN-TDLC: K-factor = 8.05dB, 100ns of delay spread, 200Hz of Doppler shift

As the existing fading channel models do not reflect NTN specific channel characteristics, fading channel models defined in NTN RLM and BFD/CBD test cases will have to be updated to one of the new NTN channel models. In such a case, the SNR conditions for the test cases will also have to be updated accordingly.

Proposal 2: For RLM and BFD/CBD test cases, propagation channel conditions need to be updated to one of the newly introduced NTN fading models from Demod session, and SNR conditions for the tests should be reevaluated.

Time and Frequency Impairments due to Satellite Mobility

It was agreed in RAN4#104 e-meeting that TE shall adjust the downlink transmission frame boundary and UL reception timing.

Agreements: (Agreed WF - R4-2214473, from RAN4#104 e-meeting)

Issue 5-3: Reference time for NTN UE timing related test cases.

    - For the test requirement, the reference time should be (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN

      - FFS NTA,UE-specific and NTA,common used by test system do not include UE GNSS estimation error and satellite positioning error from UE calculation.

    - In test cases for NGSO, a test equipment shall adjust the downlink transmission frame boundary/Doppler shift and UL reception timing according to open-loop TA control related parameters defined in the NTN specific system information and the satellite constellation with respect to UE position.

      - FFS the following aspects:

        - For the transmission timing/Doppler shift adjustment and UL reception timing, RAN4 defines a reference propagator model.

        - The reference propagator model shall be defined in such a way that those UEs using more accurate propagator model than the reference model are not penalized.

        - The reference model can be determined based on companies’ input. Eckstein Hechler based propagator model can be one of the candidate models.

For that, time instances for DL and UL shall be precisely defined in test case descriptions, e.g. Annex B. As presented in our contributions submitted in the past RAN4 meetings (R4-2203856, R4-2207960), NTN propagation delays over forward (DL) and reverse (UL) links are not symmetric (See S3/F3 and S4/F4 in Figure 1 and Figure 2) for the same slot index.

For an easier explanation, we take a feeder link propagation delay as an example. As can be read from RAN1 spec (excerpted below), Delay_common(t) is defined as a one-way propagation distance at a specific time instance ‘t’, not two-way propagation delay. Which means, in order to precisely calculate the reference timing of the two-way propagation delay for a UE transmit timing error measurement, TE should use two time instances as shown in Figure 1 and Figure 2, that is because the satellite’s position when the satellite receives UE UL uplink slot (slot #n+k) is different from the satellite’s position when the satellite relays the corresponding DL slot (slot #n+k) from the serving GW, i.e. asymmetric two-way delays, which is different from the traditional TN or GSO.

Fig. 1 Reference system model of timing relation between UE and UL timing synchronization reference point in NTN

Fig. 2 Timing Relation between UE DL Reception and UE UL Transmission

Based on the above explanation, we propose to capture how to model the time-varying time/frequency offsets and sampling frequency drift in one of satellite configuration related appendixes, e.g. Annex B, of TS38.133

Proposal 3: TE shall generate downlink signals such that the signals include the impacts of timing/frequency offsets and sampling frequency drift according to UE position, programed via AT command, and the reference satellite position, projected by Eckstein-Hechler model and the ephemeris information in the SIB19 transmitted before the most recent epoch time.

- The above should also apply to RF and Demodulation test cases, if applicable.

- As part of the time-varying timing offset characteristics to be modeled by TE, RAN4 to capture the following description in one of satellite configuration related appendixes, e.g. Annex B, of TS38.133.

  - When it is assumed that SRP is at the satellite, i.e. ta-Info is not provided in SIB19 or N_TA,common is set to a constant 0, and the time instance of UE UL transmission for slot#n at UE antenna ports, i.e. the start of slot#n minus (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc, is the start of downlink slot#n – X – Y, where X and Y are defined as below:

    - X is a one-way propagation delay of a DL transmission over the service link from the satellite to the UE at the moment when the downlink slot#n arrives at the satellite of which the position is projected by Eckstein-Hechler model and the ephemeris information in the SIB19 transmitted before the most recent epoch time and slot#n.

    - Y is a one-way propagation delay of a UL transmission over the service link from the UE to the satellite at the moment when the uplink slot#n arrives at the satellite of which the position is projected by Eckstein-Hechler model and the ephemeris information in the SIB19 transmitted before the most recent epoch time and slot#n.

    - The projected satellite positions at X and Y are assumed to be the same, hence the satellite position projected for X can be considered as the satellite position for Y, and vice versa.

Open issues and incomplete test parameters

There are parameters expected to be filled in by RAN5 based on orbit parameters and so on to be defined in RAN5 which will require satellite companies’ input. It may not be clear enough for RAN5 to figure which parameters and issues they need to work on.

Proposal 4: RAN4 to send an LS to RAN5 to inform of the list of open items and parameters expected to be closed and filled in by RAN5.

# 3.	Conclusion

In this paper, we proposed the following.

Additional delay and Requirement Applicability in HO

Proposal 1: RAN4 to add an additional delay to inter-satellite HO and CHO delay requirements to account for MIB reading time from the target cell to obtain SFN sync and select a proper RO (RACH Occasion) for PRACH transmission.

RLM and BFD/CBD Requirements

Observation 1: The following new fading models are introduced for NTN UE demodulation requirements

- New channel models: NTN-TDLA (NLOS) and NTN-TDLC (LOS)

  - NTN-TDLA: 100ns of delay spread, 200Hz of Doppler shift

  - NTN-TDLC: K-factor = 8.05dB, 100ns of delay spread, 200Hz of Doppler shift

Proposal 2: For RLM and BFD/CBD test cases, propagation channel conditions need to be updated to one of the newly introduced NTN fading models from Demod session, and SNR conditions for the tests should be reevaluated.

Time and Frequency Impairments due to Satellite Mobility

Proposal 3: TE shall generate downlink signals such that the signals include the impacts of timing/frequency offsets and sampling frequency drift according to UE position, programed via AT command, and the reference satellite position, projected by Eckstein-Hechler model and the ephemeris information in the SIB19 transmitted before the most recent epoch time.

- The above should also apply to RF and Demodulation test cases, if applicable.

- As part of the time-varying timing offset characteristics to be modeled by TE, RAN4 to capture the following description in one of satellite configuration related appendixes, e.g. Annex B, of TS38.133.

  - When it is assumed that SRP is at the satellite, i.e. ta-Info is not provided in SIB19 or N_TA,common is set to a constant 0, and the time instance of UE UL transmission for slot#n at UE antenna ports, i.e. the start of slot#n minus (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc, is the start of downlink slot#n – X – Y, where X and Y are defined as below:

    - X is a one-way propagation delay of a DL transmission over the service link from the satellite to the UE at the moment when the downlink slot#n arrives at the satellite of which the position is projected by Eckstein-Hechler model and the ephemeris information in the SIB19 transmitted before the most recent epoch time and slot#n.

    - Y is a one-way propagation delay of a UL transmission over the service link from the UE to the satellite at the moment when the uplink slot#n arrives at the satellite of which the position is projected by Eckstein-Hechler model and the ephemeris information in the SIB19 transmitted before the most recent epoch time and slot#n.

    - The projected satellite positions at X and Y are assumed to be the same, hence the satellite position projected for X can be considered as the satellite position for Y, and vice versa.

Open issues and incomplete test parameters

Proposal 4: RAN4 to send an LS to RAN5 to inform of the list of open items and parameters expected to be closed and filled in by RAN5.