3GPP TSG-RAN4 Meeting #107	                                                            R4-2307407

Incheon, KR, May 22 – May 26, 2023

Title:	Discussion on case 2 requirements for combination of NCSG and concurrent MGs

Source:	CATT

Agenda Item:	8.10.2.3

Document for:	Discussion

1 Introduction

In RAN4#106bis-e meeting, RRM requirements for the combination of Pre-MG, concurrent MGs and NCSG were discussed and the conclusions for the combination of NCSG and concurrent MGs were captured in the approved WF [1]. In this paper, we will further discuss this part and present our views.

2 Discussion

Scenarios:

In last meeting, the issue was discussed and the candidate options are as below:

- Issue 4-2-1: [Case 2] Whether to consider parallel measurements upon gap collision < Agreements from GTW >: - Option 1: Support of parallel measurements upon NCSGs collision is up to UE capability - Option 2: Do not support parallel measurements upon NCSGs collision - Issue 4-2-5: [Case 2] Whether to consider a new capability for NCSG + NCSG in an FR < Way forward >: - FFS the following options - Option 1a: - No, without UE capability if two NCSG patterns are supported based on single RF chain. - Option 2: - Yes, with UE capability - Option 2a: - Yes, with UE capability if two RF chains are in use to support NCSG+NCSG, we are fine to introduce new UE capability - Option 3: - Postpone the discussion on new capability for NCSG + NCSG until RAN4 has a consensus on parallel measurement.

The similar issue has also been discussed for case 1 and it was agreed that it is up to UE capability to support the simultaneous activation/deactivation of two Pre-MGs in the same FR. This means the case of Pre-MG + Pre-MG is supported without UE capability, but UE can optionally report the support of simultaneous (de)activation. We think the same conclusions can be used for case 2, i.e. the case NCSG + NCSG is supported but it is up to UE capability to support the parallel measurement within two NCSGs.

During the discussion, some companies mentioned that UE cannot guarantee the parallel measurement when single spare chain is assumed or the resources are overlapped within the two NCSGs. But for NCSG + NCSG case, it is possible to use different RF chains and there are already VIL requirements and the additional 4ms margin in collision definition to accommodate the preparation time for measurements, it should be possible to perform the parallel measurements when the resources are not physically overlapped. Based on the discussion, the UE capability is already a compromise solution for this issue and it should not preclude the advanced implementation. And we think this capability is agnostic to the use case of NCSG.

For the support of NCSG + NCSG case, we understand no additional UE capability is needed if UE has supported NCSG and concurrent MGs. And there is no need to have further limitation on NCSG patterns or RF chains.

Proposal 1: It is up to UE capability to support the parallel measurements upon NCSGs collision.

Proposal 2: No need to distinguish the scenarios for the capability of parallel measurements.

Proposal 3: No additional UE capability is needed for support of NCSG + NCSG case.

Potential changes for NCSG upon SCell activation:

- Issue 4-4-1: [Case 2] Potential changes for NCSG upon SCell activation < Agreements from GTW >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped.

For the UE behavior for deactivated SCell, since the association between gaps (including Pre-MG and NCSG) and MOs is configured by network, if the SCell is needed to be measured within MG after activation, it should be reconfigured to associate with the MG. So there is no ambiguity issue when the SCell is activated.

When the SCell is deactivated, if the MO is not associated with NCSG, we understand the legacy UE behavior should be followed i.e. UE measures the deactivated SCell outside MG. But after further check Rel-17 specification, we understand if NCSG is provided, the measurement can be performed within NCSG when all or parts of SMTC occasions are overlapped with NCSG. And the measurement requirements in 9.2.5 (UE measurement deactivated SCell outside gap) apply when the SMTC is fully non-overlapped with NCSG. So we understand option 2 is aligned with Rel-17 requirements and can also apply to case 2. Considering the interruption can be reduced with option 2, to move forward, we can compromise to option 2.

Proposal 4: For the UE behavior for deactivated SCell measurements with NCSG in Case 2, prefer option 1 to follow legacy behavior but can compromise to option 2 to move forward.

#### 9.1.5.3 Monitoring of multiple layers within NCSG The measurement requirements derived from CSSFwithin_ncsg,i defined in this clause are applicable provided that network provides NCSG pattern for measurement. The carrier-specific scaling factor CSSFwithin_ncsg,i for a measurement object i derived in this clause is applied to following measurement types: - - SSB-based intra-frequency measurement object without measurement gap as defined in clause 9.2.1 corresponding to an activated serving cell, when all of the SMTC occasions of this intra-frequency measurement object are overlapped by the NCSG; - - SSB-based intra-frequency measurement object with NCSG as defined in clause 9.2.1 corresponding to an activated serving cell (in non-dormancy); - - SSB-based intra-frequency measurement object corresponding to a deactivated serving cell or to an activated serving cell in dormancy, when all or part of the SMTC occasions of this intra-frequency measurement object are overlapped by the NCSG; - - SSB-based inter-frequency measurement object without measurement gap as defined in clause 9.3.1, when all of the SMTC occasions of this inter-frequency measurement object are overlapped by the NCSG; - - SSB-based inter-frequency measurement object with NCSG as defined in clause 9.3.1; - - E-UTRA inter-RAT measurement object, when the measurement can be performed with no measurement gap but NCSG as defined in clause [TBD]; - UE is expected to conduct the measurement of this measurement object i only within the NCSG. If the higher layer signaling in TS 38.331 [2] of smtc2 is present for an intra-frequency measurement object, and smtc1 is fully overlapping with NCSG and smtc2 is partially overlapping with NCSG, requirements derived from CSSFwithin_ncsg,i and CSSFoutside_gap,i are not applicable. Table 9.2.7.2-4: Measurement period for intra-frequency measurements with NCSG (deactivated SCell) (FR1) DRX cycle; T SSB_measurement_period_intra. No DRX; 5 x max(measCycleSCell, VIRP) x CSSFintra. DRX cycle≤ 320ms; 5 x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; 5 x max(measCycleSCell, VIRP, DRX cycle) x CSSFintra. Table 9.2.7.2-5: Measurement period for intra-frequency measurements with NCSG (deactivated SCell) (FR2) DRX cycle; T SSB_measurement_period_intra. No DRX; Mmeas_period with_gaps x max(measCycleSCell, VIRP) x CSSFintra. DRX cycle≤ 320ms; Mmeas_period with_gaps x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Mmeas_period with_gaps x max(measCycleSCell, VIRP, DRX cycle) x CSSFintra. Note: Requirements for measurement on deactivated SCC in this clause do not apply if SMTC on the deactivated SCC is fully non-overlapped with NCSG, and the requirements for measurement on deactivated SCC specified in clause 9.2.5 apply.

3 Summary

In this paper, we have some further discussions on the case 2 requirements for combination of NCSG and concurrent MG, and the following observations and proposals are given:

Proposal 1: It is up to UE capability to support the parallel measurements upon NCSGs collision.

Proposal 2: No need to distinguish the scenarios for the capability of parallel measurements.

Proposal 3: No additional UE capability is needed for support of NCSG + NCSG case.

Proposal 4: For the UE behavior for deactivated SCell measurements with NCSG in Case 2, prefer option 1 to follow legacy behavior but can compromise to option 2 to move forward.

4 References

- R4-2306330, WF on Rel-18 NR MG enhancements (part 1), MTK, RAN4#106bis-e

- R4-2306237, Topic summary for [106-bis-e][206] NR_MG_enh2_part1, MTK, RAN4#106bis-e