3GPP TSG-RAN4 Meeting #107	        R4-2307460 Incheon, South Korea, 22nd – 26th April 2023

Source: 	Qualcomm Inc.

Title: 	Low-power wake-up receiver RF aspects

Agenda Item:	8.21.2

Document for:	Approval

# Introduction

Study item description for low-power Wake-up Signal and Receiver for NR can be found in [1]. In this contribution we discuss the UE RF aspects and further response to the LS from RAN1 [2].

# Discussion

The study item includes the following objectives:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1]

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

- Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals  [RAN2, RAN1]

- Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1]

  - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

Study item objectives from [1] are reproduced below.

It can be seen from the objectives that RAN4 efforts are expected on the wake-up receiver architecture study as well as on wake-up signal design evaluation. From the LS [2] it can be seen that RAN1 has been focusing mainly on three different RF architecture options, RF envelope detection, heterodyne with IF envelope detection and zero-IF with baseband envelope detection. As almost all aspects on the wake-up signal are still open in RAN1, this contribution focuses on the receiver architectures, and discusses trade-offs and dependencies between the receiver architectures and wake-up signal design. Furthermore, we suggest what to reply to RAN1 in addition to the LS sent from RAN4#106 [3].

Furthermore RAN1 has send another LS to RAN4 in [7]. From the LS it can be seen that RAN1 is working also on architectures supporting FSK and OFDMA.

Observation 1: Based on the most recent LS RAN1 is looking also into RF architectures support FSK and OFDMA and not only analog envelope detection.

## Selectivity considerations for different RF architectures

Many of the questions in the RAN1 LS focus on how different RF architectures can manage with in-channel and adjacent channel interference, and we have analyzed this for each architecture option. First we discuss the general principles of how adjacent subcarrier and adjacent channel interference effects the time domain detection and then discuss ways to provide selectivity for each architecture option.

Firstly, as WUR will be implemented together with the main receiver, it can be placed in the same physical locations as main receiver. Therefore, the signal environment and power levels at adjacent channel are the same as for main receiver when observed at antenna connector. Therefore, WUR will need to operate under similar interference as main receiver.

Observation 2: RF signal levels observed at antenna connector are similar for wake-up receiver and main receiver.

For RF envelope detection majority of the selectivity comes for the RF filter. We expect that for all RF architectures, it would be reasonable to re-use the same RF filter as for main receiver and just route the signal either to main or wake-up receiver. Therefore, RF filter will be allowing the signals in the whole operating band pass through to the envelope detector.

Envelope detector will include low-pass filtering for smoothing the envelope. Additional selectivity provided after the envelope detector will not provide much help as highest input is driving the envelope and will be the main component in the detector output. Further filtering will rather smoothen out the wanted signal which in the time domain signal is rather only a tiny ripple in the interferer waveform.

We expect that the envelope detector would be tuned to the specific operating band via proper component sizing, which will result in some frequency selectivity, however, this will not make a meaningful difference. Secondly, having components sized for the operating band results in poor capabilities to support multiple operating bands, especially ones with large frequency difference.

Finally, RF envelope detection cannot address the cases where WUS is based on FSK or OFDMA.

Proposal 1: Remove RF envelope detector from RAN4 study scope and inform the decision to RAN1

For IF envelope detection, most selectivity would come from an IF-filter, which is traditionally an acoustic filter using e.g. SAW technology. As the heterodyne architecture has an LO, one IF-filter is sufficient and the architecture can support multiple operating bands.

IF-filter characteristics are important to understand. Firstly, IF filter can provide good selectivity, e.g. 20-30 dB, with just couple MHz transition bandwidth. Also any loss present in IF-filter is less important as amplification can be provided before the signal enters the IF-filter. Therefore, from pure filtering performance perspective IF-filter could be a viable solution to suppress adjacent channels and even adjacent subcarriers with some distance away from WUS.

This filtering performance will depend a lot whether WUS location is flexible within carrier bandwidth and how quickly the placement can vary. In principle, tuning the LO correctly can address the flexibility in WUS position. However, if the WUS location is constantly changing, then LO would also need to be constantly re-tuned, adding complexity to the control of the LO and tracking where WUS will be transmitted. In case IF-filter passband is made wider to allow WUS location flexibility, then selectivity against adjacent subcarriers or even adjacent channels may suffer.

Observation 2: While IF-filter can provide good selectivity against adjacent channels and even in-channel subcarriers which are not immediately adjacent to WUS, the selectivity may suffer if WUS location is flexible.

Furthermore, there are other costs associated with the IF-filter. As selection of IF-frequency is up to the device manufacturer there is no real IF-filter ecosystem in place like there is for RF filters. This means that IF-filters tend to more custom design and smaller batch products, increasing the unit cost. Furthermore, acoustic filter size will scale with the wavelength. RF size can be less than 1 square-mm, IF filters sizes can be e.g. 25 square-mm. This means that providing a well-integrated small solution may not be feasible.

Observation 3: IF-filter size and cost and their impact to practicality of the WUR design may be prohibitive aspects and need to be considered in IF envelope detection feasibility.

Next let’s look into the architectures where detection happens at baseband, either in digital domain or by envelope detector. In RAN4#106 we presented results showing that RF design can be relaxed by narrowband WUS design and limiting the WUS placement in the middle of the channel [6].

In RAN4#106bis-e evaluation scenarios were agreed in captured in the WF [8]. They are reprocuded below in Table 1

Table 1: LP-WUS evaluation scenarios

| NR RF channel BW | 5MHz for 1.4MHz WUS; 20MHz/100MHz for 5MHz WUS |
| --- | --- |
| Guardband of NR channel | Unchanged, defined in Clause 5.3.3 in TS 38.101-1 |
| WUS BW within NR channel | 1.44MHz, 5.04 MHz |
| WUS RB allocation (Note 1) | [6] RB in 1.44 MHz, total 8 RBs, or other number of RBs [24] RB in 5.04 MHz, total 28 RBs, or other number of RBs |
| WUS placement within NR channel | 3 cases: - case 1: Center; - case 2: edge; - case 3: between center and edge of NR channel, 5 MHz offset from channel edge used |
| Guard RB size of LP-WUS | - 0 RB, 1RB at each side, 2RBs at each side, or other number of RBs. - Asymmetric guard RB can also be considered |
| ACS interferer | According to RF CBW |
| Filter characteristic | 2nd to 5th order Butterworth Both analog and digital filter can be considered |
| Filter passband BW | At least WUS bandwidth (number of RBs), depends on guard RB size |
| LO frequency | Case 1: In the middle of WUS (modeling fixed WUS position) Case 2: In the middle of RF channel (modeling flexible WUS location) |
| Target ACS | TBD |
| Target ASCS | TBD |
| Target WUS SNR | TBD |
| RF impairment | FFS |
| Note 1: the maximum number of allocated WUS RBs, depends on how many Guard RBs are needed. 5MHz WUS within 5MHz NR CBW is not considered currently. |  |

The evaluation results based have been provided in Table 2. In the analysis adjacent channel bandwidth was 5 MHz. The analysis for received SINR was done with following assumptions.

- Input signal power = reference level = 0 dB

- Input signal SNR = 10 dB

- ACS signal power = 24.5 dB

- Input signal BW = 106 RB

- WUS BW = 24 RB

- ACS BW = 25 RB

- Bandwidth used for noise = input signal BW

- Guardband = 0 RB or 2 RB at either side of WUS

- Received SINR = received WUS power / (received other RB power + received noise power + received ACS power)

A one numerical example for the SINR calculations is provided next. In this example we have used the results for case #1 in Table 2, and used values for 5th order filter.

Firstly, received WUS power can be calculated from total received input signal power and the bandwidth of WUS compared to the input signal power:

Received WUS power = 0 dB -10*log10( #RBs / #WUS_RB) = 0 dB – 10*log10(106/24) = - 6.45 dB;

In addition to WUS signal, other RBs carry unwanted power to the receiver. The number of other RBs is equal to total number of RBs reduced by number of WUS RBs and guard band RBs. As such, the power level before any filtering can be easily calculated from bandwidth of the other RBs. After that, analog baseband filter will filter some of this power away. We have calculated the average filter rejection across the non-WUS RBs and attenuated the power by this amount.

Received other RB power after filtering = 0 dB - 10*log10( #RBs / (#RB –(#WUS_RB + #GB_RB))) – filtering = 0 dB – 10*log10(106 / (106 – (24+0*2) - 10 = 0 – 10*log10(106/82) -13 = -14.11

For noise power, we have used a simplification where noise power is present only within the received carrier. Some of the noise will come through unfiltered, as RBs carrying WUS cannot be subjected to filtering. The noise power at non-WUS RBs have been attenuated by the same filtering as the non-WUS RBs.

Received noise power = 0 dB + 10*log10(10^(-SNR/10)*#WUS_RB/#RB + 10^ ((-SNR - filtering)/10)*(#RB-#WUS_RB-#GB_RB)/#RB  = 0 dB + 10*log10( 10^(-10/10)*24/106 +  10^((-10-13)/10)*(106-24-0*2)/106)  = -15.76 dB

Average filter attenuation over adjacent channel has been calculated and adjacent channel power has been attenuated by that amount.

Received ACS power = 24.5 dB – avg filter suppression over adjacent channel = 24.5 dB – 63.5 = -39dB

Finally the received SINR was been calculated as ratio between the received WUS power and the unwanted power components of other RBs, noise power and ACS power.

Received SINR = 10*log10 ( 10^(-6.45/10) / ( 10^(-14.11/10) + 10^(-15.76/10) + 10^(-39/10) ) ) = 5.39 dB

While the model is very simple, it gives a good estimation on levels of different signal and noise components at the analog baseband filter output. These levels can be used to evaluate whether analog envelope detection based receiver can work, i.e. whether the analog signal SINR is good enough. Secondly, the model can be used to evaluate the ADC complexity for any digital detection based architecture, as those architectures need to have sufficient ADC dynamic range to digitize both the wanted and unwanted signal components to be able to further process those in digital domain.

Table 2: Evaluation results. SINR reported after baseband filter.

| # | RF channel BW (MHz) | WUS RB allocation | Additional guard band RBs on each side of WUS | WUS placement | LO position | Avg. filter suppression over other RBs than WUS + GB |  | Avg. filter suppression over adjacent channel / filter order |  | Received SINR / filter order |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 3 | 5 | 3 | 5 | 3 | 5 |
| 1 | 20 | 24 | 0 | Case 1 | Case 1 | 7.5 | 13 | 33.8 | 63.5 | -1.1 | 5.4 |
| 2 | 20 | 24 | 0 | Case 3 | Case 1 | 12.4 | 18.3 | 19.2 | 38.1 | -11.8 | 4.6 |
| 3 | 20 | 24 | 0 | Case 2 | Case 1 | 14.3 | 20.6 | 8.4 | 15.8 | -22.6 | -15.2 |
| 4 | 20 | 24 | 2 | Case 1 | Case 1 | 8.4 | 15.2 | 33.8 | 63.5 | -0.6 | 6.8 |
| 5 | 20 | 24 | 2 | Case 3 | Case 1 | 13.4 | 21.0 | 19.5 | 38.7 | -11.5 | 5.3 |
| 6 | 20 | 24 | 2 | Case 2 | Case 1 | 15.2 | 23.0 | 9.9 | 19.3 | -21.1 | -11.7 |
| 7 | 20 | 24 | 0 | Case 1 | Case 2 | 10 | 15.9 | 34.0 | 63.9 | 0.1 | 7.1 |
| 8 | 20 | 24 | 0 | Case 3 | Case 2 | 0.3 | 0.4 | 4.6 | 11.6 | -26.4 | -19.5 |
| 9 | 20 | 24 | 0 | Case 2 | Case 2 | 0 | 0 | 1.3 | 2.9 | -29.7 | -28.1 |
| 10 | 20 | 24 | 2 | Case 1 | Case 2 | 10.9 | 18.3 | 34.0 | 63.9 | 0.5 | 8.1 |
| 11 | 20 | 24 | 2 | Case 3 | Case 2 | 0.3 | 0.4 | 4.6 | 11.6 | -26.4 | -19.5 |
| 12 | 20 | 24 | 2 | Case 2 | Case 2 | 0.0 | 0.0 | 1.3 | 2.9 | -29.7 | -28.1 |

Firstly, it is important to note that the results in Table 2 are based on theoretical modeling and do not take into account neither required implementation margins nor even feasibility of high-order narrow passband filters. Therefore, the general trends in the data should be considered rather than exact values.

From the results in can be seen that SINR after baseband filter is very poor in case WUS is placed at RF channel edge closest to the ACS interferer. This scenario corresponds to WUS placement case 1. Due to the narrow frequency offset the analog filter cannot provide sufficient rejection to the ACS interferer. For this case analog envelope detection is unlikely to work correctly.

When WUS is 2.5 MHz offset from channel edge, SINR can be recovered by using a higher order filter. This corresponds to WUS placement case 3. Reasonable SINR can be obtained with different filter orders when WUS is placed in the middle of the channel.

The impact of the poor SINR is that analog envelope detection based receivers are unlikely to work sufficiently well if WUS is placed immediately adjacent to channel edge. Receivers utilizing further digital filtering have further possibilities to improve the SINR, but the ADC needs to have sufficient dynamic range to take the interference in.

Observation 4: Analog envelope detection based receiver is unlikely perform acceptably if WUS is placed at channel edge.

It can be seen from the data that filter rejection is clearly worse whenever flexibility in WUS location is supported, i.e. with Case 2 LO position. This is expected as wider filter passband is required for case 2. To enable Case 1 either WUS position should be fixed, or the WUS position should be known well ahead of time allowing re-tuning LO frequency to WUS frequency. The principle of the different LO placements is shown in Figure 1.

Figure 1: When LO is tuned to WUS center frequency narrower filter BW is possible and rejection is better

Observation 5: When LO can be placed in the middle of WUS allocation, interferers are easier to reject. This can be enabled by fixed WUS frequency location or scheduling timeline which allows time for LO re-tuning.

It can be seen that if full WUS flexibility, which would not allow for LO re-tuning and WUS frequency position can cover full carrier, the rejection towards interferers is very poor and remains poor even with higher order baseband filters. This would mean that more complex, higher dynamic ADC would be needed and majority of the filtering done in digital domain. This reduces possibilities for power savings in RF domain.

Observation 6: If LO re-tuning is not possible and WUS can be scheduled at carrier edge, analog baseband filtering provides little help towards interferers and therefore possibilities for RF power savings are reduced.

Based on the provided data and the observations made from it, we suggest to inform RAN1 that analog envelope detection architectures will perform poorly or not work at all under adjacent channel interference if WUS is placed immediately adjacent to channel edge. To improve the likelihood of operation multiple MHz offset is needed, but exact value needs further study. For architectures using digital detection, placing WUS away from channel edge can enable use of simpler RF HW and power savings.

Proposal 1: Inform RAN1 that analog envelope detection architectures will perform poorly or not work at all under adjacent channel interference if WUS is placed immediately adjacent to channel edge. To improve the likelihood of operation multiple MHz offset is needed, but exact value needs further study. For architectures using digital detection, placing WUS away from channel edge can enable use of simpler RF HW and power savings.

Impact of guard RBs can be also observed. Two guard RBs on either side of the WUS signal have very limited impact to the SINR after baseband filter. This means that for analog envelope detection based receiver the providing a guard band does not provide meaningful benefits. From digital filtering perspective a guard band can relax the filtering requirements, but the savings in receiver power consumption need further evaluations.

Impact of guard band was evaluated also with simulations to see how waveform impacts the need for guard band. The results captured in [9] indicate the OOK waveform is not sensitive to guard band. On the other hand, FSK performance improves with guard band. This means that FSK-waveform can benefit from guard band or more stringent (digital) filtering to more effectively suppress nose from adjacent subcarriers.

Observation 7: In the analysed scenarios providing a two RB guard band on either side of the WUS does not provide a meaningful difference in SINR after analog baseband filter, and especially OOK waveform is not sensitive on guard band.

## Noise Figure and Power consumption considerations

Generally noise figure and power consumption are competing specifications – it is very challenging to achieve both low noise figure and low power consumption at the same time and power consumption typically increases when sensitivity is improved. Some trends based on scientific publications can be found e.g. in [4] and [5] with Figure 5 below being reproduced from [4].

Figure 5: Power consumption vs. sensitivity [4]

While this work targets towards low power receiver, the noise figure cannot be allowed increase without boundary, as that can start to negatively impact performance at cell edge.

Observation 8: Low power consumption needs to be balanced with negative impacts to performance.

Overall selection of the NF is a multi-dimensional issue, which depends also on what is the target SNR where the wake-up signal will be designed to be detectable. If the target SNR is low enough, NF can be higher and power consumption lower without negative impacts to the system operation.

The typical NF assumption used in RAN4 is 9 dB noise figure, together with the reference sensitivity signal being detectable at approximately -1 dB SNR. This is based on 2Rx UE. Instead of debating noise figures in RAN4, we should guide RAN1 to design the wake-up signal taking advantage of the specified reference point and considering power consumption in the targe SNR.

Once RAN1 provides more information on wake-up signal RAN4 can then agree on applicable noise figure which enables the system to work as intended. Finally, it should be noted that such low noise figures as specified for typical NR UE cannot be reached with RF envelope detection.

Proposal 2: Inform RAN1 that required NF can be concluded based on coverage target, which is expected to full coverage of the cell, and SNR where wake-up signal can be successfully detected. For reference, 9 dB NF and -1 dB SNR is used for typical NR UE in reference sensitivity test case, but typical NR UE also has 2 receivers. RAN1 should take into account in wake-up signal design that lower SNR will enable higher NF and therefore also lower power consumption. 9 dB noise figure would not be possible to reach at least with RF envelope detection.

# Conclusions

In this contribution RF aspects for low-power wake-up receiver were discussed. Following observations and proposals were made:

Observation 1: Based on the most recent LS RAN1 is looking also into RF architectures support FSK and OFDMA and not only analog envelope detection.

Observation 2: While IF-filter can provide good selectivity against adjacent channels and even in-channel subcarriers which are not immediately adjacent to WUS, the selectivity may suffer if WUS location is flexible.

Observation 3: IF-filter size and cost and their impact to practicality of the WUR design may be prohibitive aspects and need to be considered in IF envelope detection feasibility.

Observation 4: Analog envelope detection based receiver is unlikely perform acceptably if WUS is placed at channel edge.

Observation 5: When LO can be placed in the middle of WUS allocation, interferers are easier to reject. This can be enabled by fixed WUS frequency location or scheduling timeline which allows time for LO re-tuning.

Observation 6: If LO re-tuning is not possible and WUS can be scheduled at carrier edge, analog baseband filtering provides little help towards interferers and therefore possibilities for RF power savings are reduced

Observation 7: In the analysed scenarios providing a two RB guard band on either side of the WUS does not provide a meaningful difference in SINR after analog baseband filter.

Observation 8: Low power consumption needs to be balanced with negative impacts to performance.

Proposal 1: Inform RAN1 that analog envelope detection architectures will perform poorly or not work at all under adjacent channel interference if WUS is placed immediately adjacent to channel edge. To improve the likelihood of operation multiple MHz offset is needed, but exact value needs further study. For architectures using digital detection, placing WUS away from channel edge can enable use of simpler RF HW and power savings.

Proposal 2: Inform RAN1 that required NF can be concluded based on coverage target, which is expected to full coverage of the cell, and SNR where wake-up signal can be successfully detected. For reference, 9 dB NF and -1 dB SNR is used for typical NR UE in reference sensitivity test case, but typical NR UE also has 2 receivers. RAN1 should take into account in wake-up signal design that lower SNR will enable higher NF and therefore also lower power consumption. 9 dB noise figure would not be possible to reach at least with RF envelope detection.

# References

[1]	RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, Vivo

[2]	R1-2212999, LS to RAN4 on low-power wake-up receiver architectures, RAN1

[3]	R4-2303712, Reply LS to RAN1 on LP-WUR architectures

[4]	Power Consumption of Integrated Low-Power Receivers, IEEE Journal on emerging and selected topics in circuits and systems, Vol 4, No. 3, September 2014, Emil Nilsson and Christer Svensson

[5]	David D. Wentzloff, “Low Power Radio Survey,” [Online]. www.eecs.umich.edu/wics/low_power_radio_survey.html

[6]	R4-2302276 ,Low-power wake-up receiver RF aspects, Qualcomm Inc,

[7]	R1-2304251, Reply LS to RAN4 on LP WUR architectures, RAN1

[8]	R4-2306617, WF on LP-WUS, Vivo

[9]	R1-2305360, L1 Signal design and procedures for LP-WUR, Qualcomm Incorporated