TSG-RAN Working Group 4 (Radio) meeting #107	R4-2307703

Incheon, Korea, 22nd to 26th May 2023

Source:	Ericsson

Title:	Further considerations related to coexistence simulation assumptions

Agenda item:	8.20.2.1

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#106-bis-E) simulation assumptions for SBFD adjacent channel coexistence evaluation were extensively discussed. The outcome was captured in a way-forward contribution [1] and an overview of all assumptions was collected in [2].

Simulation assumptions such as models and corresponding parameters are essential for the coexistence evaluation. They will have direct impact on the result used to draw conclusions on coexistence impact when SBFD is introduced. As part of technical background relevant for this study an Annex have been created in TR 38.858. The intention is to gather relevant information with respect to coexistence simulation assumptions. To facilitate the inclusion of the information in TR 38.858 a text proposal has been created in a companion contribution [3]. During the construction of this text proposal several technical issues and in few cases, errors have been identified. In this contribution we highlight the issues and propose solutions to fix them.

# Discussion

In the following sections technical issues related to the adjacent channel coexistence evaluation have been identified.

  - ACLR/ACS power scaling

In RAN4, ACLR and ACS are defined assuming the wanted carrier bandwidth and adjacent channel bandwidth to be equal. This means that the power in the adjacent channel can be easily associated to the wanted carrier power using ACLR and similar for ACS when receiver performance is evaluated. Fundamentally, the ACLR and ACS definition in RAN4 assumes that wanted signal bandwidth is equal to adjacent channel bandwidth.

However, for the case of a SBFD carrier where the bandwidth of the transmission slot (DL) may have different bandwidth than the reception slot (UL) a concept of extracting the interfering power level from the transmitter falling within the reception slot has been established. For simplicity, it would be easy to assume that emission PSD have a flat characteristic over frequency. However, that’s not how adjacent channel emission typically behave. The emission in adjacent channel is dominated by emissions due to intermodulation products. Intermodulation products typically have a non-flat characteristic over frequency several carrier bandwidths from the wanted carrier.

For the case of simplicity, an approach of assuming flat emission has been adopted for SBFD coexistence simulations. For future work in RAN4 where asymmetric interference cases emerge, RAN4 needs to develop more accurate models.

For a case where the emission and selectivity are assumed to have a flat characteristic the interference power level can be calculated by applying a bandwidth scaling.

Analysing the power scaling effects with relation to ACLR assuming flat spectral power density for wanted signal and adjacent channel emission we can look at the general definition of ACLR. In general terms ACLR can be defined as a ratio between wanted signal power spectral density (PD,tx) and adjacent channel power spectral density (PD,rx) in linear scale as:

ACLR=PD,txPD,rx=PtxPrxBrxBtx

, where Ptx is the wanted signal power in W, Btx in the wanted signal bandwidth in Hz, Prx is the emission power level in W and Brx is the victim bandwidth in Hz. It can notice that for the special case when Btx=Brx, the ACLR definition reduce to a ratio between two power levels. This definition is commonly used in RAN4 coexistence evaluations. However, for SBFD coexistence evaluation it is essential to also capture situations where the aggressor bandwidth is different to the victim bandwidth.

The power level at the victim Prx can be calculated in logarithmical scale in dBm as:

Prx=Ptx-ACLR-10logBtxBrx

At previous RAN4 it was agreed in [1] to adopt the following methodology for calculating the interference power level for two different scenarios. For a given aggressor power level (Pa) and ACLR value, the interference power level can be calculated as:

- For the case when Ba<Bv, e.g., the case for SBFD interfering with TDD, the power at the victim is calculated as: Pv=Pa-ACLR

- For the case when Ba>Bv, e.g., the case for legacy TDD to SBFD interference, the power at the victim is calculated as: Pv=Pa-ACLR-10logBaBv

In our view we think that power scaling should be applied for both cases to extract correct power level assuming flat emission if bandwidths are non-equal. With the suggested approach for calculation of interference power for case 1, the calculated power level will be too low. Also, we know that emission will roll-off, it will not just be zero. This assumption would go against the fundamental assumption of flat emission outside first adjacent channel. It would be better to have a consistent model for adjacent channel emission that can be adopted for all cases where scaling is required.

In RAN1 power scaling is adopted when interference power levels are derived. Since RAN4 have not defined a model for adjacent channel emission and instead adopted flat characteristics it would be good to align with RAN1 assumptions.

Proposal 1: Apply carrier configuration power scaling with bandwidth for all interference situations when ACLR and ACS is used, to calculate interferer levels consistently.

  - BS ACS levels

Minimum conducted requirement is defined at the antenna connector for BS type 1-C and at the TAB connector for BS type 1-H as listed in Table 2.2-1.

Table 2.2-1: Base station ACS requirement

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) |
| --- | --- | --- |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | PREFSENS + 6 dB | Wide Area BS: -52 Medium Range BS: -47 Local Area BS: -44 |

It can be noticed that the interference signal mean power is differentiated rather than the ACS ratio. This means that the ACS level should be kept constant (46 dBc) for all BS classes for simulation purposes. The specification is created based on an interferer signal that will change over BS classes due to different noise figure assumption.

Proposal 2: For FR1 BS set ACS to 46 dBc for all BS classes.

  - UE transmit power control model

Traditionally, RAN4 models the required UE transmit power from a link budget calculation between the BS receiver and UE transmitter. The coupling loss is ban be expressed as:

CLx-ile=-SNRtarget+UEmax,eirp-N0-NF-10logB

, where SNRtarget is the BS receiver target SNR in dB, UEmax,eirp is the UE power in dBm, N0 is the BS noise floor in dB, NF is the BS noise figure and B is the carrier bandwidth in MHz.

In the discussion related to SBFD two option on how to handle UE power control was identified.

- To make sure target SINR is met, update power control scheme considering raising noise floor due to co-site co-sector and co-site inter-sector BS-to-BS interference.

- Use the legacy power control scheme.

From last meeting way-forward contribution [1] the following agreement can be found:

Recommended WF:

using option 2 as baseline. Add following statement into final TR.

In simulation, power control scheme is only used to compensate path loss. That’s the reason why final SINR for UL is less than assumed target SINR. In commercial networks UE UL SINR could meet target value according to power control scheme in TS 38.213.

For the case of SBFD operation, the BS total interference will be higher than before, which would lead to higher UE output transmission power to be used in the network. This would lead to further study deployment scenarios where UE-to-UE interference may impact coexistence.

Proposal 3: RAN4 needs to increase priority for deployment cases where UE-to-UE interference impact coexistence.

  - UE ACLR model

Unlike previous coexistence evaluations done by RAN4 a model for UE ACLR has been adopted for the SBFD coexistence evaluation.

Instead of assuming a fixed ACLR value, the ACLR will be a function of UE transmitter power as follows:

- FR1: UE ACLR is modelled as 30 dB at UE maximum TX power, and improves 1 dB per 1 dB backoff TX power up to a maximum 10 dB improvement. i.e., at 10 dB power backoff the ACLR is 40 dB.

- FR2-1: UE ACLR is modelled as 24 dB at UE maximum TX power, and improves 1 dB per 1 dB backoff TX power up to a maximum 10 dB improvement. i.e., at 10 dB power backoff the ACLR is 34 dB.

This model will include the aspects related to that ACLR tend to improve for power levels less than maximum output power. However, there have been no technical evidence that shows that ACLR will improve 10 dB for all types of PA implementations.

It can be noticed that this model will add significant complexities to the simulator and it is not clear how it will affect the simulation results.

Also, it is not really clear how this model will work out together with the ACIR sweeping required for the result template discussed in RAN4. It remains unclear what to do if ACLR is different for each UE and variable.

Proposal 4: To confirm validity of the UE ACLR model, encourage UE vendors to capture some technical background information in TR 38.858 to justify back-off range of 10 dB.

# Conclusion

In this contribution we have identified a few technical issues and lack of technical background information. To progress the work, we propose following:

Proposal 1: Apply carrier configuration power scaling with bandwidth for all interference situations when ACLR and ACS is used to calculate interferer levels consistently.

Proposal 2: For FR1 BS set ACS to 46 dBc for all BS classes.

Proposal 3: RAN4 needs to increase priority for deployment cases where UE-to-UE interference impact coexistence.

Proposal 4: To confirm validity of the UE ACLR model, encourage UE vendors to capture some technical background information in TR 38.858 to justify back-off range of 10 dB.

# References

[1]	R4-2305921, “WF for SBFD adjacent channel co-existence study”, CMCC

[2] 	R4-2305923, “Summary of co-existence simulation parameters”, CMCC, CableLabs, Samsung

[3]	R4-2307701, “TP to TR 38.858: Addition of coexistence simulation assumptions to Annex D”, Ericsson