3GPP TSG-RAN WG4 Meeting # 107		R4-2307955

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.8.3.4

Source: 	Xiaomi

Title: 	Discussion on the scheduling measurement restriction

Document for:	Discussion

- Introduction

During the RAN4#106-bis-e meeting, the scheduling restriction and measurement restriction has been discussed with no conclusion reached yet. The WF [1] was agreed to capture some conclusion. In this paper, we try to give further analysis on this topic.

- Discussion

During the RAN4#106-bis-e meeting, it has been agreed in the WF [1] that some high level discussion will be made first and the agreement is captured as below:

### Issue 3-1-1: Conditions/cases that scheduling restriction for L1 measurements can be relaxed for multi-Rx

<Way forward >:

Company is encouraged to bring analysis on high-level principle in the next meeting.

In our companion discussion paper for L1-RSRP measurement with GBBR, the mechanism of groupbasedbeamreporting-r17 has been discussed and some observations and proposals can be re-used here for measurement and scheduling restrictions. The table 1 of the L1-RSRP reporting is captured again here for further discussion.

Table 1 Illustration of the L1-RSRP report of groupbasedbeamreporting-r17

| CMR set ID with largest L1-RSRP | group ID | CMR resource ID in group#0 | CMR resource ID in group#1 |
| --- | --- | --- | --- |
| CMR set ID （1bit） | group ID#0 | SSBRI/CRI#， L1-RSRP（7bit） | SSBRI/CRI#， L1-RSRP（4bit） |
|  | group ID#1 | SSBRI/CRI#， L1-RSRP（4bit） | SSBRI/CRI#， L1-RSRP（4bit） |
|  | group ID#2 | SSBRI/CRI#， L1-RSRP（4bit） | SSBRI/CRI#， L1-RSRP（4bit） |
|  | group ID#3 | SSBRI/CRI#， L1-RSRP（4bit） | SSBRI/CRI#， L1-RSRP（4bit） |

This table 1 has already indicateed that for each L1-RSRP measurement of the groupbasedbeamreporting-r17, the UE is required to measure the SSB or CSI-RS based reference signal simultaneously and corresponding L1-RSRP measurement result for the simultaneous measurement will be reported. In this case, the measurement restriction should not apply.

Observation 1: The groupbasedbeamreporting-r17 report mechanism has required to measure the SSB or CSI-RS based reference signal simultaneously.

In this case, there has already beam pairs that UE can receive simultaneously with the reported SSB or CSI-RS indicator. To consider the scheduling restriction, the measurement of SSB or CSI-RS reference signals of a specific GBBR pair, if the active TCI state for such data reception is QCLed with the other SSB or CSI-RS reference signals of the same GBBR pair, then their should be no scheduling restriction.

Proposal 1: The measurement of SSB or CSI-RS reference signal of a specific GBBR pair, if the active TCI state for such data reception is the same or QCLed with the other SSB or CSI-RS reference signal of the same GBBR pair, then there should be no scheduling restriction.

For the L1 measurement restriction, similar principle apply. The measurement of SSB or CSI-RS reference signal of a specific GBBR pair, if the other measurement of SSB or CSI-RS reference signal is the same or QCLed with the other SSB or CSI-RS reference signal of the same GBBR pair, then there should be no scheduling restriction.

Proposal 2: The measurement of SSB or CSI-RS reference signal of a specific GBBR pair, if the other measurement of SSB or CSI-RS reference signal is the same or QCLed with the other SSB or CSI-RS reference signal of the same GBBR pair, then there should be no scheduling restriction.

For the scheduling restriction for L3 measurements, since currently the simultaneous reception is based GBBR and only L1-RSRP is considered in such case, the reported GBBR beam pair with L1-RSRP simultaneous reception are all fine beams which is hard to be used to consider the L3 measurement. In this case, it is suggested not to consider the scheduling restriction enhancement for L3 measurements based on GBBR.

Proposal 3: Not to consider the scheduling restriction enhancement for L3 measurements based on GBBR.

3	Conclusions

In this paper, we give further discussion on multi-RX chain issues, the observations and proposals are captured as below:

Observation 1: The groupbasedbeamreporting-r17 report mechanism has required to measure the SSB or CSI-RS based reference signal simultaneously.

Proposal 1: The measurement of SSB or CSI-RS reference signal of a specific GBBR pair, if the active TCI state for such data reception is the same or QCLed with the other SSB or CSI-RS reference signal of the same GBBR pair, then there should be no scheduling restriction.

Proposal 2: The measurement of SSB or CSI-RS reference signal of a specific GBBR pair, if the other measurement of SSB or CSI-RS reference signal is the same or QCLed with the other SSB or CSI-RS reference signal of the same GBBR pair, then there should be no scheduling restriction.

Proposal 3: Not to consider the scheduling restriction enhancement for L3 measurements based on GBBR.

4 References

[1] R4-2306318		WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1), VIVO