3GPP TSG-RAN WG4 Meeting # 107                                   R4-2308019

# Incheon, May 22 – May 26, 2023

Agenda item:	8.10.2.3

Source:	China Telecom

Title:	Discussion on Case 2 requirements for NCSG and concurrent MG

Document for:	Discussion

- Introduction

In RAN4 #106-bis-e meeting, RRM core requirements for pre-configured MGs, multiple concurrent MGs and NCSG were discussed further and the WF [1] was approved.

In this paper, we present our views about Case 2 RRM requirements for the combination of NCSG and concurrent MG issues captured in the WF [1].

- Discussion

Regarding Case 2 RRM requirements for the combination of NCSG and concurrent MG, there are issues about parallel measurements for NCSG + NCSG and others being discussed in this paper.

Parallel measurements for NCSG + NCSG

Status in the WF [1]:

- Issue 4-2-1: [Case 2] Whether to consider parallel measurements upon gap collision < Agreements from GTW >: - Option 1: Support of parallel measurements upon NCSGs collision is up to UE capability - Option 2: Do not support parallel measurements upon NCSGs collision

In our understanding, scenario with parallel measurement is not a common and typical scenario, however, the definition of parallel measurement is complicated and needs heavy workload, since we don’t have enough time to expand the discussion, we prefer not to consider parallel measurements upon NCSGs collision at least for this release. Besides, UE needs two spare RF chains to support parallel measurements, but UE cannot always guarantee there are two available RF chains being used for parallel measurements. Therefore, we think it’s not necessary to consider parallel measurements upon gap collision.

Proposal 1: It’s not necessary to consider parallel measurements upon gap collision.

Other Rel-17 rules to be revisited

Status in the WF [1]:

- Issue 4-4-1: [Case 2] Potential changes for NCSG upon SCell activation < Agreements from GTW >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped.

In our views, when the SCell is deactivated, the deactivated SCell’s MO being measured within NCSG can avoid interruption, it will be beneficial for performance. If there isn’t too much workload, the deactivated SCell’s MO being measured within NCSG can be introduced. However, if there is too much workload, we are fine with legacy UE behavior.

Proposal 2: When the SCell is deactivated, the deactivated SCell’s MO being measured within NCSG can avoid interruption, it will be beneficial for performance.

- Conclusion

The contribution presents our views about Case 2 RRM requirements for the combination of NCSG and concurrent MG, with the following proposals:

Proposal 1: It’s not necessary to consider parallel measurements upon gap collision.

Proposal 2: When the SCell is deactivated, the deactivated SCell’s MO being measured within NCSG can avoid interruption, it will be beneficial for performance.

- References

- R4-2306330, WF on Rel-18 NR MG enhancements, MediaTek inc., RAN WG4 Meeting #106-bis-e, April, 2023.