3GPP TSG-RAN WG4 Meeting # 107                                   R4-2308171

# Incheon, KR, May 22 – May 26, 2023

# Source: 	ZTE Corporation

# Title: 	Tx requirements for 3Tx for band combinations within two bands

# Agenda item:	7.29.3.1

# Document for:	Approval

# 1 Introduction

In last meeting, how to include the Tx requirements for 3Tx PC2/1.5 inter-band NR CA/ENDC in the spec were approved, which are summarized below:

1.General principle of introducing Tx Requirement for 3Tx CA/DC

      - For 3Tx inter-band UL CA+UL MIMO, reuse the single CC requirements and TxD or UL MIMO requirements for corresponding sessions.

      - For 3Tx inter-band UL ENDC+UL MIMO, reuse the single CC requirements and TxD or UL MIMO requirements in TS 36.101 and in TS 38.101-1 respective for corresponding sessions.

2. Introduce new sub-clause for inter-band ENDC + UL MIMO with clause suffix H for 38.101-3, and make it clear in the spec that this is for 3Tx UE.

3. Apply clause name 6.2/3/4/5H.3 for simultaneous 3Tx with inter-band UL CA Tx requirements.

4. 3Tx band combinations can be defined in MOP tables of clause with suffix H (inter-band CA+MIMO)

5. Existing ΔTIB,c/ΔRIB,c requirements could be applied, there is no need to specifically define new ΔTIB,c/ΔRIB,c for 3Tx.

6. no new capabilities introduced to indicate UE supporting 3Tx in 2bands.

7. 3Tx requirements are restricted to FWA in this release as in the WID

Meanwhile, there are still some open issues such as SAR/Pcmax, etc. In this contributions, we give some further discussions on the Tx RF requirements for 3Tx inter-band UL CA/ENDC.

# 2	Discussion

### 2.1 Configuration table

- First, we think the requested 3Tx combination in the WID should be included in the configuration tables in clause 5 in TS38.101-1 for NR CA and TS38.101-3 for ENDC. Although 3Tx band combinations can be defined in MOP tables of clause with suffix H, MOP tables are only for UL configurations without the DL combination information. By using the current basket WID approach, for example the configurations of UL_CA_n28A-n41_DL CA_n28A-n41A/B/C, and UL_CA_n28A-n41_DL CA_n28A-n41A-n78A/(2A) share the same UL MOP requirements of UL_CA_n28A-n41.

The configurations table aims to include the configuration band combination information for NR CA(also include BCS information) and ENDC including DL and UL configurations. The PC3 and PC2 band combinations configurations were already included in the common tables under clause 5.5A.3 in TS38.101-1. Therefore, we think the concurrent 3Tx inter-band NR CA/ENDC combination can be included in the existing configuration tables with specific note to identify the band combination with 1Tx in one band, and 2Tx in the other band.

Proposal 1. Configuration for concurrent 3Tx inter-band NR CA/ENDC can be included in the existing configuration tables (i.e. tables in clause 5.5A in TS38.101-1 for inter-band NR CA, and tables in clause 5.5B in TS38.101-3 for inter-band ENDC ) with specific note to identify the band combination with 1Tx in one band, and 2Tx in the other band.

### 2.2 SAR compliance for PC1.5

There is a PC1.5 band combination listed in the WID, i.e.

      - CA power class or EN-DC power class is PC1.5

        - PC3 FDD band 1Tx + PC1.5 TDD band 2Tx (UL MIMO and TxD)

Issue 2-5-1: PC1.5 inter-band CA/EN-DC SAR compliance - Proposals - Option 1: Use 0.5*maxUplinkDutyCycle-interBandCA-PC2 if indicated as the threshold for PC1.5 UL duty cycle, below which PC1.5 requirements apply. If absent, UE shall work on power class PC1.5 regardless of UL duty cycle and may use P-MPRc as defined in 6.2.4 in TS 38.101-1 or other means if necessary. - Option 2: For PC1.5 band combination, SAR compliance with the possible new signalling for the duty cycle mechanism need to be further studied. - Option 3: Study whether the existing UE capabilities (PC2-related maxUplinkDutyCycle) can be applied or new UE capabilities are needed. - WF - No consensus is reached for now and more study in next meeting.

Actually for PC2 inter-band CA, the dutycycle threshold calculation are as below:

- DutyNR, x *( PNR,x/ P26)*SARratioNR, x + DutyNR, y *(PNR, y/ P26)* SARratioNR, y  ≤ Duty threshold

where, SARratioNR, x = 50%/DutycycleNR, x   ; SARratioNR, y = 50%/DutycycleNR, y

In the spec, the above formula was simplified as 50%  (DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y)in which the actual percentage of uplink symbols transmitted(i.e. DutyNR, x and DutyNR, x) for the constituent bands are considered due to SAR measurements will be responded with different impacts by different band frequencies. It shall be noted that at that time, the SARratioNR, x for 23dBm NR band is 100%, and also it shall be noted that there is no SAR duty cycle scheme for 26dBm NR FDD band.

It is straightforward to apply the same approaches as PC2 inter-band UL CA for PC1.5 inter-band UL CA, i.e. per band duty cycles are included, and the corresponding per UE capability (i.e. maxDutyNR,x, maxDutyNR,y).

Observation. Actual percentage of uplink symbols transmitted (i.e. DutyNR, x and DutyNR, x) for the constituent bands are considered for PC2 inter-band CA.

Proposal 2. Apply same approaches as PC2 inter-band UL CA for PC1.5 inter-band UL CA

For concurrent PC1.5 3Tx inter-band CA, currently there is only PC3+PC1.5 (UL-MIMO band) power configurations. It seems the existing formula can be reused if the per band(PC1.5) duty cycle and PC1.5 BC are 0.5*maxUplinkDutyCycle-PC2-FR1 and 0.5*maxUplinkDutyCycle-interBandCA-PC2, respectively. Therefore, we have the proposal below

Proposal 3. Use 0.5*maxUplinkDutyCycle-PC2-FR1 and 0.5*maxUplinkDutyCycle-interBandCA-PC2 if indicated as the threshold for PC1.5 UL-MIMO band duty cycle and PC1.5 UL duty cycle.

### 2.2 Pcmax changes

Issue 2-2-4: Pcmax changes - Proposals - Option 1: Define configured transmitted power by considering up to PC1.5 (as below figure) for inter-band CA/EN-DC PC1.5 with 3Tx. - WF - Further study the delta Ppowerclass,CA changes for the SAR solutions.

For the delta Ppowerclass value for 3Tx inter-band UL ENDC and NR CA, still it can based on the 2Tx power fallback behaviour, i.e. 3dB or 6dB pending on the conditions for power fallback behaviour. For the values themself, we think 3/6dB can be reused for both HPUE 3Tx inter-band UL ENDC and NR CA although there were no 3Tx PC1.5 ENDC combination requested so far.

Proposal 4. Define configured transmitted power by considering up to PC1.5:

ΔPpowerclass,CA  = 6dB for a PC1.5 capable UE

ΔPpowerclass,CA = 3dB for a PC2 capable UE

Where power fallback behaviour can be further study, pending on the conclusion of SAR solution

# 3 Conclusion

In this contribution, we give some further discussions on the Tx RF requirements for 3Tx inter-band UL CA/ENDC band combination, where one of the constituent band supports UL MIMO. The conclusions are:

Proposal 1. Configuration for concurrent 3Tx inter-band NR CA/ENDC can be included in the existing configuration tables (i.e. tables in clause 5.5A in TS38.101-1 for inter-band NR CA, and tables in clause 5.5B in TS38.101-3 for inter-band ENDC ) with specific note to identify the band combination with 1Tx in one band, and 2Tx in the other band.

Observation. Actual percentage of uplink symbols transmitted (i.e. DutyNR, x and DutyNR, x) for the constituent bands are considered for PC2 inter-band CA.

Proposal 2. Apply same approaches as PC2 inter-band UL CA for PC1.5 inter-band UL CA

Proposal 3. Use 0.5*maxUplinkDutyCycle-PC2-FR1 and 0.5*maxUplinkDutyCycle-interBandCA-PC2 if indicated as the threshold for PC1.5 UL-MIMO band duty cycle and PC1.5 UL duty cycle.

Proposal 4. Define configured transmitted power by considering up to PC1.5:

ΔPpowerclass,CA  = 6dB for a PC1.5 capable UE

ΔPpowerclass,CA = 3dB for a PC2 capable UE

Where power fallback behaviour can be further study, pending on the conclusion of SAR solution

# 4	Reference

[1] RP-223517, Low NR band 4Rx for handheld UE and 3Tx for inter-band UL CA and EN-DC, OPPO,etc.

[2] R4-2303523, WF on 3Tx requirements, OPPO

[3] R4-2301250, Discussion on 3Tx inter-band UL CA/ENDC within two bands, ZTE

[4] R4-2300954, TR 38.850 v0.1.0 HPUE_FR1_FDD_NR_CADC_R18, China Unicom