# 3GPP TSG-RAN WG4 Meeting # 107	                                  R4-2308178

# Incheon, KR, May 22 – May 26, 2023

Source:	ZTE Corporation

Title:	TP for TR38.718-03-01_Correct the MSD test points for 3DL_2UL CA_n3A-n8A-n41A

Agenda Item:	7.10.2

Document for:	Approval

- Introduction

The MSD test point for the UL Fc and DL Fc of n41 are 2516MHz, which are:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n8-n41 | n3 | 1722.5 | 5 | 25 | 1817.5 | N/A | FDD | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n41 | 2610 | 10 | 50 | 2610 | 28.0 | FDD | IMD24 |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 26.0 | FDD | IMD24 |
|  | n41 | 2516 | 10 | 50 | 2516 | N/A | FDD | N/A |
| NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

However, by simple calculation, for the IMD2 caused by UL CA_n3-n41 falls into n8, i.e. f41_UL-f3_UL = f8_DL, by using  {f41_UL , f3_UL}={2516, 1725}MHz, we cannot get 945MHz for f8_DL. Hence, the MSD test point for n41 should be corrected.

By using the same MSD test point of n41 as DC_3-8_n41, the f41_UL  shall be 2670MHz. So in this TP, MSD test point for the UL Fc and DL Fc of n41 are corrected from 2516MHz to 2670MHz.

- Reference

- [1] TR38.718-03-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2),v0.4.0

- Text Proposal

----- Start of TP -----

# 5.4			CA_n3-n8-n41

### 5.4.1	Common for 1 band UL and 2 bands UL CA

#### 5.4.1.1	Operating bands for CA

Table 5.4.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

#### 5.4.1.2	Channel bandwidths per operating band for CA

Table 5.4.1.2-1: Supported bandwidths per CA band combination of band n3+n8+n41

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n8A-n41A | CA_n3A-n8A CA_n3A-n41A CA_n8A-n41A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |

#### 5.4.1.3	∆TIB,c and ∆RIB,c values

For CA_n3A-n8A-n41A, the TIB,c and RIB,c values have already been included in the TS38.101-1.

### 5.4.2	Specific for 2 bands UL CA

#### 5.4.2.1	UE co-existence studies

The co-existence for the fallback 2DL/2UL of CA_n3A-n8A, CA_n3A-n41A and CA_n8A-n41A have already been analyzed. In terms of the co-existence studies of corresponding fallbacks, it can be observed:

IMD 2 and IMD3 caused by n3+n8 may fall into the its own band n41 Rx

IMD 2 and IMD3 caused by n3+n41 may fall into the its own band n8 Rx

IMD2 caused by n8+n41 may fall into the its own band n3 Rx

#### 5.4.2.2	REFSENS requirements

For IMD2 caused by n8+n41, the IMD2 frequency range is calculated by the lower edge and upper edge of the bands, which is 1581MHz~1810MHz (i.e. f41-f8) , while the band n3 DL frequency range is 1805MHz~1880MHz, the overlapping frequency range is only 5MHz. However, considering the minimum channel bandwidths of band n8 and n41, which are 5MHz and 10MHz, respectively. The IMD2 frequency range is 1588.5 MHz ~ 1802.5 MHz, which means there are no test points of the IMD2 product fall into the own band n3 DL frequency range. Therefore, there is no need to define such MSD values.

For the other IMDs, the MSD requirement are defined in table 5.4.2.2-1:

Table 5.4.2.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n8-n41 | n3 | 1722.5 | 5 | 25 | 1817.5 | N/A | FDD | N/A |
|  | n8 | 887.5 | 5 | 25 | 932.5 | N/A | FDD | N/A |
|  | n41 | 2610 | 10 | 50 | 2610 | 28.0 | FDD | IMD24 |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | FDD | N/A |
|  | n8 | 900 | 5 | 25 | 945 | 26.0 | FDD | IMD24 |
|  | n41 | 2670 | 10 | 50 | 2670 | N/A | FDD | N/A |
|  | n3 | 1712.5 | 5 | 25 | 1807.5 | [25] | FDD | IMD2 near-miss |
|  | n8 | 882.5 | 5 | 25 | 927.5 | N/A | FDD | N/A |
|  | n41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

----- End of TP -----